3GPP TSG-RAN WG4 Meeting #92-Bis	R4-1910861

# Chongqing, China, 14th – 18th Oct, 2019

Source: 	Huawei, HiSilicon

Title:	Work plan on addition of asymmetric channel bandwidth for n66

Agenda Item:	9.25

Document for:	Approval

1	Introduction

In last RAN plenary meeting #85, a new WI was approved on addition of asymmetric channel bandwidth for n66 [1].

In this contribution we provide some discussion and work plan for the WI.

2	Discussion

The objective of the WI [1] is cited as below,

The objective of this WI is to add to NR band n66 the support of 25MHz, 30MHz channel bandwidth, and DL channel bandwidth of 15 MHz, 25 MHz, 30 MHz and 50 MHz for FDD asymmetric UL and DL channel bandwidth combinations, with supported subcarrier spacing of 15 kHz, 30 kHz and 60 kHz. Specifically, it includes the following aspects:

- Introduce 25, 30 and 50 MHz BS channel bandwidths

- Specify UE RF requirements for 25 MHz, 30 MHz channel bandwidth for UL and DL, and 25, 30 and 50 MHz channel bandwidth for DL.

- Introduce additional FDD asymmetric UL and DL channel bandwidth combinations as shown in Table below (highlighted by yellow).

| NR Band | Channel bandwidths for UL (MHz) | Channel bandwidths for DL (MHz) |
| --- | --- | --- |
| n66 | 5, 10, | 15, 20, 25, 30, 40 |
|  | 20 | 25, 30, 40, 50 |
|  | 25 | 30, 40, 50 |
|  | 30 | 25, 40, 50 |

Basically the WI covers two aspects: 1) add new channel bandwidths; 2) add more asymmetric UL and DL combinations. More specifically, the needed work is summarized as below,

- Impact to TS 38.104

  - Add 25, 30 and 50 MHz BS channel bandwidths in the spec

- Impact to TS 38.101-1

  - Introduce 25, 30 and 50 MHz channel bandwidth for DL. UE reference sensitivity should be defined for these channel bandwidths

  - Introduce 25 and 30 MHz channel bandwidth for UL. The additional emission requirements for NS_03 should be defined for 25 and 30 MHz

  - Introduce asymmetric UL and DL channel bandwidth combinations

3	Work plan

The following work plan is proposed,

- RAN4#92bis

  - Draft CR for TS 38.104

  - Discuss UE REFSENS requirement

  - Discuss additional emission requirement for NS_03

  - Discuss the support of asymmetric channel bandwidth

- RAN4#93

  - Finalize UE RF requirement

- RAN4#94

  - Finalize CRs for TS 38.104 and TS 38.101-1.

4	References

[1] RP-192276, New WI proposal: Addition of asymmetric channel bandwidth for n66