3GPP TSG-RAN WG4 Meeting #92Bis	R4-1910917

Chongqing, China, 14th – 18th Oct.,

Agenda item:		8.7.3

Source:	MediaTek Inc.

Title:	Discussion on IDLE/INACTIVE mode CA measurement

Document for:		Discussion

# 1	Introduction

This paper is revised from our previous paper R4-1908366 [1]. In RAN4 #92 meeting, it is agreed that supporting IDLE/INACTIVE mode CA measurement should not extend the measurement capability defined in Rel-15 [2]. Besides, a way forward is also agreed for further discussion of the related issues [3]:

Agreements: - RAN4 defines measurement requirements for UE early measurement in IDLE and INACTIVE - RAN4 defines UE requirements for NR Inter-RAT for early measurements - RAN4 defines UE requirements for LTE Inter-RAT for early measurements - The supported measurement quantities for early measurements are RSRP and RSRQ - RAN4 defines measurement accuracy for early measurement reporting - RAN4 defines measurement period for measurements for early reporting - RAN4 captures requirements for early reporting based on SIB configuration - In NR, supporting IDLE/INACTIVE mode CA measurement should not extend the measurement capability defined in Rel-15 Way forward: - Discuss the definition and restriction on carriers for early measurement reporting. - Discuss if the LTE way of using search thresholds for limiting the UE measurements for idle mode measurements for reporting. - Analyze the necessary changes and proposed requirements.

In this paper, we further discuss the definition and restriction on carriers for NR IDLE/INACTIVE CA measurement.

# 2	Discussion

We would like to first discuss the UE requirements for LTE inter-RAT. In current RAN4 spec, TS36.133 Section 4.9 has specified the IDLE mode CA measurement for LTE SA mode UE during Rel-15 euCA WI. For UE which supports ca-IdleModeMeasurements, UE shall perform measurement on the configured overlapping and non-overlapping inter-frequency carriers for IDLE mode measurement reporting shown as Table 1.

Table 1: Measurements of inter-frequency CA candidate cells

| Serving cell signal quality | Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ |  | Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ |  |
| --- | --- | --- | --- | --- |
| Overlapping with carriers for re-selection | Overlapping | Non-overlapping | Overlapping | Non-overlapping |
| # of early measurement reporting carriers | 3 (0 or 1 inter-frequency carrier may be a non-overlapping carrier) |  | 1 (0 or 1 inter-frequency carrier may be a non-overlapping carrier) |  |
| Measurement delay | 4.2.2.4 | N/A | N/A | N/A |
| Measurement accuracy | 9.1.3B.2 & 9.1.6B.2 | 9.1.3B.3 & 9.1.6B.3 | 9.1.3B.3 & 9.1.6B.3 | 9.1.3B.3 & 9.1.6B.3 |

Where the definition of overlapping and non-overlapping carriers are:

- Overlapping: a carrier configured by higher layer for early measurement reporting and inter-frequency mobility measurements.

- Non-overlapping: a carrier configured by higher layer for early measurement reporting while not configured for inter-frequency mobility measurements.

Besides, TS36.133 Section 4.2.2.9 specifies that UE shall be capable of monitoring a total of at least 8 carrier frequency layers in RRC_IDLE state. As we mentioned in the introduction, RAN4 already agreed that for UE supporting IDLE/INACTIVE mode CA measurement, the measurement capability defined in Rel-15 should not be extended. Therefore, when UE already monitors 8 carrier frequency layers in E-UTRA measurements RRC_IDLE state, only the overlapping carriers need to be reported to Network for the early measurement reporting.

What makes things difference is the serving cell signal quality. If one of the serving cell signal quality is no larger than a certain threshold, i.e., Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ, the number of early measurement reporting is 3; otherwise, the number of early measurement reporting is 1.

Observation 1: UE monitors more carriers for LTE IDLE mode CA measurement when one of the serving cell signal quality is no larger than a certain threshold.

In our understanding, UE might prefer to allocate more resources (time and power) for cell re-selection when serving cell signal quality is no larger than a certain threshold. On the other hand, when serving cell signal quality is good, i.e., Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ, UE has less intention to perform frequent measurement for cell re-selection. In this case, UE can spend more time and power on candidate CA carriers. So we suggest

Proposal 1: For NR IDLE/INACTIVE mode CA measurement, UE monitors less carriers when one of the serving cell signal quality is no larger than a certain threshold and measures more carriers for CA measurement otherwise.

We then discuss the restrictions on the early measurement reporting, i.e., 0 or 1 inter-frequency carrier may be a non-overlapping carrier. Assuming that Srxlev ≤ SnonIntraSearchP, and UE is monitoring 8 carriers and the RSRP for these carriers are as shown in Table 2.

Table 2: Example of IDLE/INACTIVE mode CA measurement

|  | Non-overlapping carriers | Overlapping carriers |
| --- | --- | --- |
| RSRP Ranking | F1: -82dBm | F4: -90dBm |
|  | F2: -86dBm | F5: -91dBm |
|  | F3: -89dBm | F6: -93dBm |
|  |  | F7: -95dBm |
|  |  | F8: -97dBm |

Since UE can only report 0 or 1 carrier from the set of non-overlapping carrier, UE may report:

- “1” non-overlapping carrier, e.g., UE reports F1, F4, F5.

  - F2 and F3 with higher RSRPs than all overlapping carriers are not reported.

- “0” non-overlapping carrier, e.g., UE reports F4, F5, F6.

  - F1, F2 and F3 with higher RSRPs than all overlapping carriers are not reported.

In our understanding, this is strange that UE cannot report the carrier with better RSRP to network. Therefore, we suggest to remove such restriction and propose

Proposal 2: In IDLE/INACTIVE mode CA measurement, RAN4 to remove the restriction on the number of overlapping and non-overlapping carriers in early measurement reporting and only specify the total number of carriers.

# 4	Summary

In this contribution, we have the following observations:

Observation 1: UE monitors more carriers for LTE IDLE mode CA measurement when one of the serving cell signal quality is no larger than a certain threshold.

And we propose

Proposal 1: For NR IDLE/INACTIVE mode CA measurement, UE monitors less carriers when one of the serving cell signal quality is no larger than a certain threshold and measures more carriers for CA measurement otherwise.

Proposal 2: In IDLE/INACTIVE mode CA measurement, RAN4 to remove the restriction on the number of overlapping and non-overlapping carriers in early measurement reporting and only specify the total number of carriers.

# Reference

- R4-1908366, Discussion on IDLE mode CA measurement, Mediatek Inc., Aug., 2019.

- R4-191XXXX, RAN4 #92 Meeting report, Aug., 2019.

- R4-1909997, Way Forward on MR-DC RRM, Xi'an, Nokia, Aug., 2019.