3GPP TSG-RAN WG4 Meeting #92bis	R4-1910980

Chongqing, China, 14th – 18th Oct. 2019

Title: 	TP for Rx requirements for NR V2X

Source: 	CATT

Agenda item:	9.4.3.2

Document for:	Approval

- Introduction

In the document [1], the RF requirements for NR V2X in FR1 are discussed. Some general Rx requirements for NR UE RF can be reused for NR V2X. These requirements should be captured in TR38.886.

The text proposals are attached in Section 3 in this document.

- Reference

[1]	R4-1908377, Discussion on NR V2X UE RF requirements, CATT

- Annex: TP for TR38.886 for Rx requirements for NR V2X

9.1	NR V2X UE Rx requirements in FR1

9.1.1	Reference sensitivity for NR V2X Communication

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes [x] with parameters specified in Table 9.1.1-1.

Table 9.1.1-1: Reference sensitivity of NR V2X

| NR V2X Band | Channel bandwidth / Reference sensitivity (dBm) |  |  |  |  |  |  |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 10 MHz | 20 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |  |  |  |
| n47 |  | TBD | TBD |  | TBD |  |  |  |  |  |  | HD |  |
| NOTE 1: Reference measurement channel is defined in [x]. NOTE 2: The signal power is specified per port. |  |  |  |  |  |  |  |  |  |  |

9.1.2	Maximum input level for NR V2X Communication

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes [x] with parameters specified in Table 9.1.2-1.

Table 9.1.2-1: Maximum input level

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 10 MHz | 20 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |  |
| Power in Transmission Bandwidth Configuration |  | dBm |  | -222 | -222 |  | -222 |  |  |  |  |  |  |
|  |  |  |  | -233 | -233 |  | -233 |  |  |  |  |  |  |
| NOTE 1: Reference measurement channel is defined in Annex [x]. NOTE 2: This requirement is applicable for 16QAM. NOTE 3: This requirement is applicable for 64QAM. |  |  |  |  |  |  |  |  |  |  |

9.1.3	Adjacent Channel Selectivity (ACS) for NR V2X Communication

The NR V2X UE shall fulfil the minimum requirement specified in Table 9.1.3-1 for all values of an adjacent channel interferer up to -22 dBm. However it is not possible to directly measure the ACS, instead the lower and upper range of test parameters are chosen in Table 9.1.3-2 and Table 8.1.3-3 where the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex [x].

Table 9.1.3-1: Adjacent channel selectivity for NR V2X

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 10 MHz | 20 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| ACS | dB | FFS | FFS |  | FFS |  |  |  |  |  |

Table 9.1.3-2: Test parameters for Adjacent channel selectivity for NR V2X, Case 1

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 10 MHz | 20 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| Power in Transmission Bandwidth Configuration | dBm | PREFSENS_V2X + 14 dB |  |  |  |  |  |  |  |  |
| PInterferer | dBm | PREFSENS_V2X + [FFS]dB | PREFSENS_V2X + [FFS]dB |  | PREFSENS_V2X + [FFS]dB |  |  |  |  |  |
| BWInterferer | MHz | 10 | 10 |  | 10 |  |  |  |  |  |
| FInterferer (offset) | MHz | 10+0.0125 / -10-0.0125 | 15+0.0075 / -15-0.0075 |  | 25+0.0125 / -25-0.0125 |  |  |  |  |  |
| NOTE 1: The interferer is QPSK modulated PUSCH containing data and reference symbols. Normal cyclic prefix is used. The data content shall be uncorrelated to the wanted signal and modulated according to clause [x] of TS38.211. |  |  |  |  |  |  |  |  |  |  |

Table 9.1.3-3: Test parameters for Adjacent channel selectivity for NR V2X, Case 2

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 10 MHz | 20 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| Power in Transmission Bandwidth Configuration | dBm | FFS | FFS |  | FFS |  |  |  |  |  |
| PInterferer | dBm | -22 |  |  |  |  |  |  |  |  |
| BWInterferer | MHz | 10 | 10 |  | 10 |  |  |  |  |  |
| FInterferer (offset) | MHz | 10+0.0125 / -10-0.0125 | 15+0.0075 / -15-0.0075 |  | 25+0.0125 / -25-0.0125 |  |  |  |  |  |
| NOTE 1: The interferer is QPSK modulated PUSCH containing data and reference symbols. Normal cyclic prefix is used. The data content shall be uncorrelated to the wanted signal and modulated according to clause [x] of TS38.211. |  |  |  |  |  |  |  |  |  |  |

9.1.4	Spurious response for NR V2X Communication

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex [x] with parameters specified in Table 9.1.4-1.

Table 9.1.4-1: Spurious response parameters

| Rx parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 10 MHz | 20 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| Power in Transmission Bandwidth Configuration | dBm | PREFSENS_V2X + channel bandwidth specific value below |  |  |  |  |  |  |  |  |
|  |  | 9 | 9 |  | 9 |  |  |  |  |  |
| NOTE 1: Reference measurement channel is specified in Annex [x]. |  |  |  |  |  |  |  |  |  |  |

Table 9.1.4-2: Spurious response

| Parameter | Unit | Level |
| --- | --- | --- |
| PInterferer (CW) | dBm | -44 |
| FInterferer | MHz | Spurious response frequencies |

9.1.5	Intermodulation characteristics for NR V2X Communication

The throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annex [x] with parameters specified in Table 9.1.5-1 for the specified wanted signal mean power in the presence of two interfering signals

Table 9.1.5-1: Wide band intermodulation

| Rx Parameter | Units | Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 10 MHz | 20 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| Power in Transmission Bandwidth Configuration | dBm | PREFSENS_V2X + channel bandwidth specific value below |  |  |  |  |  |  |  |  |
|  |  | 9 | 9 |  | 9 |  |  |  |  |  |
| PInterferer 1 (CW) | dBm | -46 |  |  |  |  |  |  |  |  |
| PInterferer 2 (Modulated) | dBm | -46 |  |  |  |  |  |  |  |  |
| BWInterferer 2 | MHz | 10 | 10 |  | 10 |  |  |  |  |  |
| FInterferer 1 (Offset) | MHz | -BW/2 – 15 / +BW/2 + 15 | -BW/2 – 15 / +BW/2 + 15 |  | -BW/2 – 15 / +BW/2 + 15 |  |  |  |  |  |
| FInterferer 2 (Offset) | MHz | 2*FInterferer 1 |  |  |  |  |  |  |  |  |
| NOTE 1: Reference measurement channel is specified in Annex A.8.2 NOTE 2: The interferer is QPSK modulated PUSCH containing data and reference symbols. Normal cyclic prefix is used. The data content shall be uncorrelated to the wanted signal and modulated according to clause 5 of TS36.211 |  |  |  |  |  |  |  |  |  |  |

9.2	NR V2X UE Rx requirements in FR2