3GPP TSG-RAN WG4 Meeting #92bis                                                             R4-1910983

# Chongqing, China, 14th–18th Oct., 2019

Source: 	CATT

Title: 	Discussion on HST PUSCH demodulation requirement

Agenda item:	8.17.2.3

Document for:	Agreement

- Introduction

In the recent RAN4#92 meeting, a WF[1] on NR HST BS demodulation requirements was approved.  There are some open issues (such as maximum Doppler shift, front loaded DMRS start symbol,propagation conditions) for PUSCH demodulation needing to be solved in RAN4#92bis.

The maximum Doppler shift options are shown as follows:

- Maximum Doppler shift - Single tap HST 350km/h - 15kHz SCS: 1340Hz - 30kHz SCS: 2334Hz - Single tap HST 500km/h - 15kHz SCS: - Option 1: 1944Hz - Option 2: 1500Hz - Option 3: 1400Hz - Option 4: 1250Hz - 30kHz SCS: - Option 1: 3334Hz - Option 2: 2600Hz - Option 3: 3000Hz - Option 4: 2300Hz

The front loaded DMRS start symbol options are shown as follows:

- l0 for PUSCH mapping type A - Provide the simulation results for 350km/h and 500km/h and evaluate the following configurations and make decision: - Option 1: l0 = 3 - Option 2: l0 = 2

The propagation conditions options(HST scenario 1option 1 and HST scenario 1option 2) are shown as follows:

This paper will give our view on maximum Doppler shift, front loaded DMRS start symbol and propagation conditions for NR HST PUSCH based on evaluationresults for HST PUSCH.

- Discussion

2.1 Theoretical analysis

The maximum Doppler shift for BS for 450km/h and 500km/h at 2.1GHz and 3.6GHz are shown in Table 2-1 respectively.

Table 2-1 maximum Doppler shift for BS for 450km/h and 500km/h at  2.1GHz, and 3.6GHz

| speeed | 2.1GHz | 3.6GHz |
| --- | --- | --- |
| 450km/h | 1750Hz | 3000Hz |
| 500km/h | 1944Hz | 3334Hz |

Table 2-2 DMRS Doppler tracking capability [2]

| DMRS configuration | SCS (KHz) | Maximum Interval of RS | Frequency Offset range |
| --- | --- | --- | --- |
| NR 1+1+1 (3,7,11) | 15 | 0.285ms | +/- 1750Hz |
| NR 1+1+1 (2,7,11) | 15 | 0.357ms | +/- 1400Hz |
| NR 1+1+1 (3,7,11) | 30 | 0.1425ms | +/- 3500Hz |
| NR 1+1+1 (2,7,11) | 30 | 0.1786ms | +/- 2800 Hz |

As indicated in the following Table2-2 in contribution [2], the maximum tracking frequency offset of front loaded DMRS start symbol l0=3 at 15kHz and 30kHz are 1750Hz and 3500Hz, which are greater than 1400Hz and 2800Hz of front loaded DMRS start symbol l0=2 at 15kHz and 30kHz respectively. The Doppler tracking capability of front loaded DMRS start symbol l0=3 at DMRS configuration (1+1+1) is better than that of l0=2 at 15 kHz and 30 kHz.

Observation 1: In theory, the Doppler tracking capability of front loaded DMRS start symbol l0=3 at DMRS configuration (1+1+1) is better than that of l0=2 at 15 kHz and 30 kHz.

The Doppler shift 1944Hz for 500km/h at 2.1GHz is greater than 1750Hz which is maximum tracking frequency offset of front loaded DMRS start symbol l0=3 at 15 kHz, so 1944Hz for 500km/h at 2.1GHz exceeds the Doppler tracking capability of DMRS configuration with l0=3. According to the Table 2-1, the 1750Hz are important maximum Doppler shift options for 15 kHz for evaluation, while 3334Hz and 3000Hz are important maximum Doppler shift options for 30 kHz, which don’t exceed the maximum tracking frequency offset of front loaded DMRS start symbol l0=3 at 15kHz and 30kHz respectively. So the following maximum Doppler shift options are chosen for evaluation.

- Maximum Doppler shift - Single tap HST 350km/h - 15kHz SCS: 1340Hz - 30kHz SCS: 2334Hz - Single tap HST 500km/h - 15kHz SCS: - Option 1: 1944Hz - Option 5: 1750Hz - 30kHz SCS: - Option 1: 3334Hz - Option 3: 3000Hz

The other detailed simuation asumptions for 350km/h and 500km/h are shown in Table A-1 and Table A-2 in Annex respectively

2.2 Evaluation results for 350km/h

From the Table2-2, the 1340Hz is in the range of maximum tracking frequency offset of front loaded DMRS start symbol l0=3 and l0=2 at 15kHz(1750, and 1400Hz), and the 2334Hz is in the range of maximum tracking frequency offset of front loaded DMRS start symbol l0=3 and l0=2 at 30kHz(3500,2800Hz), so the l0=3 and l0=2 can track the 1340Hz and 2334 Hz frequency offset better.

The evaluation results for 350km/h at 2.1GHz(15kHz SCS) and for 350km/h at 3.6GHz(30kHz SCS) are shown in Table 2-3 and Table 2-4 respectively. From the Table 2-3 and Table 2-4, SNRs @70% of maximum throughputin all popagation conditions are around -6.5 dB. It is observed thatboth l0=3 and l0=2 can achieve good performance in HST-scenario 3, HST-scenario 1option 1, HST-scenario 1option 2 with 1340Hz or 2334Hz frequency offset. The evaluation results are consistent with the theoretical analysis. In all propagation conditions with 350km/h, the performance of l0=3 and l0=2 are similar and the performance in the HST Scenario 1option 1 and HST Scenario 1option 2 with 350km/h are similar.

Observation 2: In three propagation conditions with 350km/h, the performance of l0=3 and l0=2 are similar.

Observation 3: The performance in the HST Scenario 1option 1 and HST Scenario 1option 2with 350km/h are similar.

Table 2-3 Evaluation results for 350km/h at 2.1GHz (15 kHz SCS)

| waveform | SCS-BW | Carrier Frequency (GHz) | Number of DMRS symbols | Front loaded DMRS start symbol l0 | Propagation conditions | frequency offset(Hz) | SNR @70% of maximum throughput |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CP-OFDM | 15kHz, 10MHz | 2.1 | 1+1+1 | 3 | HST- Scenario 3 | 1340 | -6.32 |
| CP-OFDM | 15kHz, 10MHz | 2.1 | 1+1+1 | 3 | HST- Scenario 1option 1 | 1340 | -6.34 |
| CP-OFDM | 15kHz, 10MHz | 2.1 | 1+1+1 | 3 | HST- Scenario 1option 2 | 1340 | -6.33 |
| CP-OFDM | 15kHz, 10MHz | 2.1 | 1+1+1 | 2 | HST- Scenario 3 | 1340 | -6.52 |
| CP-OFDM | 15kHz, 10MHz | 2.1 | 1+1+1 | 2 | HST- Scenario 1option 1 | 1340 | -7.53 |
| CP-OFDM | 15kHz, 10MHz | 2.1 | 1+1+1 | 2 | HST- Scenario 1option 2 | 1340 | -6.52 |

Table 2-4 Evaluation results for 350km/h at 3.6GHz (30 kHz SCS)

| waveform | SCS-BW | Carrier Frequency (GHz) | Number of DMRS symbols | Front loaded DMRS start symbol l0 | Propagation conditions | frequency offset(Hz) | SNR @70% of maximum throughput |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CP-OFDM | 30kHz, 40MHz | 3.6 | 1+1+1 | 3 | HST- Scenario 3 | 2334 | -5.46 |
| CP-OFDM | 30kHz, 40MHz | 3.6 | 1+1+1 | 3 | HST- Scenario 1option 1 | 2334 | -5.35 |
| CP-OFDM | 30kHz, 40MHz | 3.6 | 1+1+1 | 3 | HST- Scenario 1 option 2 | 2334 | -5.49 |
| CP-OFDM | 30kHz, 40MHz | 3.6 | 1+1+1 | 2 | HST- Scenario 3 | 2334 | -6.43 |
| CP-OFDM | 30kHz, 40MHz | 3.6 | 1+1+1 | 2 | HST- Scenario 1option 1 | 2334 | -6.42 |
| CP-OFDM | 30kHz, 40MHz | 3.6 | 1+1+1 | 2 | HST- Scenario 1option 2 | 2334 | -6.42 |

2.3 Evaluation results for 500km/h

The evaluation results for 500km/h at 2.1GHz(15kHz SCS) and for 500km/h at 3.6GHz(30kHz SCS) are shown in Table 2-5 and Table 2-6 respectively.From Table 2-5, for HST- Scenario3 with 1944Hz, the SNR @70% of maximum throughputwith l0=3 and l0=2 are -6.3dB and -6.5dB respectively. It is observed thatthe perfomance of l0=3 and l0=2 are similar, and the 1944Hz is applicable for HST- Scenario 3 for 500km/h at15 kHz SCS.

Observation 4: The performance of l0=3 and l0=2 are similar in HST- Scenario 3 with 1944Hz, and the 1944Hz is applicable for HST- Scenario 3 for 500km/h at15 kHz SCS.

From Table 2-5,  for HST- Scenario 1option 1with 1944Hz frequency offset , the SNR @70% of maximum throughput with l0=3 and l0=2 are 15.52dB and 23.36dB respectively, which show the 1944Hz isn’t applicable for HST- Scenario 1option 1. ForHST- Scenario 1option 1with 1750Hz frequency offset, the SNR @70% of maximum throughput with l0=3 and l0=2 are -4.99 dB and 3.93dB respectively. It is obeserved thatthe perfomance of l0=3 is better than l0=2 HST- Scenario 1option 1with 1750Hz frequency offset, and the 1750Hz is applicable for HST- Scenario 1option 1 for 500km/h at15 kHz SCS.

Observation 5: The performance of l0=3 is better than l0=2 in HST- Scenario 1option 1with 1750Hz frequency offset, and the 1750Hz is applicable for HST- Scenario1 option1 for 500km/h at15 kHz SCS.

From Table 2-5,  for HST- Scenario 1option 2with 1944Hz frequency offset, the SNR @70% of maximum throughput with l0=3 and l0=2 are -5.43dB and 0.8dB respectively, whichperfomance of l0=3 is better than l0=2 HST- Scenario 1option 2with 1944Hz frequency offset, and the 1944Hz is applicable for HST- Scenario 1option 2 for 500km/h at15 kHz SCS in case of l0=3.ForHST- Scenario 1option 2with 1750Hz frequency offset, the SNR @70% of maximum throughput with l0=3 and l0=2 are -6.34 dB and -3.94dB respectively,  which show perfomance of l0=3 is slightly better than l0=2 HST- Scenario 1option 1with 1750Hz frequency offset, the 1750Hz is applicable for HST- Scenario 1option 2 for 500km/h at15 kHz SCS.

Observation 6: The performance of l0=3 is better than l0=2 in HST- Scenario 1option 2with 1944Hz frequency offset, and the 1944Hz and 1750Hz are also applicable for HST- Scenario 1option 2 for 500km/h at15 kHz SCS in case of l0=3.

Table 2-5 Evaluation results for 500km/h at2.1GHz (15 kHz SCS)

| waveform | SCS-BW | Carrier Frequency (GHz) | Number of DMRS symbols | Front loaded DMRS start symbol l0 | Propagation conditions | frequency offset(Hz) | SNR @70% of maximum throughput |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CP-OFDM | 15kHz, 10MHz | 2.1 | 1+1+1 | 3 | HST- Scenario 3 | 1944 | -6.3 |
| CP-OFDM | 15kHz, 10MHz | 2.1 | 1+1+1 | 3 | HST- Scenario 1option 1 | 1944 | 15.52 |
| CP-OFDM | 15kHz, 10MHz | 2.1 | 1+1+1 | 3 | HST- Scenario 1option 2 | 1944 | -5.43 |
| CP-OFDM | 15kHz, 10MHz | 2.1 | 1+1+1 | 2 | HST- Scenario 3 | 1944 | -6.5 |
| CP-OFDM | 15kHz, 10MHz | 2.1 | 1+1+1 | 2 | HST- Scenario 1option 1 | 1944 | 23.36 |
| CP-OFDM | 15kHz, 10MHz | 2.1 | 1+1+1 | 2 | HST- Scenario 1option 2 | 1944 | 0.8 |
| CP-OFDM | 15kHz, 10MHz | 2.1 | 1+1+1 | 3 | HST- Scenario 3 | 1750 | -6.31 |
| CP-OFDM | 15kHz, 10MHz | 2.1 | 1+1+1 | 3 | HST- Scenario 1option 1 | 1750 | -4.99 |
| CP-OFDM | 15kHz, 10MHz | 2.1 | 1+1+1 | 3 | HST- Scenario 1option 2 | 1750 | -6.34 |
| CP-OFDM | 15kHz, 10MHz | 2.1 | 1+1+1 | 2 | HST- Scenario 3 | 1750 | -6.51 |
| CP-OFDM | 15kHz, 10MHz | 2.1 | 1+1+1 | 2 | HST- Scenario 1option 1 | 1750 | 3.93 |
| CP-OFDM | 15kHz, 10MHz | 2.1 | 1+1+1 | 2 | HST- Scenario 1option 2 | 1750 | -3.94 |

From Table 2-6, for HST- Scenario 3 with 3334Hz, the SNR @70% of maximum throughputwith l0=3 and l0=2 are -5.44dB and -6.4dB respectively. It is observed that the perfomance of l0=3 and l0=2 are similar, and the 3334Hz is applicable for HST- Scenario 3 for 500km/h at30 kHz SCS.

Observation 7: The performance of l0=3 and l0=2 are similar in HST-Scenario 3 with 3334Hz, and the 3334Hz is applicable for HST- Scenario 3 for 500km/h at30 kHz SCS.

From Table 2-6,  for HST- Scenario 1option 1with 3334Hz frequency offset, the SNR @70% of maximum throughput with l0=3 and l0=2 are -5.43dB and 0.66dB respectively. It is observed thatthe perfomance of l0=3 is better than l0=2 HST-Scenario 1option 1with 3334Hz frequency offset, and the 3334Hz is applicable for HST-Scenario 1option 1 for 500km/h at30 kHz SCS in case of l0=3.

Observation 8: The performance of l0=3 is better than l0=2 in HST- Scenario 1option 1with 3334Hz frequency offset, and the3334Hz is applicable for HST- Scenario1 option1 for 500km/h at30 kHz SCS in case of l0=3.

From Table 2-6, forHST- Scenario 1option 2with 3334Hz frequency offset, the SNR @70% of maximum throughput with l0=3 and l0=2 are -5.47 dB and -3.58dB respectively. It is observed that the perfomance of l0=3 is slightly better than l0=2 HST-Scenario 1option 1with 3334Hz frequency offset, and the 3334Hz is applicable for HST-Scenario 1option 2 for 500km/h at30 kHz SCS.

Observation 9: The performance of l0=3 is better than l0=2 in HST-Scenario 1option 2with 3334Hz frequency offset, and the 3334Hz are also applicable for HST- Scenario1 option2 for 500km/h at30 kHz SCS.

Table 2-6 Evaluation results for 500km/h at3.6GHz (30 kHz SCS)

| waveform | SCS-BW | Carrier Frequency (GHz) | Number of DMRS symbols | Front loaded DMRS start symbol l0 | Propagation conditions | frequency offset(Hz) | SNR @70% of maximum throughput |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CP-OFDM | 30kHz, 40MHz | 3.6 | 1+1+1 | 3 | HST- Scenario 3 | 3334 | -5.44 |
| CP-OFDM | 30kHz, 40MHz | 3.6 | 1+1+1 | 3 | HST- Scenario 1option 1 | 3334 | -5.43 |
| CP-OFDM | 30kHz, 40MHz | 3.6 | 1+1+1 | 3 | HST- Scenario 1option 2 | 3334 | -5.47 |
| CP-OFDM | 30kHz, 40MHz | 3.6 | 1+1+1 | 2 | HST- Scenario 3 | 3334 | -6.4 |
| CP-OFDM | 30kHz, 40MHz | 3.6 | 1+1+1 | 2 | HST- Scenario 1option 1 | 3334 | 0.66 |
| CP-OFDM | 30kHz, 40MHz | 3.6 | 1+1+1 | 2 | HST- Scenario 1option 2 | 3334 | -3.58 |
| CP-OFDM | 30kHz, 40MHz | 3.6 | 1+1+1 | 3 | HST- Scenario 3 | 3000 | -5.34 |
| CP-OFDM | 30kHz, 40MHz | 3.6 | 1+1+1 | 3 | HST- Scenario 1option 1 | 3000 | -5.4 |
| CP-OFDM | 30kHz, 40MHz | 3.6 | 1+1+1 | 3 | HST- Scenario 1option 2 | 3000 | -5.45 |
| CP-OFDM | 30kHz, 40MHz | 3.6 | 1+1+1 | 2 | HST- Scenario 3 | 3000 | -6.42 |
| CP-OFDM | 30kHz, 40MHz | 3.6 | 1+1+1 | 2 | HST- Scenario 1option 1 | 3000 | -1.04 |
| CP-OFDM | 30kHz, 40MHz | 3.6 | 1+1+1 | 2 | HST- Scenario 1option 2 | 3000 | -6.43 |

Based on above analsysis on evaluation results, we have the following proposals.

Proposal 1:  DMRS configuraton(1+1+1) with l0=3 is more applicable for NR HST.

Proposal 2:  The following HST single tap channel models for 500km/h are approved by RAN4.

  - Tunnel channel model

| Parameter | Value |
| --- | --- |
|  | Scenario 3 |
| Ds | 300 m |
| Dmin | 2 m |
| v | 500km/h |
| fd | 15kHz SCS: 1944Hz 30kHz SCS: 3334Hz |

  - Open space

| Parameter | Value | value |
| --- | --- | --- |
|  | Scenario 1 | Scenario 1a |
| Ds | 1000 m | 700 m |
| Dmin | 50 m | 150 m |
| v | 500km/h | 500km/h |
| fd | 15kHz SCS: 1750Hz 30kHz SCS: 3334Hz | 15kHz SCS: 1944Hz or 1750Hz 30kHz SCS: 3334Hz |

- Conclusion

This paperprovided evaluation results for HST. Based on our evaluation results for NR HST PUSCH, we have the following observations and proposals.

Observation 1: In theory, the Doppler tracking capability of front loaded DMRS start symbol l0=3 at DMRS configuration (1+1+1) is better than that of l0=2 at 15 kHz and 30 kHz.

Observation 2: In three propagation conditions with 350km/h, the performance of l0=3 and l0=2 are similar.

Observation 3: The performance in the HST Scenario 1option 1 and HST Scenario 1option 2with 350km/h are similar.

Observation 4: The performance of l0=3 and l0=2 are similar in HST-Scenario 3 with 1944Hz, and the 1944Hz is applicable for HST- Scenario 3 for 500km/h at15 kHz SCS.

Observation 5: The performance of l0=3 is better than l0=2 in HST-Scenario 1option 1with 1750Hz frequency offset, and the 1750Hz is applicable for HST-Scenario 1option 1 for 500km/h at15 kHz SCS.

Observation 6: The performance of l0=3 is better than l0=2 in HST-Scenario 1option 2with 1944Hz frequency offset, and the 1944Hz and 1750Hz are also applicable for HST-Scenario 1option 2 for 500km/h at15 kHz SCS in case of l0=3.

Observation 7: The performance of l0=3 and l0=2 are similar in HST- Scenario 3 with 3334Hz, and the 3334Hz is applicable for HST- Scenario 3 for 500km/h at30 kHz SCS.

Observation 8: The performance of l0=3 is better than l0=2 in HST- Scenario 1option 1with 3334Hz frequency offset, and the3334Hz is applicable for HST- Scenario 1option 1 for 500km/h at30 kHz SCS in case of l0=3.

Observation 9: The performance of l0=3 is better than l0=2 in HST- Scenario 1option 2with 3334Hz frequency offset, and the 3334Hz are also applicable for HST- Scenario 1option 2 for 500km/h at30 kHz SCS.

Proposal 1:  DMRS configuraton(1+1+1) with l0=3 is more applicable for NR HST.

Proposal 2:  The following HST single tap channel models for 500km/h are approved by RAN4.

  - Tunnel channel model

| Parameter | Value |
| --- | --- |
|  | Scenario 3 |
| Ds | 300 m |
| Dmin | 2 m |
| v | 500km/h |
| fd | 15kHz SCS: 1944Hz 30kHz SCS: 3334Hz |

  - Open space

| Parameter | Value | value |
| --- | --- | --- |
|  | Scenario 1 | Scenario 1a |
| Ds | 1000 m | 700 m |
| Dmin | 50 m | 150 m |
| v | 500km/h | 500km/h |
| fd | 15kHz SCS: 1750Hz 30kHz SCS: 3334Hz | 15kHz SCS: 1944Hz or 1750Hz 30kHz SCS: 3334Hz |

- References

- R4-1910128,Way forward on NR HST BS demodulation requirements,Huawei, HiSilicon,RAN4#92

- R4-1908120,Discussion and initial simulation results for NR HST PUSCH,Samsung, RAN4#92

- Annex

Table 5-1 Simulationassumption for HST PUSCH for 350km/h with CP-OFDM

| Parameter | value |
| --- | --- |
|  | FR1 |
| Transform precoding | Disabled |
| Number of Tx | 1 |
| Number of Rx | 2 |
| Number of layers | 1 |
| Transmission scheme | Identity matrix (TPMI index 0) |
| symbols length | 14 |
| start symbol index | 0 |
| Time domain resource allocation type | type A |
| MCS index | 2 |
| PTRS | Not configured |
| Timing offset | 0 |
| Frequency offset | 0 |
| Code block group, Frequency hopping, Limited buffer rate matching | Disabled |
| Number of HARQ transmissions | 4 |
| RV | {0,2,3,1} |
| Testing metric | SNR @70% of maximum throughput |
| DMRS type | type 1 |
| Number of DMRS symbols | 1+1+1 |
| Front loaded DMRS start symbol | l0= 3; l0=2 |
| SCS and BW | 15kHz: 10MHz; 30kHz: 40MHz |
| Frequency domain resource | Full applicable test bandwidth |
| Carrier frequency (GHz) | For 15kHz SCS: 2.1GHz For 30kHz SCS: 3.6GHz |
| fd | For 15kHz SCS: 1340Hz For 30kHz SCS: 2334Hz |
| Propagation condition | HST- Scenario 3(Tunnel channel model)(Ds=300m, Dmin=2m, v=350km/h); HST -Scenario 1option 1(Open space) (Ds=1000m, Dmin=50m, v=350km/h); HST -Scenario 1option 2(Open space)(Ds=700m, Dmin=150m, v=350km/h) |

Table 5-1 Simulationassumption for HST PUSCH for 500km/h with CP-OFDM

| Parameter | value |
| --- | --- |
|  | FR1 |
| Transform precoding | Disabled |
| Number of Tx | 1 |
| Number of Rx | 2 |
| Number of layers | 1 |
| Transmission scheme | Identity matrix (TPMI index 0) |
| symbols length | 14 |
| start symbol index | 0 |
| Time domain resource allocation type | type A |
| MCS index | 2 |
| PTRS | Not configured |
| Timing offset | 0 |
| Frequency offset | 0 |
| Code block group, Frequency hopping, Limited buffer rate matching | Disabled |
| Number of HARQ transmissions | 4 |
| RV | {0,2,3,1} |
| Testing metric | SNR @70% of maximum throughput |
| DMRS type | type 1 |
| Number of DMRS symbols | 1+1+1 |
| Front loaded DMRS start symbol | l0= 3; l0=2 |
| SCS and BW | 15kHz: 10MHz; 30kHz: 40MHz |
| Frequency domain resource | Full applicable test bandwidth |
| Carrier frequency (GHz) | For 15kHz SCS: 2.1GHz For 30kHz SCS: 3.6GHz |
| fd | For 15kHz SCS: - Option 1: 1944Hz - Option5: 1750Hz For 30kHz SCS: - Option 1: 3334Hz - Option 3: 3000Hz |
| Propagation condition | HST- Scenario 3(Tunnel channel model)(Ds=300m, Dmin=2m, v=500km/h); HST -Scenario 1option 1(Open space) (Ds=1000m, Dmin=50m, v=500km/h); HST -Scenario 1option 2(Open space)(Ds=700m, Dmin=150m, v=500km/h) |