3GPP TSG-RAN WG4 Meeting #92bis                                                          R4-1910985

# Chongqing, China, 14th–18th Oct., 2019

Source: 	CATT

Title: 	Discussion on PUSCH performance requirement with 30% throughput metric

Agenda item:	8.18.2.1

Document for:	Agreement

- Introduction

In RAN #84meeting, a new WID [1] on NR performance requirement enhancement was agreed with the following objectives of PUSCH requirements for 30% TP test point.

BS demodulation requirements: - To study the tests coverage of PUSCH requirements for 30% TP test point, limited test cases will be introduced if the existing test coverage is insufficient.

This paper will provide our view on PUSCH performance requirement with 30% throughput testing metric.

- Discussion

The contribution [2] indicates HARQ processes can bring much higher throughput gain at low SNR compared to that at high SNR. Given that the SNR for PUSCH with 70% throughput metric is much higher than that for PUSCH with 30% throughput metric, the HARQ processing capability of the BS probably cannot be effectively verified without a lower SNR in the case of 70% throughput metric. That is to say, the existing test case for PUSCH with 70% throughput metric is insufficient for testing the HARQ processing capability of the BS.

Proposal 1: It is necessary to introduce PUSCH with 30%throughput metric tosignificantly reflect the HARQ processing capability.

The 30% throughput metricis specified in Rel-8 LTE, which can be used as a reference for NR PUSCH with 30% throughputmetric. In terms of the minimum requirements for LTE PUSCHwith 30% throughput metric in TS 36.104, thenumber of Txwas required to be 1, and the number of Rx was required to be 2,4, and 8. When it comes to NR PUSCH with 30% throughput metric,To limit the test case, the number of TX and RX can be chosen as 1 and 2 respectively. Whilethe number of layer for NR PUSCHwith 30% throughput metric should be 1.

On the other hand, the HARQ process gain is largely associated with the MCS index. Only a low MCS index can effectively reflect the HARQ process gain (or soft combining gain). According to the minimum requirements for LTE PUSCH with 30% throughput metric specified in TS 36.104, the FRC A3-x and A4-x were required. The modulation order for FRC A3-x is QPSK, while that for FRC A4-x is 16QAM. The same modulation order (QPSK and 16QAM) is recommended to be used for NR PUSCH with 30% throughput metric. In terms of the existing MCS index (2, 16, 20) for NR PUSCH with 70% throughput metric, the MCS index 2, 16 and 20 denote QPSK, 16QAM and 64QAM respectively.It is thus proposed that the MCS index 2 and 16 can be used for NR PUSCH with 30% throughput metric. More specifically, the existing propagation conditions for MCS index 2 and 16 for NR PUSCH with 70% throughput metric can be used for NR PUSCH with 30% throughput metric.

Both CP-OFDM and DFT-s-OFDM for PUSCH are supported by NR standards. DFT-s-OFDM waveform with low MCS index can improve the cell coverage compared to CP-OFDM waveform. Therefore, DFT-s-OFDM for NR PUSCH with 30% throughput metric cannot be ingored. Meanwhile, CP-OFDM waveform is essential to be tested with 30% throughput metric because it is the new waveform designed for NR PUSCH. To conclude, it is proposed to adopt both CP-OFDM and DFT-s-OFDM for NR PUSCH with 30% throughput metric.

To limit test cases, selecting CBW per SCS is sufficient to reflect the HARQ processing capability. The same CBW and SCS combination can be used for CP-OFDM and DFT-s-OFDM. In the test cases for NR DFT-s-OFDM with 70% throughput metric, the 15kHz SCS for 5 MHz CBW(25PRB) and 30kHz SCS for 10MHz CBW(24 PRB) for FR1, and 60kHz SCS for 50MHz CBW (30 PRB) and 120kHz SCS for 50MHz CBW (30 PRB) for FR2 are required. It is thus proposed that the same SCS and CBW combinations can be used for NR PUSCH with 30% throughput metric for CP-OFDM and DFT-s-OFDM.The other test parameters for NR PUSCHwith 30% throughput metric can reuse the test parameters for NR PUSCHwith 70% throughput metric[3].Based on the above discussions, the test parameters for NR PUSCHwith 30% throughput metric are summarized in Table A5-1 in Annex.

Proposal 2: RAN4 can adopt the test parameters for NR PUSCHwith 30% throughput metricin Table A5-1 and Table A5-2 in Annex.

The evaluation results for NR PUSCH with 30% throughput metric based on proposed test parameters in Table A5-1 and Table A5-2 are shown in Table 2-1, Table 2-2, Table 2-3 and Table 2-4.

Table 2-1 Evaluation results for NR PUSCH with 30% throughput metric with CP-OFDM at 4GHz

| waveform | Carrier Freqency(GHz) | Tx/Rx | Time domain resource allocation type | SCS and BW | MCS level | Propagation condition | DMRS | metric | CATT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CP-OFDM | 4 | 1T2R | type A | 15kHz, 5MHz | MCS2 | TDLB100-400 | DMRS 1+1 | 70% TP | -4.59 |
| CP-OFDM | 4 | 1T2R | type A | 15kHz, 5MHz | MCS2 | TDLB100-400 | DMRS 1+1 | 30% TP | -9.01 |
| CP-OFDM | 4 | 1T2R | type A | 15kHz, 5MHz | MCS16 | TDLC300-100 | DMRS 1+1 | 70% TP | 7.99 |
| CP-OFDM | 4 | 1T2R | type A | 15kHz, 5MHz | MCS16 | TDLC300-100 | DMRS 1+1 | 30% TP | 0.53 |
| CP-OFDM | 4 | 1T2R | type A | 30kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1+1 | 70% TP | -4.77 |
| CP-OFDM | 4 | 1T2R | type A | 30kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1+1 | 30% TP | -9.25 |
| CP-OFDM | 4 | 1T2R | type A | 30kHz, 10MHz | MCS16 | TDLC300-100 | DMRS 1+1 | 70% TP | 7.27 |
| CP-OFDM | 4 | 1T2R | type A | 30kHz, 10MHz | MCS16 | TDLC300-100 | DMRS 1+1 | 30% TP | 0.44 |
| CP-OFDM | 4 | 1T2R | type B | 15kHz, 5MHz | MCS2 | TDLB100-400 | DMRS 1+1 | 70% TP | [TBD] |
| CP-OFDM | 4 | 1T2R | type B | 15kHz, 5MHz | MCS2 | TDLB100-400 | DMRS 1+1 | 30% TP | [TBD] |
| CP-OFDM | 4 | 1T2R | type B | 15kHz, 5MHz | MCS16 | TDLC300-100 | DMRS 1+1 | 70% TP | 7.93 |
| CP-OFDM | 4 | 1T2R | type B | 15kHz, 5MHz | MCS16 | TDLC300-100 | DMRS 1+1 | 30% TP | 0.49 |
| CP-OFDM | 4 | 1T2R | type B | 30kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1+1 | 70% TP | -4.82 |
| CP-OFDM | 4 | 1T2R | type B | 30kHz, 10MHz | MCS2 | TDLB100-400 | DMRS 1+1 | 30% TP | -9.33 |
| CP-OFDM | 4 | 1T2R | type B | 30kHz, 10MHz | MCS16 | TDLC300-100 | DMRS 1+1 | 70% TP | 7.21 |
| CP-OFDM | 4 | 1T2R | type B | 30kHz, 10MHz | MCS16 | TDLC300-100 | DMRS 1+1 | 30% TP | 0.37 |

Table 2-2 Evaluation results for NR PUSCH with 30% throughput metric with DFT-s-OFDM at 4GHz

| waveform | Carrier Freqency(GHz) | Tx/Rx | Time domain resource allocation type | SCS and BW | MCS level | Propagation condition | DMRS | metric | CATT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | 4 | 1T2R | type A | 15kHz, 5MHz (25PRB) | MCS2 | TDLB100-400 | DMRS 1+1 | 70% TP | -4.41 |
| DFT-s-OFDM | 4 | 1T2R | type A | 15kHz, 5MHz (25PRB) | MCS2 | TDLB100-400 | DMRS 1+1 | 30% TP | -8.99 |
| DFT-s-OFDM | 4 | 1T2R | type A | 30kHz, 10MHz (24 PRB) | MCS2 | TDLB100-400 | DMRS 1+1 | 70% TP | -4.53 |
| DFT-s-OFDM | 4 | 1T2R | type A | 30kHz, 10MHz (24 PRB) | MCS2 | TDLB100-400 | DMRS 1+1 | 30% TP | -9.18 |
| DFT-s-OFDM | 4 | 1T2R | type B | 15kHz, 5MHz (25PRB) | MCS2 | TDLB100-400 | DMRS 1+1 | 70% TP | -4.38 |
| DFT-s-OFDM | 4 | 1T2R | type B | 15kHz, 5MHz (25PRB) | MCS2 | TDLB100-400 | DMRS 1+1 | 30% TP | -8.99 |
| DFT-s-OFDM | 4 | 1T2R | type B | 30kHz, 10MHz (24 PRB) | MCS2 | TDLB100-400 | DMRS 1+1 | 70% TP | -4.6 |
| DFT-s-OFDM | 4 | 1T2R | type B | 30kHz, 10MHz (24 PRB) | MCS2 | TDLB100-400 | DMRS 1+1 | 30% TP | -9.28 |

Table 2-3 Evaluation results for NR PUSCH with 30% throughput metric with CP-OFDM at 30GHz

| waveform | Carrier Freqency(GHz) | Tx/Rx | Time domain resource allocation type | SCS and BW | MCS level | Propagation condition | DMRS | metric | CATT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CP-OFDM | 30 | 1T2R | type B | 60kHz, 50MHz | MCS2 | TDLA30-300 | DMRS 1 | 70% TP | -4.49 |
| CP-OFDM | 30 | 1T2R | type B | 60kHz, 50MHz | MCS2 | TDLA30-300 | DMRS 1 | 30% TP | -8.93 |
| CP-OFDM | 30 | 1T2R | type B | 60kHz, 50MHz | MCS16 | TDLA30-300 | DMRS 1 | 70% TP | 9 |
| CP-OFDM | 30 | 1T2R | type B | 60kHz, 50MHz | MCS16 | TDLA30-300 | DMRS 1 | 30% TP | 0.98 |
| CP-OFDM | 30 | 1T2R | type B | 120kHz, 50MHz | MCS2 | TDLA30-300 | DMRS 1 | 70% TP | -4.33 |
| CP-OFDM | 30 | 1T2R | type B | 120kHz, 50MHz | MCS2 | TDLA30-300 | DMRS 1 | 30% TP | -8.19 |
| CP-OFDM | 30 | 1T2R | type B | 120kHz, 50MHz | MCS16 | TDLA30-300 | DMRS 1 | 70% TP | 7.81 |
| CP-OFDM | 30 | 1T2R | type B | 120kHz, 50MHz | MCS16 | TDLA30-300 | DMRS 1 | 30% TP | 1.16 |

Table 2-4 Evaluation results for NR PUSCH with 30% throughput metric with DFT-s-OFDM at 30GHz

| waveform | Carrier Freqency(GHz) | Tx/Rx | Time domain resource allocation type | SCS and BW | MCS level | Propagation condition | DMRS | metric | CATT |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DFT-s-OFDM | 30 | 1T2R | type B | 60kHz, 50MHz (30 PRB) | MCS2 | TDLA30-300 | DMRS 1 | 70% TP | -3.93 |
| DFT-s-OFDM | 30 | 1T2R | type B | 60kHz, 50MHz (30 PRB) | MCS2 | TDLA30-300 | DMRS 1 | 30% TP | -8.68 |
| DFT-s-OFDM | 30 | 1T2R | type B | 120kHz, 50MHz (30 PRB) | MCS2 | TDLA30-300 | DMRS 1 | 70% TP | [TBD] |
| DFT-s-OFDM | 30 | 1T2R | type B | 120kHz, 50MHz (30 PRB) | MCS2 | TDLA30-300 | DMRS 1 | 30% TP | [TBD] |

- Conclusion

In this contribution, we discuss the test parameters for NR PUSCH with 30% throughput metric and give our proposals as follows.

Proposal 1: It is necessary to introduce PUSCH with 30% throughput metric to significantly reflect the HARQ processing capability.

Proposal 2: RAN4 can adopt the test parameters for NR PUSCHwith 30% throughput metricin Table A5-1 and Table A5-2 in Annex.

- References

- RP-191587,New WID on NR performance requirement enhancement, China Telecom,RAN#84

- R4-1906994,Remaining issues on NR PUSCH,NTT DOCOMO, INC.,RAN4#91

- R4-1904717,WF on NR PUSCH demodulation requirements, Nokia, Nokia Shanghai Bell, RAN4#90Bis

- Annex

Table A5-1 Test Parameters for CP-OFDM with 30% throughput metric

| Parameter | value |  |
| --- | --- | --- |
|  | FR1 | FR2 |
| Transform precoding | Disabled | Disabled |
| Number of Tx | 1 | 1 |
| Number of Rx | 2 | 2 |
| Number of layers | 1 | 1 |
| Transmission scheme | Identity matrix (TPMI index 0) | Identity matrix (TPMI index 0) |
| DMRS type | type 1 | type 1 |
| Number of DMRS symbols | 1+1 | 1 |
| symbols length | 14 | 10 |
| start symbol index | 0 | 0 |
| Time domain resource allocation type | type A and B | type B |
| Frequency domain resource | Full applicable test bandwidth | Full applicable test bandwidth |
| MCS index | 2, 16 | 2, 16 |
| Carrier frequency (GHz) | 4 | 30 |
| Propagation condition | MCS 2: TDL-B 100ns, 400 Hz MCS 16: TDL-C 300ns, 100 Hz | MCS 2: TDL-A 30ns, 300Hz MCS 16: TDL-A 30ns, 300Hz |
| SCS and BW | 15kHz: 5MHz; 30kHz: 10MHz; | 60kHz: 50MHz; 120kHz: 50MHz; |
| PTRS | Not configured | QPSK: Not configured 16 and 64 QAM: (Configured with KPTRS =2, LPTRS =1) |
| Timing offset | 0 | 0 |
| Frequency offset | 0 | 0 |
| Code block group, Frequency hopping, Limited buffer rate matching | Disabled | Disabled |
| Number of HARQ transmissions | 4 | 4 |
| Testing metric | SNR @30% of maximum throughput | SNR @30% of maximum throughput |

Table A5-2 Test Parameters for DFT-s-OFDM with 30% throughput metric

| Parameter | value |  |
| --- | --- | --- |
|  | FR1 | FR2 |
| Transform precoding | enabled | enabled |
| Number of Tx | 1 | 1 |
| Number of Rx | 2 | 2 |
| Number of layers | 1 | 1 |
| DMRS type | type 1 | type 1 |
| Number of DMRS symbols | 1+1 | 1 |
| symbols length | 14 | 10 |
| start symbol index | 0 | 0 |
| Time domain resource allocation type | type A and B | type B |
| Frequency domain resource | 15kHz: 25 PRB; 30kHz: 24 PRB (middle of test BW) | 60kHz: 30 PRB; 120kHz: 30 PRB (middle of test BW) |
| MCS index | 2 | 2 |
| Carrier frequency (GHz) | 4 | 30 |
| Propagation condition | MCS 2: TDL-B 100ns, 400 Hz | MCS 2: TDL-A 30ns, 300Hz |
| SCS and BW | 15kHz: 5 MHz; 30kHz: 10 MHz | 60kHz: 50MHz; 120kHz: 50MHz |
| PTRS | Not configured | Not configured |
| Timing offset | 0 | 0 |
| Frequency offset | 0 | 0 |
| Code block group, Frequency hopping, Limited buffer rate matching | Disabled | Disabled |
| Number of HARQ transmissions | 4 | 4 |
| Testing metric | SNR @30% of maximum throughput | SNR @30% of maximum throughput |