3GPP TSG-RAN WG4 Meeting #92bis	R4-1911017

Chongqing , China, 14th – 18th Oct 2019

Agenda item:       8.18.1.3

Source:                Intel Corporation

Title:	                     Simulation results for PMI reporting with larger number of TX ports

Document for:     Discussion

# Introduction

In NR performance requirement enhancement WI, one of the objectives is for RAN4 to define requirements for larger than 8 TX ports. In RAN4#92 introducing PMI requirements for larger than 8 ports was discussed and WF [1] was agreed.

- Type of requirements - PMI only - Frequency range - FR1 - Number of Tx ports - Option 1: 16, 32 - Option 2: 16 - Codebook type - Type I single panel codebook - PMI tests: - Option 1: Cover wideband PMI and subband in different tests, not duplicate all the tests - Option 2: Only Wideband - Test metric: - As baseline, Relative throughput ratio between following PMI and random PMI at SNR point corresponding to 90% TP with follow PMI Test Configuration - CHBW/SCS: - 10MHz/15kHz for FDD - 40MHz/30kHz for TDD - Antenna correlation: - XP high (as baseline) - Propagation condition: - TDLA30-5 (as baseline) - Codebook construction: - 16 Tx ports: (N1, N2) = (8, 1) or (4, 2) - 32 Tx ports (if PMI reporting requirement for 32 Tx ports will be defined): (N1, N2) = (4, 4) or (8, 2) or (16, 1) - Number Rx antennas: - 2Rx, 4Rx - Rank: - Option 1: up to rank 4 for 4Rx, and up to rank 2 for 2Rx - Option 2: up to rank 2 - MCS: - Up to 64QAM

Interested companies were encouraged to provide simulation results in RAN4#92bis. In this contribution we present initial simulation results for PMI test cases with larger than 8 TX ports.

# Discussion

## Simulation Results with 16 TX ports

Simulation assumptions for results with 16 TX ports are provided in Table 1 below.

Table 1: Simulation parameters

| Parameter | Value |
| --- | --- |
| CBW/SCS | 10MHz/15KHz |
| Antenna configuration | 16x2; X-Pol High 16x4; X-Pol High |
| Channel Model | TDLA30-5Hz TDLB100-5Hz |
| Codebook Construction (N1,N2) | (4,2) |
| PDSCH Type | Type A |
| MCS | MCS16, 16QAM |
| Number of MIMO layers | 1 2 |
| PMI reporting | Wideband Subband |

### Single layer PMI tests

Figure 1: Wideband PMI results 16x2

Figure 2: Subband PMI results 16x2

Figure 3: Wideband PMI results 16x4

Figure 4: Subband PMI results 16x4

Observation #1: With subband PMI, 16Tx 1 layer the throughput with random PMI is very small at the test point making the test irrelevant.

Proposal #1: For defining tests where metric is ratio of throughput between follow PMI and random PMI, the throughput ratio at test point should be reasonable

### Dual layer PMI tests

Figure 5: Subband PMI results 16x2

Figure 6: Subband PMI results 16x4

Observation #2: Throughput ratio for subband PMI with 16Tx are reasonable with 2 layers.

Proposal #2: Define subband PMI tests with 16 TX with 2 layers

## Simulation Results with 32 TX ports

Simulation assumptions for results with 32 TX ports are provided in Table 1 below.

Table 1: Simulation parameters

| Parameter | Value |
| --- | --- |
| CBW/SCS | 10MHz/15KHz |
| Antenna configuration | 32x2; X-Pol High 32x4; X-Pol High |
| Channel Model | TDLA30-5Hz TDLB100-5Hz |
| Codebook Construction (N1,N2) | (4,4) |
| PDSCH Type | Type A |
| MCS | MCS20, 64QAM |
| Number of MIMO layers | 2 |
| PMI reporting | Wideband Subband |

### Dual layer PMI tests

Figure 7: Wideband PMI results 32x2

Figure 8: Subband PMI results 32x2

Figure 9: Wideband PMI results 32x4

Figure 10: Subband PMI results 32x4

Observation #3: With subband PMI, 32Tx 2 layer the throughput with random PMI is very small at the test point making the test irrelevant.

Proposal #3: Define wideband PMI tests with 32 TX with 2 layers

# Conclusion

In this paper we have provided initial simulation results for PMI test cases with 16, 32 TX ports. We recommend that RAN4 considers them in finalizing the test case configurations for PMI tests with larger than 8 TX ports. Our proposals are summarized below:

Observation #1: With subband PMI, 16Tx 1 layer the throughput with random PMI is very small at the test point making the test irrelevant.

Proposal #1: For defining tests where metric is ratio of throughput between follow PMI and random PMI, the throughput ratio at test point should be reasonable

Observation #2: Throughput ratio for subband PMI with 16Tx are reasonable with 2 layers.

Proposal #2: Define subband PMI tests with 16 TX with 2 layers

Observation #3: With subband PMI, 32Tx 2 layer the throughput with random PMI is very small at the test point making the test irrelevant.

Proposal #3: Define wideband PMI tests with 32 TX with 2 layers

# Reference

- R4-1910017, “Way forward on PMI reporting requirements for Tx ports larger than 8 and up to 32”, Ericsson