3GPP TSG-RAN WG4 Meeting #92bis	R4-1911019

Chongqing , China, 14th – 18th Oct 2019

Agenda item:       8.9.2.1

Source:                Intel Corporation

Title:	                     On test feasibility and methodology for high reliability metric for NR URLLC

Document for:     Discussion

# Introduction

In RAN4#92 test feasibility for URLLC was discussed and the following agreements were made in [1]:

Evaluation of the feasibility for testing reliability of 99.999% - The goal of the evaluation should be to provide estimates of test time(s) and confidence level(s) for the methods in slide 3 (or other methods if identified) so that a conclusion on test feasibility, confidence level, reasonable test time and test methodology can be reached. - Key factors impacting evaluation - Target reliability level: 99.999% - Possible confidence level (CL): {95%, 98%, 99%, 99.5%} - Number of errors Test Methodology for testing reliability of 99.999% - Option 1: Run one long test to reach reliability of 99.999% - Option 2: Run multiple small duration tests with different channel seeds and combine the results later - Option 3: Leave to RAN5 to define (for UE only) - Option 3a: Use RAN5 method for BS if applicable - Option 4: Extrapolation to low BLER for some test cases - Option 5: UTRA BLER test methodology from TS 25.141/34.121-1 as a starting point - Other options are not precluded Test methodology for low latency - Test metric of delay is not feasible for low latency test in RAN4 - Low latency related features are feasible to be tested under the test metrics other than delay, such as throughput and/or BLER. Reliability and low latency - No combined performance requirements will be defined by meeting both reliability of 99.999% and low latency requirements

In this contribution we present our views on test feasibility and test methodology for testing high reliability metric of 99.999% for URLLC.

# Discussion

## Test time analysis

In [2] we provided some initial test time analysis in static and fading channels based on methodology used in RAN5 for LTE [3].

Table 1 below provides the minimum number of samples needed for 95% CL to achieve 99.999% reliability in static channel conditions based on the method used in RAN5. The actual testing time depends on other test configurations such as duplexing mode, SCS, single packet transmission time. For the assumption of slot based PDSCH transmission in FDD mode, in Table 1 the minimum test times for 15 KHz and 30 KHz SCS are presented.

Table 1: Minimum test time for 10-5 BLER

| Number of errors | Minimum number of samples | Minimum test time (mins) |  |
| --- | --- | --- | --- |
|  |  | SCS 15KHz | SCS 30KHz |
| 0 | 299573 | 5 | 3 |
| 1 | 399428 | 7 | 4 |
| 2 | 547461 | 10 | 5 |
| 3 | 674977 | 12 | 6 |
| 4 | 792479 | 14 | 7 |
| 5 | 903735 | 16 | 8 |
| 6 | 1010611 | 17 | 9 |
| 7 | 1114195 | 19 | 10 |
| 8 | 1215187 | 21 | 11 |
| 9 | 1314066 | 22 | 11 |
| 10 | 1411183 | 24 | 12 |
| 20 | 2323296 | 39 | 20 |
| 30 | 3178125 | 53 | 27 |
| 40 | 4003379 | 67 | 34 |
| 50 | 4809750 | 81 | 41 |
| 100 | 8691165 | 145 | 73 |

Based on the methodology used in RAN5, in order to achieve 99.999% reliability with 95% confidence level, the minimum testing time in static channel conditions is feasible for up to at least 10 observed errors.

Observation #1: In static channel conditions by limiting the number of errors to 10 or fewer, it might be feasible to test reliability of 99.999% with 95% confidence interval.

The RAN5 methodology in [3] for determining minimum test time is based on 95% confidence level. In [1] it was agreed to evaluate testing time for higher confidence levels. In table 2 below we provide minimum number of samples of error free transmission for different confidence levels

Table 2: Minimum number of samples for different CL

| Confidence Level | Minimum Number of samples |
| --- | --- |
| 95 | 299573 |
| 98 | 391202 |
| 99 | 460517 |
| 99.5 | 529832 |

In case minimum test times for other confidence levels need to be evaluated for more number of observed errors, RAN4 should request some input from RAN5 on extending the methodology in TS 36.521-1 [3] to higher confidence levels.

Proposal #1: To evaluate test time for confidence levels other than 95%, RAN4 requests RAN5 to provide methodology to extend to higher confidence levels

In [2] we provided some initial analysis on test time in fading channel conditions based on methodology from RAN5 in [3]. The minimum number of slots for target BLER of 10%, 1%, 0.1%, 0.01% was derived for TDLB100-400Hz channel and the results are presented in table 3 below.

Table 3: Minimum test time to achieve target BLER in fading conditions

| Target BLER | Minimum number of samples | Minimum test time (sec) |  |
| --- | --- | --- | --- |
|  |  | SCS 15KHz | SCS 30KHz |
| 0.1 | 608 | 1 | 1 |
| 0.01 | 4473 | 5 | 3 |
| 0.001 | 15377 | 16 | 8 |
| 0.0001 | 155864 | 156 | 78 |

Extrapolating the results, the minimum testing time for 10-5 target BLER in the same channel conditions would likely be around 26 minutes for 15KHz SCS and 13 minutes for 30KHz SCS.

Observation #2: The minimum testing time for 10-5 target BLER in high Doppler channel conditions (TDLB100-400Hz) would likely be around 26 minutes for 15KHz SCS and 13 minutes for 30KHz SCS.

For channel conditions with low Doppler and high antenna correlation, the minimum testing time would be significantly higher, likely in the order of hours and doesn’t seem feasible.

Observation #3: The testing time for target reliability of 99.999% in low Doppler or high antenna channel correlation conditions doesn’t seem feasible

Based on the observations above, we recommend to introduce testing of high reliability of 99.999% in static channel conditions.

Proposal # 2: Prioritize test cases in static channel for high reliability target of 99.999%

In a link level simulation environment 1 million samples can be run in parallel with multiple seeds. But in actual test environment the test would likely need to be run in one shot for a long duration. It would be helpful if TE vendors could comment on the testing feasibility for large number of samples.

Proposal #3: RAN4 takes TE vendor’s input in deciding feasibility based on test time

## Test methodology

In [1] several options for test methodology were listed.

Test Methodology for testing reliability of 99.999%

- Option 1: Run one long test to reach reliability of 99.999%

- Option 2: Run multiple small duration tests with different channel seeds and combine the results later

- Option 3: Leave to RAN5 to define (for UE only)

- Option 3a: Use RAN5 method for BS if applicable

- Option 4: Extrapolation to low BLER for some test cases

- Option 5: UTRA BLER test methodology from TS 25.141/34.121-1 as a starting point

Option1, 2

In order to achieve the target reliability of 99.999%, option 1 and option 2 would be relevant. However, we recommend option 1. We believe that it is feasible to run a long simulation in static channel conditions with reasonable test time. For reasons listed in Section 2.1, we don’t think it’s practical to run long tests in fading channel to achieve the target 99.999% reliability.

For option 2, the actual test time for running small duration tests might end up taking longer time than one long test considering test setup times. Also, the target reliability cannot be guaranteed by running tests in small duration and combining results later.

Observation #4: Running multiple tests for shorter duration might end up taking longer time taking test setup into account

Proposal #4: In order to achieve target reliability of 99.999% run one long test

Option 3

The option of leaving it to be defined and discussed RAN5 doesn’t help achieve the objective of the WI. The WI objective included RAN4 evaluating the feasibility and introducing a methodology for testing high reliability metric.

Observation #5: Leaving the discussion and definition to RAN5 doesn’t help achieve performance part objective of WI

Option 4

In cases where we cannot run long enough tests to achieve the target reliability, extrapolation of results could be a suitable option, for e.g., in fading channel conditions. But the procedure and methodology for extrapolation of results for low target BLER from higher BLER points should be discussed in detailed.

Proposal #5: If defining methodology based on extrapolation to low BLER is determined to be feasible, discuss detailed methodology in RAN4

Option 5

The UTRA BLER test methodology is similar to that in LTE for static channel conditions. However the methodology is very different for fading propagation conditions. We prefer to use methodology defined in LTE as it might be more suitable for NR.

Proposal #6: Use RAN5 methodology from LTE as it might be better suited for NR

# Conclusion

In this paper we discuss the test feasibility and methodology for target reliability of 99.999% and have the following observations and proposals:

Observation #1: In static channel conditions by limiting the number of errors to 10 or fewer, it might be feasible to test reliability of 99.999% with 95% confidence interval.

Proposal #1: To evaluate test time for confidence levels other than 95%, RAN4 requests RAN5 to provide methodology to extend to higher confidence levels

Observation #2: The minimum testing time for 10-5 target BLER in high Doppler channel conditions (TDLB100-400Hz) would likely be around 26 minutes for 15KHz SCS and 13 minutes for 30KHz SCS.

Observation #3: The testing time for target reliability of 99.999% in low Doppler or high antenna channel correlation conditions doesn’t seem feasible

Proposal # 2: Prioritize test cases in static channel for high reliability target of 99.999%

Proposal #3: RAN4 takes TE vendor’s input in deciding feasibility based on test time

Observation #4: Running multiple tests for shorter duration might end up taking longer time taking test setup into account

Proposal #4: In order to achieve target reliability of 99.999% run one long test

Observation #5: Leaving the discussion and definition to RAN5 doesn’t help achieve performance part objective of WI

Proposal #5: If defining methodology based on extrapolation to low BLER is determined to be feasible, discuss detailed methodology in RAN4

Proposal #6: Use RAN5 methodology from LTE as it might be better suited for NR

# Reference

- R4-1910044, “Way forward on URLLC test methodology”, Huawei, HiSilicon

- R4-1908198, “Discussion on test feasibility for URLLC requirements”, Intel Corporation

- TS 36.521-1