3GPP TSG RAN WG4 Meeting #92-Bis	R4-1911152

Chongqing, CN, 14-18 Oct. 2019

Agenda item:	7.3.3

Source:	SoftBank Corp.

Title:	TP for TR 36.716-03-01: LTE CA_1-11-42

Document for:	Approval

1	Introduction

3B LTE CA of CA_1-11-42 was approved in RAN#85[1]. This TP is to capture the basic aspects for LTE CA.

2.  Reference

[1]  RP-192083  WID revision: Rel-16 LTE inter-band CA for 3 bands DL with 1 band UL; rapporteur: Huawei

Annex.

[Unchanged Parts Skipped]

5.X	CA_1-11-42

5.X.1	Channel bandwidths per operating band for CA

Table 5.X.1-1: Inter-band CA operating bands

| E-UTRA Operating Band | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
| 11 | 1427.9 MHz | – | 1447.9 MHz | 1475.9 MHz | – | 1495.9 MHz | FDD |
| 42 | 3400 MHz | – | 3600 MHz | 3400 MHz | – | 3600 MHz | TDD |

Table 5.X.1-2: Supported E-UTRA bandwidths per CA configuration for 3DL inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_1A-11A-42A | - | 1 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  |  | 11 |  |  | Yes | Yes |  |  |  |  |
|  |  | 42 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_1A-11A-42C | - | 1 |  |  | Yes | Yes | Yes | Yes | 70 | 0 |
|  |  | 11 |  |  | Yes | Yes |  |  |  |  |
|  |  | 42 | See CA_42C Bandwidth Combination Set 0 in Table 5.6A.1-1 |  |  |  |  |  |  |  |

5.X.2	∆TIB and ∆RIB values

For CA_1-11-42, the TIB,c and RIB,c values are given in the tables below.

Table 5.X.2-1: ΔTIB,c

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_1-11-42 | 1 | 0.3 |
|  | 11 | 0.4 |
|  | 42 | 0.8 |

Table 5.X.2-2: ΔRIB

| Inter-band CA Configuration | E-UTRA Band | ΔRIB [dB] |
| --- | --- | --- |
| CA_1-11-42 | 1 | 0 |
|  | 11 | 0 |
|  | 42 | 0.5 |

[Unchanged Parts Skipped]