3GPP TSG-RAN WG4 Meeting #92bis	R4-1911186

# Chongqin, China, 14th– 18th Oct, 2019

Source: 	Huawei, HiSilicon

Title:	TP for TR 37.823: PRB Alignment

Agenda Item:	7.10.2

Document for:	Approval

# 1	Introduction

In last RAN4#92 meeting, a WF for Rel-16 LTE-MTC coexistence with NR is agreed [1], where it specifies that the PRB alignment should be captured in the TR. This paper proposes the TP for this topic.

# 2	References

[1] R4-1907849, WF for Rel-16 LTE-MTC coexistence with NR

# 3	Text Proposal

## <Start of Text Proposal>

## 7.2 PRB alignment

### 7.2.1 Impact of LTE-MTC Narrowband Operation

LTE-MTC UEs reuse legacy LTE PSS/SSS/PBCH for initial cell selection, which occupies the centre 72 subcarriers in the system BW excluding the DC subcarrier. After MSG1 in the random access procedure, the UE may use only 6 PRBs in a narrowband for transmissions and receives if no frequency-hopping is configured. Otherwise hopping between multiple narrowbands may be performed. The mapping between the narrowband and the PRB is defined in 3GPP TS 36.211 Section 6.2.7. Figure 7.2-1 illustrates the mapping for system BW up to 5 MHz.

Figure 7.2-1 Mapping between LTE PRB and Narrowband

For up to at least Rel-15 LTE-MTC operation in NR in-band, the number of NR PRBs reserved for LTE-MTC should be large enough to accommodate all the PRBs within the LTE transmission BW configuration.

### 7.2.2 Intuitive Analysis

Consider the case of SCS=15 kHz for NR. A total of +1 consecutive NR PRBs in the DL need to be reserved for LTE-MTC owing to the unused DC subcarrier. The set of allowed values for  is given by 3GPP TS 36.104. Depending on the parity of , the DC subcarrier is either in the middle of a LTE PRB or in between two LTE PRBs. It’s easily seen that a LTE PRB can be aligned with NR PRBs if the following conditions are satisfied, and at most half of the PRBs in the LTE system BW can be aligned:

- The LTE DC subcarrier coincides with the lowest (highest) subcarrier, i.e. subcarrier index 0 (11) of a NR PRB if is even;

- Or the LTE DC subcarrier coincides with the middle subcarrier, i.e. subcarrier index 6 (5) of a NR PRB if is odd;

- The LTE PRB is on the lower (upper) side of the LTE DC subcarrier.

In the UL, the LTE DC subcarrier is a valid subcarrier that belongs to a PRB and a 7.5 kHz frequency shift is employed to mitigate the effect of carrier leakage. Hence, the LTE PRBs may be aligned with the NR PRBs only if the operating NR band also supports 7.5 kHz frequency shift. Furthermore, owing to the restrictions of TX-RX frequency separation as defined in 3GPP TS 36.101, the aforementioned conditions on the DL DC subcarrier should be met so that the LTE DL PRBs on the lower side of the DC subcarrier are aligned with the NR DL PRBs. As a result, all of the LTE UL PRBs can be aligned with the NR UL PRBs.

Figure 7.2-2 illustrates an example of 1.4 MHz LTE carrier inside a NR carrier. In the DL, the LTE DC subcarrier coincides with the subcarrier 0 of an NR RB. As a result, LTE RB 0, 1 and 2 are aligned with NR RBs. In the UL, all LTE RBs are aligned with NR RBs after 7.5 kHz shift. Additionally, Figure 7.2-3 illustrates an example of 3 MHz LTE carrier inside a NR carrier. In the DL, the LTE DC subcarrier coincides with the subcarrier 6 of an NR RB. As a result, LTE RB 0-6 are aligned with NR RBs. In the UL, all LTE RBs are aligned with NR RBs after 7.5 kHz shift.

Figure 7.2-2: Example PRB alignment for a 1.4 MHz LTE carrier (having one LTE-MTC NB) inside a NR carrier

Figure 7.2-3: Example PRB alignment for a 3 MHz LTE carrier (having two LTE-MTC NBs) inside a NR carrier

### 7.2.3 Quantitative Analysis

Let NRBNRand NRBLTE denote the maximum number of RBs in the NR and LTE carrier, respectively. Let FREFNR and FcLTE denotes the RF reference frequency for NR and carrier frequency for LTE, respectively. It’s known that:

- NR RF reference frequency FREFNR corresponds to subcarrier 0 of RB NRBNR2 if NRBNR is even, and it corresponds to subcarrier 6 of RB NRBNR2 if NRBNR is odd;

- LTE carrier frequency FcLTE corresponds to the DC subcarrier with extra 7.5 kHz offset in the UL. In the DL, it locates between two adjacent RBs if NRBLTE is even, and it locates in the middle of a RB if NRBLTE is odd.

In order to optimize resource utilization, it is desired to align LTE PRBs with NR PRBs (in fact at most half of the PRBs in the DL and all in the UL as described in Section 7.2.2). In the following, we further formulate the conditions described in Section 7.2.2 based on different NR channel raster sizes.

7.2.3.1 NR 100 kHz Channel Raster

For subcarrier alignment (SCS=15 kHz), the NR reference frequency and LTE carrier frequency should satisfy the following condition:

100m-100n=15k          (7.2.3.1-1)

with:

- m being an integer identifying the LTE carrier on the 100 kHz raster, i.e. FcLTE=100m kHz;

- n being an integer identifying the NR carrier on the 100 kHz raster, i.e. FREFNR=100n kHz;

- k being an integer representing the subcarrier offset from the NR RF reference frequency.

From Eq. (7.2.3.1-1), it can be obtained that:

k=20p , with p being an integer 		(7.2.3.1-2).

We also have k>0 if the LTE carrier is on the upper side of the NR carrier and k<0 if the LTE carrier is on the lower side. Next we further derive the LTE carrier position in terms of RB offset as well as subcarrier index within a RB.

If NRBNR is even, the NR RF reference frequency FREFNR corresponds to subcarrier 0 of RB NRBNR2. Hence, we have:

- q≜k12, the RB offset from the NR RB containing the subcarrier corresponding to FREFNR

- r≜k%12,  0≤r≤11,  the index of the subcarrier corresponding to FcLTE

With these definitions, we re-write k=20p as:

20p=12q+r,            (7.2.3.1-3)

or r=4(5p-3q)		(7.2.3.1-4).

It can be seen that valid values for subcarrier index r are: 0, 4 and 8. Note that the value of RB offset q is limited by the NR channel bandwidth, i.e. -NRBNR2≤q≤NRBNR2. For ensuring subcarrier alignment between the LTE-MTC carrier and the NR carrier, the task is to find a suitable solution of (p,q,r) that satisfies Eq. (7.2.3.1-4). Further considering PRB alignment, the condition #1 as described in Section 7.2.2 may be met if additional constraint of r=0 is added. However, the condition #2 as described in Section 7.2.2 can never be met since r=6 or 5 is not valid.

On the other hand, if NRBNR is odd, the NR RF reference frequency FREFNR corresponds to subcarrier 6 of RB NRBNR2.

Taking this offset into account, we have q=k+612 for RB offset and r≜(k+6)%12 for subcarrier index. And Eq. (7.2.3.1-2) can be re-write as:

20p+6=12q+r,            (7.2.3.1-5)

or r=2*(10p-3*(2q-1))	(7.2.3.1-6).

It can be seen that valid values for subcarrier index r are: 2, 6 and 10. For ensuring subcarrier alignment between the LTE-MTC carrier and the NR carrier, the task is to find a suitable solution of (p,q,r) that satisfies Eq. (7.2.3.1-6). Further considering PRB alignment, the condition #2 as described in Section 7.2.2 may be met if additional constraint of r=6 is added. However, the condition #1 as described in Section 7.2.2 can never be met since r=0 or 11 is not valid.

7.2.3.2 NR 15 kHz Channel Raster

Corresponding to the E-UTRA bands that support LTE-MTC, most of the NR counterparts use 100 kHz channel raster. So far, only band n41 uses SCS-based channel raster. For the sake of PRB alignment, only SCS=15 kHz is considered. Considering subcarrier alignment, the NR reference frequency and LTE carrier frequency should satisfy the following condition:

100m-15n=15k,		(7.2.3.2-1)

with:

- m being an integer identifying the LTE carrier on the 100 kHz raster, i.e. FcLTE=100m kHz;

- n being an integer identifying the NR carrier on the 15 kHz raster, i.e. FREFNR=15n kHz;

- k being an integer representing the subcarrier offset from the NR RF reference frequency.

Rewrite Eq. (7.2.3.1-1) as:

100m=15(n+k).                   (7.2.3.2-2)

This means that the LTE carrier should be on a raster of 300 kHz, i.e. m=3p, with p being an integer. Together with Eq. (7.2.3.2-2), we have:

k=20p-n.                   (7.2.3.2-3)

Reusing the definition of RB offset q and subcarrier index r in Section 7.2.3.1, we have:

20p-n=12q+r, if NRBNR is even,           (7.2.3.2-4)

20p-n+6=12q+r, if NRBNR is odd.           (7.2.3.2-5)

Note that the values of p and q are limited by the NR channel bandwidth, e.g. -NRBNR2≤q≤NRBNR2. For ensuring subcarrier alignment between the LTE-MTC carrier and the NR carrier, the task is to find a suitable solution of (p,q,r) that satisfies Eq. (7.2.3.2-4) or Eq. (7.2.3.2-5), for a given NR reference frequency, i.e. fixed n. Further considering PRB alignment, the conditions on subcarrier index r as described in Section 7.2.2 should be met as additional constraints. Note that such a solution may not always exist.

## <End of Text Proposal>