3GPP TSG-RAN WG4 Meeting #92-Bis                 	R4-1911187

Chongqing, China, 14th – 18th October 2019

Source:	Nokia, Nokia Shanghai Bell

Title:	NR Rel-15 BS demodulation simulations on TDD patterns

Agenda item:	6.12.2.2.1

Document for:	Discussion

Introduction

In the RAN4#92 meeting, it was proposed to remove the TDD pattern requirement for PUSCH minimum performance requirements. Some concerns were raised, on the impact of different TDD pattern configurations on the simulation outcomes used to define the minimum performance requirements.

Possible impacts from the TDD pattern were expected to be due to edge effects on the first and last symbols in a PUSCH TTI, and different observed fading channel realisations depending on the pause between PUSCH TTIs (i.e., more correlation between retransmissions of UL TBs in the case of more FDD-like TDD patterns, which might impact RV combining performance).

Hence, the way forward on NR Rel-15 BS demodulation requirements [1], calls for studies on this subject.

In this contribution we provide the results of our simulation campaign on the TDD pattern impact on PUSCH minimum performance requirements.

Simulation results

Unless otherwise stated in the following tables, the simulation setup follows the standard PUSCH setup, as captured in [2].

Table 1: Simulation setup and results summary (1T2R for all)

|  |  |  |  |  |  |  |  |  | SINR @ 70% TP 30kHz SCS |  |  |  |  | DIFF |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MCS | PRB | Channel |  | CBW | SCS | DMRS |  | PTRS | 10U | 1D9U | 7D1S2U | 9D1U |  | 10U vs. 1D9U | 10U vs. 7D1S2U | 10U vs. 9D1U |
| 2 | 273 | TDLB100 | Pusch CP-OFDM | 100MHz | 30kHz | 1+1 | TypeB | NoPtrs | -4.83 | -4.83 | -4.88 | -4.86 |  | 0.00 | -0.05 | -0.03 |
| 16 | 273 | TDLC300 | Pusch CP-OFDM | 100MHz | 30kHz | 1+1 | TypeB | NoPtrs | 7.60 | 7.60 | 7.70 | 7.67 |  | 0.00 | 0.10 | 0.07 |
| 20 | 273 | TDLA30 | Pusch CP-OFDM | 100MHz | 30kHz | 1+1 | TypeB | NoPtrs | 10.56 | 10.56 | 10.46 | 10.50 |  | 0.00 | -0.10 | -0.06 |
| 2 | 273 | TDLB100 | Pusch CP-OFDM | 100MHz | 30kHz | 1+1 | TypeB | NoPtrs | -10.85 | -10.85 | -10.88 | -10.87 |  | 0.00 | -0.03 | -0.02 |
| 16 | 273 | TDLC300 | Pusch CP-OFDM | 100MHz | 30kHz | 1+1 | TypeB | NoPtrs | 0.34 | 0.34 | 0.34 | 0.34 |  | 0.00 | 0.00 | 0.00 |
| 20 | 273 | TDLA30 | Pusch CP-OFDM | 100MHz | 30kHz | 1+1 | TypeB | NoPtrs | 3.46 | 3.46 | 3.33 | 3.35 |  | 0.00 | -0.13 | -0.11 |
| 2 | 273 | TDLB100 | Pusch CP-OFDM | 100MHz | 30kHz | 1+1 | TypeB | NoPtrs | -0.99 | -0.99 | -0.97 | -0.95 |  | 0.00 | 0.02 | 0.04 |
| 16 | 273 | TDLC300 | Pusch CP-OFDM | 100MHz | 30kHz | 1+1 | TypeB | NoPtrs | 16.30 | 16.30 | 16.15 | 16.19 |  | 0.00 | -0.15 | -0.11 |
| 20 | 273 | TDLA30 | Pusch CP-OFDM | 100MHz | 30kHz | 1+1 | TypeB | NoPtrs | 19.59 | 19.59 | 19.61 | 19.66 |  | 0.00 | 0.02 | 0.07 |
| 2 | 24 | TDLB100 | Pusch DFTs OFDM | 100MHz | 30kHz | 1+1 | TypeB | NoPtrs | -5.03 | -5.03 | -5.00 | -5.05 |  | 0.00 | 0.03 | -0.02 |
|  |  |  |  |  |  |  |  |  | SINR @ 70% TP 15kHz SCS |  |  |  |  | DIFF |  |  |
| MCS | PRB | Channel |  | CBW | SCS | DMRS |  | PTRS | 5U | 1D4U | 3D1S1U | 4D1U |  | 5U vs. 1D4U | 5U vs. 3D1S1U | 5U vs. 4D1U |
| 2 | 106 | TDLB100 | Pusch CP-OFDM | 20MHz | 15kHz | 1+1 | TypeA | NoPtrs | -3.94 | -3.94 | -3.78 | -3.78 |  | 0.00 | 0.16 | 0.16 |
| 16 | 106 | TDLC300 | Pusch CP-OFDM | 20MHz | 15kHz | 1+1 | TypeA | NoPtrs | 7.79 | 7.79 | 7.82 | 7.82 |  | 0.00 | 0.03 | 0.03 |
| 20 | 106 | TDLA30 | Pusch CP-OFDM | 20MHz | 15kHz | 1+1 | TypeA | NoPtrs | 9.67 | 9.67 | 9.66 | 9.66 |  | 0.00 | -0.01 | -0.01 |
|  |  |  |  |  |  |  |  |  | SINR @ 70% TP 120kHz SCS |  |  |  |  | DIFF |  |  |
| MCS | PRB | Channel |  | CBW | SCS | DMRS |  | PTRS | 5U | 1D4U | 3D1S1U | 4D1U |  | 5U vs. 1D4U | 5U vs. 3D1S1U | 5U vs. 4D1U |
| 2 | 66 | TDLA30 | Pusch CP-OFDM | 100MHz | 120kHz | 1+0 | TypeB | NoPtrs | -5.45 | -5.45 | -5.45 | -5.45 |  | 0.00 | 0.00 | 0.00 |
| 16 | 66 | TDLA30 | Pusch CP-OFDM | 100MHz | 120kHz | 1+0 | TypeB | NoPtrs | 7.25 | 7.25 | 7.30 | 7.30 |  | 0.00 | 0.05 | 0.05 |
| 20 | 66 | TDLA30 | Pusch CP-OFDM | 100MHz | 120kHz | 1+0 | TypeB | NoPtrs | 9.73 | 9.73 | 9.80 | 9.80 |  | 0.00 | 0.07 | 0.07 |

Conclusion

In this contribution we provided the results of our simulation campaign on the TDD pattern impact on PUSCH minimum performance requirements. These results will be discussed in our companion discussion paper on remaining NR Rel-15 BS demodulation performance requirement general issues.

- References

- R4-1910063, Way forward on NR Rel-15 BS demodulation requirements, Huawei, HiSilicon, RAN4#92.

- R4-1907241, WF on NR PUSCH demodulation requirements, Nokia, Nokia Shanghai Bell, RAN4#91.