3GPP TSG-RAN WG4 Meeting #92-Bis                 	R4-1911193

Chongqing, China, 14th – 18th October 2019

Source:	Nokia, Nokia Shanghai Bell

Title:	NR Rel-16 HST BS demodulation PRACH simulation results

Agenda item:	8.17.2.3

Document for:	Discussion

Introduction

In the RAN4#92 meeting, a plethora of possible configurations and environments for NR_HST BS demodulation PRACH performance requirements were proposed [1].

Main clusters of parameterization options are found around, maximum doppler shift, channel modelling, PRACH formats, as well as test metrics.

In this contribution we provide the results of our simulation campaign on possible performance requirement parametrization in PRACH.
These results will be discussed in our companion discussion paper on remaining NR_HST Rel-16 BS demodulation PRACH performance requirements.

PRACH Simulation results

Unless otherwise noted, the same setting as in Rel-15 non-HST PRACH demodulation performance are used [2], and the preamble configurations follow this table

Table 1: Default preamble configurations.

| Burst format | Restricted Set | Ncs | Logical sequence index | preamble id v |
| --- | --- | --- | --- | --- |
| 0 | Type A | 15 | 384 | 0 |
|  | Type B | 15 | 30 | 30 |
| Short | - | 15 kHz: 23 30 kHz: 46 60 and 120 kHz: 69 | 0 | 0 |

The performance metric is SNR @ False alarm probability < 0.1% and missed detection < 99%.

FR1 1T2R

Table 1: PRACH FR1 1T2R.

| PRACH FR1 1T2R |  | Propagation condition | Frequency offset (Hz) | Nokia |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Ideal |  |
| Format 0(TypeA) | 1.25kHz | AWGN | 0 | -15.31 |  |
|  |  | AWGN | 1340 | -15.2 |  |
|  |  | TDLC300-100 | 400 | -7.52 |  |
| Format 0(TypeB) | 1.25kHz | AWGN | 1875 | -13.78 |  |
|  |  | AWGN | 1944 | -13.84 |  |
|  |  | AWGN | 2334 | -14.61 |  |
|  |  | TDLC300-100 | 400 | -6.93 |  |
| Format A2 | 15kHz | AWGN | 0 | -14.67 |  |
|  |  |  | 1250 | -13.1 |  |
|  |  |  | 1400 | -12.73 |  |
|  |  |  | 1500 | -12.32 |  |
|  |  |  | 1944 | -10.52 |  |
|  |  | TDLC300-100 | 400 | -7.21 |  |
|  | 30kHz | AWGN | 0 | -14.53 |  |
|  |  |  | 2300 | -13.21 |  |
|  |  |  | 2600 | -12.78 |  |
|  |  |  | 3000 | -12.23 |  |
|  |  |  | 3334 | -11.61 |  |
|  |  | TDLC300-100 | 400 | -7.83 |  |
| Format A3 | 15kHz | AWGN | 0 | -16.03 |  |
|  |  |  | 1250 | -15.15 |  |
|  |  |  | 1400 | -14.95 |  |
|  |  |  | 1500 | -14.79 |  |
|  |  |  | 1944 | -13.94 |  |
|  |  | TDLC300-100 | 400 | -8.44 |  |
|  | 30kHz | AWGN | 0 | -15.86 |  |
|  |  |  | 2300 | -15.15 |  |
|  |  |  | 2600 | -14.95 |  |
|  |  |  | 3000 | -14.69 |  |
|  |  |  | 3334 | -14.39 |  |
|  |  | TDLC300-100 | 400 | -9.27 |  |
| Format B4 | 15kHz | AWGN | 0 | -18.72 |  |
|  |  |  | 1250 | -17.04 |  |
|  |  |  | 1400 | -16.69 |  |
|  |  |  | 1500 | -16.37 |  |
|  |  |  | 1944 | -14.63 |  |
|  |  | TDLC300-100 | 400 | -11.23 |  |
|  | 30kHz | AWGN | 0 | -18.56 |  |
|  |  |  | 2300 | -17.21 |  |
|  |  |  | 2600 | -16.87 |  |
|  |  |  | 3000 | -16.31 |  |
|  |  |  | 3334 | -15.69 |  |
|  |  | TDLC300-100 | 400 | -11.94 |  |
| Format C2 | 15kHz | AWGN | 0 | -14.73 |  |
|  |  |  | 1250 | -13.11 |  |
|  |  |  | 1400 | -12.71 |  |
|  |  |  | 1500 | -12.42 |  |
|  |  |  | 1944 | -10.58 |  |
|  |  | TDLC300-100 | 400 | -7.11 |  |
|  | 30kHz | AWGN | 0 | -14.53 |  |
|  |  |  | 2300 | -13.18 |  |
|  |  |  | 2600 | -12.81 |  |
|  |  |  | 3000 | -12.21 |  |
|  |  |  | 3334 | -11.64 |  |
|  |  | TDLC300-100 | 400 | -7.77 |  |

FR1 1T4R

Table 1: PRACH FR1 1T4R.

| PRACH FR1 1T4R |  | Propagation condition | Frequency offset (Hz) | Nokia |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Ideal |  |
| Format 0(TypeA) | 1.25kHz | AWGN | 0 | -17.56 |  |
|  |  | AWGN | 1340 | -17.51 |  |
|  |  | TDLC300-100 | 400 | -12.67 |  |
| Format 0(TypeB) | 1.25kHz | AWGN | 1875 | -15.99 |  |
|  |  | AWGN | 1944 | -15.96 |  |
|  |  | AWGN | 2334 | -16.77 |  |
|  |  | TDLC300-100 | 400 | -12.06 |  |
| Format A2 | 15kHz | AWGN | 0 | -17.14 |  |
|  |  |  | 1250 | -15.59 |  |
|  |  |  | 1400 | -15.07 |  |
|  |  |  | 1500 | -14.83 |  |
|  |  |  | 1944 | -12.95 |  |
|  |  | TDLC300-100 | 400 | -12.16 |  |
|  | 30kHz | AWGN | 0 | -16.95 |  |
|  |  |  | 2300 | -15.68 |  |
|  |  |  | 2600 | -15.26 |  |
|  |  |  | 3000 | -14.66 |  |
|  |  |  | 3334 | -14.65 |  |
|  |  | TDLC300-100 | 400 | -12.12 |  |
| Format A3 | 15kHz | AWGN | 0 | -18.12 |  |
|  |  |  | 1250 | -17.33 |  |
|  |  |  | 1400 | -17.07 |  |
|  |  |  | 1500 | -16.94 |  |
|  |  |  | 1944 | -16.04 |  |
|  |  | TDLC300-100 | 400 | -13.21 |  |
|  | 30kHz | AWGN | 0 | -17.98 |  |
|  |  |  | 2300 | -17.37 |  |
|  |  |  | 2600 | -17.11 |  |
|  |  |  | 3000 | -16.82 |  |
|  |  |  | 3334 | -16.53 |  |
|  |  | TDLC300-100 | 400 | -13.19 |  |
| Format B4 | 15kHz | AWGN | 0 | -20.76 |  |
|  |  |  | 1250 | -19.22 |  |
|  |  |  | 1400 | -18.78 |  |
|  |  |  | 1500 | -18.47 |  |
|  |  |  | 1944 | -16.7 |  |
|  |  | TDLC300-100 | 400 | -15.73 |  |
|  | 30kHz | AWGN | 0 | -20.64 |  |
|  |  |  | 2300 | -19.33 |  |
|  |  |  | 2600 | -18.9 |  |
|  |  |  | 3000 | -18.37 |  |
|  |  |  | 3334 | -17.74 |  |
|  |  | TDLC300-100 | 400 | -15.69 |  |
| Format C2 | 15kHz | AWGN | 0 | -17.1 |  |
|  |  |  | 1250 | -15.46 |  |
|  |  |  | 1400 | -15.14 |  |
|  |  |  | 1500 | -14.74 |  |
|  |  |  | 1944 | -12.91 |  |
|  |  | TDLC300-100 | 400 | -12.21 |  |
|  | 30kHz | AWGN | 0 | -16.98 |  |
|  |  |  | 2300 | -15.67 |  |
|  |  |  | 2600 | -15.25 |  |
|  |  |  | 3000 | -14.69 |  |
|  |  |  | 3334 | -13.98 |  |
|  |  | TDLC300-100 | 400 | -12.17 |  |

FR1 1T8R

Table 1: PRACH FR1 1T8R.

| PRACH FR1 1T8R |  | Propagation condition | Frequency offset (Hz) | Nokia |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | Ideal |  |
| Format 0(TypeA) | 1.25kHz | AWGN | 0 | -19.67 |  |
|  |  | AWGN | 1340 | -19.62 |  |
|  |  | TDLC300-100 | 400 | -16.63 |  |
| Format 0(TypeB) | 1.25kHz | AWGN | 1875 | -18.08 |  |
|  |  | AWGN | 1944 | -18.05 |  |
|  |  | AWGN | 2334 | -18.79 |  |
|  |  | TDLC300-100 | 400 | -15.92 |  |
| Format A2 | 15kHz | AWGN | 0 | -18.76 |  |
|  |  |  | 1250 | -17.14 |  |
|  |  |  | 1400 | -16.71 |  |
|  |  |  | 1500 | -16.37 |  |
|  |  |  | 1944 | -14.63 |  |
|  |  | TDLC300-100 | 400 | -15.26 |  |
|  | 30kHz | AWGN | 0 | -18.62 |  |
|  |  |  | 2300 | -17.28 |  |
|  |  |  | 2600 | -16.86 |  |
|  |  |  | 3000 | -16.26 |  |
|  |  |  | 3334 | -15.67 |  |
|  |  | TDLC300-100 | 400 | -14.88 |  |
| Format A3 | 15kHz | AWGN | 0 | -19.85 |  |
|  |  |  | 1250 | -19.02 |  |
|  |  |  | 1400 | -18.83 |  |
|  |  |  | 1500 | -18.66 |  |
|  |  |  | 1944 | -17.81 |  |
|  |  | TDLC300-100 | 400 | -16.6 |  |
|  | 30kHz | AWGN | 0 | -19.71 |  |
|  |  |  | 2300 | -18.98 |  |
|  |  |  | 2600 | -18.81 |  |
|  |  |  | 3000 | -18.53 |  |
|  |  |  | 3334 | -18.15 |  |
|  |  | TDLC300-100 | 400 | -15.98 |  |
| Format B4 | 15kHz | AWGN | 0 | -22.44 |  |
|  |  |  | 1250 | -20.85 |  |
|  |  |  | 1400 | -20.45 |  |
|  |  |  | 1500 | -20.12 |  |
|  |  |  | 1944 | -18.34 |  |
|  |  | TDLC300-100 | 400 | -19.02 |  |
|  | 30kHz | AWGN | 0 | -22.29 |  |
|  |  |  | 2300 | -20.92 |  |
|  |  |  | 2600 | -20.59 |  |
|  |  |  | 3000 | -19.95 |  |
|  |  |  | 3334 | -19.38 |  |
|  |  | TDLC300-100 | 400 | -18.6 |  |
| Format C2 | 15kHz | AWGN | 0 | -18.73 |  |
|  |  |  | 1250 | -17.15 |  |
|  |  |  | 1400 | -16.72 |  |
|  |  |  | 1500 | -16.4 |  |
|  |  |  | 1944 | -14.59 |  |
|  |  | TDLC300-100 | 400 | -15.5 |  |
|  | 30kHz | AWGN | 0 | -18.64 |  |
|  |  |  | 2300 | -17.27 |  |
|  |  |  | 2600 | -16.88 |  |
|  |  |  | 3000 | -16.26 |  |
|  |  |  | 3334 | -15.66 |  |
|  |  | TDLC300-100 | 400 | -14.92 |  |

Conclusion

In this contribution we provided the results of our simulation campaign on possible performance requirement parametrization in PRACH for NR_HST Rel-16 BS demodulation performance requirements.
These results are discussed in detail in our companion discussion paper.

- References

- R4-1910128, Way forward on NR HST BS demodulation requirements, 3GPP TSG-RAN WG4 Meeting #92, Huawei, HiSilicon..

- R4-1813757, Way forward on NR PRACH demodulation requirements, Ericsson, RAN4#88-Bis