3GPP TSG-RAN WG4 Meeting #92-Bis                 	R4-1911196

Chongqing, China, 14th – 18th October 2019

Source:	Nokia, Nokia Shanghai Bell

Title:	NR Rel-16 performance requirement enhancement BS demodulation simulation results

Agenda item:	8.18.2

Document for:	Discussion

Introduction

In the RAN4#92 meeting, the need for NR Rel-16 performance requirement enhancement BS demodulation simulations was captured in the adhoc minutes [1].
Simulation input was requested to decide on test cases for 30%TPUT requirements (especially to decide, which MCS are most likely to “profit” from supplementary 30%TPUT testing), and the performance requirement input for the agreed 2T2R MCS12 test cases is needed.

In this contribution we provide the results of our simulation campaigns for these two topics. The results will be discussed in a separate companion discussion contribution.

Simulation results - 30%TPUT

Unless stated otherwise in the following tables, the simulation setup follows the standard PUSCH setup, as captured in [2].

Table 1: Simulation summary 30%TPUT SINR.

| ID | Ant | MCS | PRB | Channel |  |  | CBW | SCS | DMRS |  | PTRS | SINR @ 30% TP | SINR @ 70% TP | Mean Nbr of Harq ReTX @ 30% TP | Mean Nbr of Harq ReTX @ 70% TP | TPUT curve non-linearity (qualitative) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1002 | 1T2R | 2 | 52 | TDLB100 | CP-OFDM | FR1 | 10MHz | 15kHz | 1+1 | TypeA | NoPtrs | -8.84 | -4.68 | 2.13 | 0.43 | low |
| 1004 | 1T2R | 16 | 52 | TDLC300 | CP-OFDM | FR1 | 10MHz | 15kHz | 1+1 | TypeA | NoPtrs | 0.54 | 7.79 | 2.24 | 0.43 | high |
| 1008 | 1T2R | 2 | 106 | TDLB100 | CP-OFDM | FR1 | 40MHz | 30kHz | 1+1 | TypeA | NoPtrs | -9.04 | -4.80 | 2.22 | 0.43 | low-mid |
| 1010 | 1T2R | 16 | 106 | TDLC300 | CP-OFDM | FR1 | 40MHz | 30kHz | 1+1 | TypeA | NoPtrs | 0.51 | 7.60 | 2.21 | 0.43 | mid-high |
| 1025 | 1T2R | 2 | 132 | TDLA30 | CP-OFDM | FR2 | 100MHz | 60kHz | 1+0 | TypeB | PTRS | -9.03 | -4.88 | 2.09 | 0.43 | none-low |
| 1026 | 1T2R | 16 | 132 | TDLA30 | CP-OFDM | FR2 | 100MHz | 60kHz | 1+0 | TypeB | PTRS | 0.89 | 8.87 | 2.22 | 0.43 | mid-high |
| 1030 | 1T2R | 2 | 66 | TDLA30 | CP-OFDM | FR2 | 100MHz | 120kHz | 1+0 | TypeB | PTRS | -9.09 | -5.45 | 1.98 | 0.43 | none |
| 1031 | 1T2R | 16 | 66 | TDLA30 | CP-OFDM | FR2 | 100MHz | 120kHz | 1+0 | TypeB | PTRS | 0.84 | 7.80 | 2.15 | 0.43 | mid |
| 1202 | 1T2R | 2 | 25 | TDLB100 | DFTs OFDM | FR1 | 5MHz | 15kHz | 1+1 | TypeA | NoPtrs | -9.11 | -4.73 | 1.99 | 0.43 | low |
| 1204 | 1T2R | 2 | 24 | TDLB100 | DFTs OFDM | FR1 | 10MHz | 30kHz | 1+1 | TypeA | NoPtrs | -9.27 | -5.09 | 2.07 | 0.43 | none |

Notes: The simulation results table also contains the mean number of HARQ re-transmissions. At the 30% TPUT working point on average 2.1 reTx can be expected. This measured result is smaller than the theoretically expected one of 2.33, which is discussed in the companion contribution.
Furthermore, there is a column describing the TPUT curve non-linearity in qualitative measures. This result becomes clear, when looking at the TPUT vs. SINR curves in the following.

Figure 1: TPUT vs. SINR, for 1002 and 1004.

Figure 2: TPUT vs. SINR, for 1008 and 1010.

Figure 3: TPUT vs. SINR, for 1025 and 1026.

Figure 4: TPUT vs. SINR, for 1030 and 1031.

Figure 5: TPUT vs. SINR, for 1202 and 1204.

Simulation results - MCS 12

Unless stated otherwise in the following tables, the simulation setup follows the standard PUSCH setup, as captured in [2].

Table 1: Simulation summery 2T2R MCS12.

|  |  | PUSCH 2T2R |  |  | Nokia |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Ideal |
| FR2 with PTRS | 60kHz, 100MHz | MCS12 | TDLA30-300 | DMRS 1 | 13.10 |
|  |  |  |  | DMRS 1+1 | 11.13 |
|  | 120kHz, 100MHz | MCS12 | TDLA30-300 | DMRS 1 | 10.54 |
|  |  |  |  | DMRS 1+1 | 10.47 |
| FR2 without PTRS | 60kHz, 100MHz | MCS12 | TDLA30-300 | DMRS 1 | 12.27 |
|  |  |  |  | DMRS 1+1 | 10.69 |
|  | 120kHz, 100MHz | MCS12 | TDLA30-300 | DMRS 1 | 10.06 |
|  |  |  |  | DMRS 1+1 | 10.03 |
| FR2 with PTRS | 60kHz, 50MHz | MCS12 | TDLA30-300 | DMRS 1 | 12.94 |
|  |  |  |  | DMRS 1+1 | 11.06 |
|  | 120kHz, 50MHz | MCS12 | TDLA30-300 | DMRS 1 | 11.09 |
|  |  |  |  | DMRS 1+1 | 10.65 |
|  | 120kHz, 200MHz | MCS12 | TDLA30-300 | DMRS 1 | 10.67 |
|  |  |  |  | DMRS 1+1 | 10.54 |
| FR2 without PTRS | 60kHz, 50MHz | MCS12 | TDLA30-300 | DMRS 1 | 11.88 |
|  |  |  |  | DMRS 1+1 | 10.55 |
|  | 120kHz, 50MHz | MCS12 | TDLA30-300 | DMRS 1 | 10.50 |
|  |  |  |  | DMRS 1+1 | 10.13 |
|  | 120kHz, 200MHz | MCS12 | TDLA30-300 | DMRS 1 | 10.21 |
|  |  |  |  | DMRS 1+1 | 10.12 |

Conclusion

In this contribution we provided the results of our simulation campaigns for 30%TPTUT and FR2 2T2R MCS2. These results will be discussed in our companion discussion paper on remaining NR Rel-16 BS demodulation performance enhancement requirements.

- References

- R4-1910006, Ad Hoc minutes for NR Rel-15 and Rel-16 BS demodulation, Huawei, HiSilicon, RAN4#92.

- R4-1907241, WF on NR PUSCH demodulation requirements, Nokia, Nokia Shanghai Bell, RAN4#91.