3GPP TSG-RAN WG4 Meeting #92-Bis                                       R4-1911312

Chongqing, China, 14th – 18th Oct, 2019

Agenda Item:	6.10.8.3

Source:	MediaTek Inc.

Title:	Discussion on scheduling restriction due to L1-RSRP measurement

Document for:		Discussion

- Introduction

Scheduling restriction has been introduced for L1-RSRP measurement. With scheduling restrictions due to L1-RSRP measurement, the UE is not expected to transmit or receive PDCCH/PDSCH/CSI-RS for tracking/CSI-RS for CQI on RS for L1-RSRP measurement symbols to be measured for L1-RSRP measurement. In this paper, one clarification on “symbols to be measured” is discussed for L1-RSRP reporting.

- Clarification on “symbols to be measured” for L1-RSRP measurement

For scheduling restriction due to L1-RSRP reporting, it may have different interpretations on “symbols to be measured”. For example, it would depend on measurement resource and reporting type to decide which symbols to be measured for L1-RSRP measurement.

The reporting types can be periodic, semi-persistent, and aperiodic. For periodic reporting, it can only apply to periodic measurement resources, and naturally all the periodic resources configured for L1-RSRP measurement can be considered as“symbols to be measured” for L1-RSRP measurement.

For aperiodic reporting, it can apply to periodic, semi-persistent, and aperiodic measurement resources. It can observe that once the aperiodic reporting is triggered by DCI, UE shall report L1-RSRP on the allocated UL resources. In other words, it would assume UE already has the knowledge about the UE RX beam for the measurement resource (e.g. SSB#1) and it will not provide additional time for UE RX beam sweeping after the triggering of aperiodic reporting. Therefore, all the measurement resources configured for L1-RSRP measurement should be considered as“symbols to be measured”, even before the triggering of aperiodic reporting.

Observation 1: For aperiodic reporting for L1-RSRP, it doesn’t provide additional time for UE RX beam sweeping after the triggering of aperiodic reporting.

For the semi-persistent reporting, it can apply to periodic or semi-persistent measurement resources. Similarly as aperiodic reporting, once the semi-persistent reporting is triggered by MAC-CE or DCI, it would not provide additional time for UE RX beam sweeping, and thus the configured measurement resources should be considered as symbols to be measured.

Proposal 1: Scheduling restriction for L1-RSRP measurement should be based measurement resource configured for L1-RSRP measurement, instead of reporting types.

- Summary

In this paper, one clarification on “symbols to be measured” is discussed for the scheduling restriction due to L1-RSRP measurements. We have the following observations and proposals:

Observation 1: For aperiodic reporting for L1-RSRP, it doesn’t provide additional time for UE RX beam sweeping after the triggering of aperiodic reporting.

Proposal 1: Scheduling restriction for L1-RSRP measurement should be based measurement resource configured for L1-RSRP measurement, instead of reporting types.

References

[1] TS38.133, NR; Requirements for support of radio resource management, V15.7.0.