3GPP TSG-RAN WG4 Meeting #92bis                                                              R4-19XXXX

Chongqing, China, 14th-18th Oct, 2019

Source:	Samsung, Verizon

Title:	TP for TR 37.716-21-11: DC_13-48_n66

Agenda item:			9.4.2

Document for:	Approval

# 1. Introduction

In RAN#85 meeting, the revised WID on EN-DC for 3 bands DL with 2 bands UL (2 LTE band + 1 NR band) was approved [1], in which the latest EN-DC for 3 bands DL with 2 bands UL (2 LTE band + 1 NR band) was attached. This contribution is a text proposal for TR 37.716-21-11 to include DC_13A-48A_n66A, DC_13A-48B_n66A, DC_13A-48D_n66A and DC_13A-48E_n66A.

# 2. Reference

- RP-192087, Revised WID on EN-DC for 3bands DL with 2 bands UL (2 LTE band + 1 NR band), Huawei.

# 3. Text Proposal

<Start of Text Proposal>

## 5.1.x	DC_13-48_n66

### 5.1.x.1	Operating bands for DC

Table 5.1.x.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_13-48_n66 | CA_13-48 | n66 | No |

### 5.1.x.2	Configurations for DC

Table 5.1.x.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_13A-48A_n66A DC_13A-48B_n66A DC_13A-48D_n66A DC_13A-48E_n66A | DC_13A_n66A | CA_13A-48A CA_13A-48B CA_13A-48D CA_13A-48E | n66A |

### 5.1.x.3	Coexistence study

Co-existence studies of this 3DL/2UL DC configuration are already covered in the constituent fall-back modes, there is no own Rx impact on the 3rd band.

### 5.1.x.4	∆TIB and ∆RIB values

DC_13-48_n66 could reuse the TIB,c and RIB,c values of CA_13A-48A-66A  as given in the below tables.

Table 5.1.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_13-48_n66 | 13 | 0.3 |
|  | 48 | 0.8 |
|  | n66 | 0.6 |

Table 5.1.x.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_13-48_n66 | 13 | 0 |
|  | 48 | 0.5 |
|  | n66 | 0.2 |

### 5.1.x.5	REFSENS requirements

There is no additional requirement for this band combination.

<End of Text Proposal>