3GPP TSG-RAN WG4 Meeting #92bis    	R4-1911377

Chongqing, China, 14th – 18th Oct, 2019

Agenda item:	6.12.2.3.1

Source:	Samsung

Title:	Ideal and impairment simulation results for NR PUCCH with multi-slot

Document for:	Discussion

# 1	Introduction.

During RAN4#92 meeting, the related open issues on frequency hopping and simulation assumption for requirement of multi-slot NR PUCCH are approved in [1]. Based on the agreement, the ideal and impairment are provided for requirement finalization.

# 2	Simulation Results

In this subsection, based on the simulation assumption for PUCCH with multi-slot as follows, the ideal and impairment results are provided.

Table 1: Test parameters for PUCCH with multi-slot

| Parameters |  | PUCCH Format 1 |
| --- | --- | --- |
| Bandwidth/SCS |  | 40 MHz/ 30kHz |
| Antenna |  | 1T2R |
| Propagation condition |  | TDLC300-100 |
| Carrier frequency (GHz) |  | 4GHz |
| (#Bits, #Symbols, #PRBs) |  | (2, 14,1) |
| initialCyclicShift |  | 0 |
| StartingSymbolIndex |  | 0 |
| Index of orthogonal sequence (time-domain-OCC) |  | 0 |
| Inter/Intra-slot frequency hopping |  | Enabled inter-slot frequency hopping, disable for intra-slot frequency hopping |
| Number of slots |  | 2 |
| Test metric | DTX to ACK probability <1% | √ |
|  | Missed ACK probability < 1% | √ |
|  | NACK2ACK < 0.1% | √ |

Table 2: Ideal results for PUCCH with multi-slot

| Number of Tx antennas | Number of Rx antennas | Propagation conditions and correlation | Missed ACK probability (<1%) | NACK2ACK probability (<0.1%) |
| --- | --- | --- | --- | --- |
| 1 | 2 | TDLC300-100 Low | -9.4 | -8.6 |

Table 3: Impairment results for PUCCH with multi-slot

| Number of Tx antennas | Number of Rx antennas | Propagation conditions and correlation | Missed ACK probability (<1%) | NACK2ACK probability (<0.1%) |
| --- | --- | --- | --- | --- |
| 1 | 2 | TDLC300-100 Low | -7.4 | -6.6 |

# 3	Conclusion

In this contribution, the ideal and impairment results of PUCCH with multi-slot for requirement finalization are provided.

# References

R4-1907242, WF on multi-slot PUCCH performance requirements, ZTE.