3GPP TSG-RAN WG4 Meeting #92bis              	R4-1911382

Chongqing, China, 14th – 18th Oct, 2019

Agenda item:	7.13.4.1

Source:	Samsung

Title:	Discussion and simulation results for HST PUSCH in LTE Rel-16

Document for:	Discussion

# 1	Introduction

During HST PUSCH for LTE Rel-16 discussion in RAN4#92 meeting, some agreements for PUSCH are captured in the WF [1] as follows:

    - The following deployment scenario are considered:

      - Open space

        - Bi-directional (1st priority)

        - Uni-directional

      - Tunnel for multi-antennas

        - Bi-directional (1st priority)

        - Uni-directional

    - BS-Railway track distance (Dmin) and Initial distance of the train from BS (Ds/2):

      - Same as existing HST scenario 1 for open space and 3 for tunnel

      - Other options are not precluded

  - WF on PUSCH for HST evaluation (not for requirement)

    - Deployment scenarios and related parameters (e.g., Ds, Dmin, fd) will be decided based on evaluation results. The following parameters should be considered in the evaluation

    - Maximum Doppler shift will be based  on the outcome of simulation results and could be less than 2 times downlink Doppler shift

For initial evaluation about the impact of high Doppler shift, some test cases are introduced.

In this contribution, the initial results are presented based on initial simulation assumption.

# 2	Simulation results

## 2.1	Simulation assumption for PUSCH

As captured in the WF [1], there are four options Doppler value listed for initial evaluation purpose.

| Parameter | Value |  |  |  |
| --- | --- | --- | --- | --- |
|  | Open space |  | Tunnel for multi-antennas |  |
|  | Bi-directional (1st priority) | Uni-directional | Bi-directional (1st priority) | Uni-directional |
| Ds | 1000 m |  | 300 m |  |
| Dmin | 50m |  | 2m |  |
| V | 500km/h (lower speeds are not precluded) |  | 500km/h(Other options are not precluded) |  |
| fd | 1944Hz, 1894Hz,1844Hz,1794Hz,1744Hz,1694Hz |  |  |  |

n table 1, we provide the simulation assumptions for BS demodulation, which is reused in TR36.878.

Table 1: simulation assumptions for PUSCH demodulation performance evaluation

| Parameters | Unit | Values |
| --- | --- | --- |
| Bandwidth | MHz | 10 |
| MCS for PUSCH |  | QPSK 1/3 |
| Propagation condition and correlation matrix |  | AWGN |
| Antenna configuration |  | 1x2 |
| Reference receiver |  | MRC |
| Noise estimation |  | Practical |
| Time and frequency track |  | Practical |

Meanwhile, the existing test parameters for high speed train and FRC table specified in the 104 spec are provided for simulation results alignment purpose.

Table 2: Test parameters for high speed train

| Parameter | Value |
| --- | --- |
| Maximum number of HARQ transmissions | 4 |
| RV sequence | 0,2,3,1,0,2,3,1 |
| Uplink-downlink allocation for TDD | Configuration 1(2 :2) |
| Subframes in which PUSCH is transmitted | For FDD ubframe #0 and #8 in radio frames for which SFN mod 4 = 0 subframe #6 in radio frames for which SFN mod 4 = 1 subframe #4 in radio frames for which SFN mod 4 = 2 subframe #2 in radio frames for which SFN mod 4 = 3 For TDD Subframe #2 in each radio frame |
| Subframes in which PUCCH is transmitted (Note 1, Note 2) | For FDD Subframe #5 in radio frames For TDD Subframe # 3 in radio frames |
| Note 1: The configuration of PUCCH (format 2) is optional Note 2: The SNR values per antenna shall be set to -4.5dB and -1.5dB for Scenario 1 and Scenario 2, respectively |  |

Table 2: FRC parameters for performance requirements (QPSK 1/3)

| Reference Channel | A3-5 |
| --- | --- |
| Allocated resource blocks | 50 |
| DFT-OFDM Symbols per subframe | 12 |
| Modulation | QPSK |
| Code rate | 1/3 |
| Payload size (bits) | 5160 |
| Transport block CRC (bits) | 24 |
| Code block CRC size (bits) | 0 |
| Number of code blocks - C | 1 |
| Coded block size including 12bits trellis termination (bits) | 15564 |
| Total number of bits per sub-frame | 14400 |
| Total symbols per sub-frame | 7200 |

## 2.2	Performance requirement

In this subsection, the view on the requirement expect for maximum Doppler value.

- Duplex

The requirement with both TDD and FDD should be considered.

- Antenna configuration

As for antenna configuration, similar with LTE, for HST open space scenario, only requirement with 1x2 antenna configuration is specified, and with HST tunnel scenario, 1x1 antenna configuration considering the realistic operation.

- Measurement channel

As indicated in table 2,   with both PUSCH and PUCCH are configured. However, the configuration of PUCCH is optional.

- BW

Similar with Rel-14, the requirements should be specified for all the channel bandwidth 1.4/3/5/10/15/20.

Proposal 1: The follows should be introduced for HST PUSCH requirement in Rel-16

Duplex: TDD and FDD

Antenna configuration: 1x2 for open space scenario, 1x1 for tunnel scenario

Measurement channel: PUSCH, the configuration of PUCCH is optional

BW:  1.4/5/10/15/20 MHz

## 2.2	Simulation Results

In this subsection, the initial simulation results are provided. In Figure 1, we provide the ideal simulation results for PUSCH under in Open Space and Tunnel for bi-directional scenario with QPSK 1/3

Figure 1: Simulation results of BS demodulation performance with High Doppler shift value

In Figure 2, the normalization of TP is provided.

Figure 2: Normalization TP of BS demodulation performance with High Doppler shift value

Meanwhile, the target SNR with 70% TP are provided for both open space and tunnel for multi-antennas scenario.

| SCS, BW | MCS | Antenna Configuration | Doppler Value | RB | Test Metric | SNR |
| --- | --- | --- | --- | --- | --- | --- |
| 15KHz, 10MHz | QPSK 1/3 | 1x2 | Open space 1944Hz | 50 | 70% | -3.9dB |
| 15KHz, 10MHz | QPSK 1/3 | 1x2 | Open space 1944Hz | 50 | 70% | -3.8dB |

Proposal 2: 1944Hz frequency offset with QPSK 1/3 can be introduced for HST PUSCH requirement in Rel-16

# 3	Conclusion

In this contribution, the view of requirement with HST PUSCH in Rel-16 is provided. Meanwhile, the initial simulation result for alignment is presented.

Proposal 1: The follows should be introduced for HST PUSCH requirement in Rel-16

Duplex: TDD and FDD

Antenna configuration: 1x2 for open space scenario, 1x1 for tunnel scenario

Measurement channel: PUSCH, the configuration of PUCCH is optional

BW:  1.4/5/10/15/20 MHz

Proposal 2: 1944Hz frequency offset with QPSK 1/3 can be introduced for HST PUSCH requirement in Rel-16

# References

R4-1907747, Way forward on LTE HST BS demodulation performance requirement, NTT DOCOMO