3GPP TSG-RAN WG4 Meeting #92bis              	R4-1911384

Chongqing, China, 14th– 18th Oct, 2019

Agenda item:	8.9.2.1

Source:	Samsung

Title:	View on URLLC performance requirements in NR Rel-16

Document for:	Discussion

# 1	Introduction

During the last RAN4 meeting, based on the objective of URLLC WI, the test methodology for test metric of 99.999% reliability with testing time consideration, and test methodology for low latency requirements, are discussed. Some initial agreements are captured in the WF[1] as follow:

  - Evaluation of the feasibility for test reliability of 99.999%

    - Take the following factors into account to evaluate the feasibility of test 99.999% reliability

      - The goal of the evaluation should be to provide estimates of test time(s) and confidence level(s) for the methods in slide 3 (or other methods if identified) so that a conclusion on test feasibility, confidence level, reasonable test time and test methodology can be reached.

      - Key factors impacting evaluation

- Target reliability level: 99.999%

- Possible confidence level (CL): {95%, 98%, 99%, 99.5%}

- Number of errors

      - Other factors to describe

- The following factors may be relevant to the evaluation and assumptions used should be described when presenting results:

- Propagation condition (e.g., static, fading)

- Antenna configuration

- Number of retransmission

- Duplexing mode and TDD configuration

- Slot based transmission or non-slot based transmission

- MCS

- Rank

- PDSCH mapping type

- DMRS configuration

- CBW/SCS

  - Test methodology for test reliability of 99.999%

    - Test methodology

      - Option 1: Run one long test to reach reliability of 99.999%

      - Option 2: Run multiple small duration tests with different channel seeds and combine the results later

      - Option 3: Leave to RAN5 to define (for UE only)

      - Option 3a: Use RAN5 method for BS if applicable

      - Option 5: UTRA BLER test methodology from TS 25.141/34.121-1 as a starting point

      - Other options are not precluded

  - Test methodology for low latency

    - Test metric of delay is not feasible for low latency test in RAN4

    - Low latency related features are feasible to be tested under the test metrics other than delay, such as throughput and/or BLER.

  - Reliability and low latency

    - No combined performance requirements will be defined by meeting both reliability of 99.999% and low latency requirements

In this contribution, some initial simulations are performed to investigate the test time and reliability based on the URLLC feature.

# 2	Discussion

## 2.1	Test time factors

In this section, serval factors are considered for evaluate the feasibility of reliability of 99.999%. Some initial results are provided.

Number of error observations

As following, one example for investigating the number of error with targeting 10^(-5) BLER is provided.

Table 1: TDLA30-10, MCS0, 7 symbols, 15KHz SCS, 10MHz, 52RB without retransmission

| SNR | HARQ num | err number | BLER | total subframe |
| --- | --- | --- | --- | --- |
| 2 | 0 | 1 | 1.00E-05 | 100000 |
| 2 | 0 | 2 | 8.74E-06 | 228826 |
| 2 | 0 | 3 | 1.31E-05 | 228829 |
| 2 | 0 | 4 | 1.75E-05 | 228830 |
| 2 | 0 | 5 | 1.93E-05 | 258442 |

Table 2: TDLA30-10, MCS0, 7 symbols, 15KHz SCS, 10MHz, 52RB with retransmission

| SNR | HARQ num | err number | BLER | total subframe |
| --- | --- | --- | --- | --- |
| -7 | 3 | 1 | 5.83E-05 | 17165 |
| -7 | 3 | 2 | 1.88E-04 | 106555 |
| -7 | 3 | 3 | 1.00E-05 | 300000 |
| -7 | 3 | 4 | 9.70E-06 | 412403 |
| -7 | 3 | 5 | 1.06E-05 | 471280 |

As indicated in the table1, around 260000 subframes are needed to observe 5 number of errors with around targeting 10-5 BLER. If targeting at least 100 errors observed, it is expected that the number of subframes is about 52000000. If HARQ retransmission is considered as illustrated in table 2, more number of subframes is needed.

Meanwhile, in terms of simulation setup for obtaining the whole performance curve with SNR region, more simulation time are consumed.

Proposal 1: loose number of errors observation should be considered for high reliability requirement of URLLC

Propagation condition (e.g., static, fading)

As for BS demodulation test, there is no static condition. For NR Rel-15, there are three types channel model specified for demodulation requirement with TDLA30-10, TDLB100-400 and TDLC300-100, respectively. Normally, long coherent time is expected to increase the overall test time. In that sense, based on current channel, TDLB100-400 should be considered for URLLC 10^(-5) BLER testing.  As shown in table 3 and table 4, we compare the number of errors observation with 10^(-5) BLER target for TDLA30-10 and TDLB100-400 fading channel.

Figure 1: BLER performance with mini-slot 2symbols, SCS 15 KHz, BW 10 MHz, 52RB with MCS 8

Table 3 Number of error for TDLA 30-10, 7symbols, SCS15KHz, BW 10MHz, MCS0, 1T8R

| SNR | HARQ num | err number | BLER | total subframe |
| --- | --- | --- | --- | --- |
| -15 | 3 | 1 | 0.00001 | 100000 |
| -15 | 3 | 2 | 7.24042E-06 | 276227 |
| -15 | 3 | 3 | 9.74542E-06 | 307837 |
| -15 | 3 | 4 | 0.00001 | 400000 |

Table 4 Number of error for TDLB100-400, 7symbols, SCS15KHz, BW 10MHz, MCS0, 1T8R

| SNR | HARQ num | err number | BLER | total subframe |
| --- | --- | --- | --- | --- |
| -16 | 3 | 1 | 0.00001 | 100000 |
| -16 | 3 | 2 | 0.00001 | 200000 |
| -16 | 3 | 3 | 1.26176E-05 | 237764 |
| -16 | 3 | 4 | 1.44575E-05 | 276673 |
| -16 | 3 | 5 | 1.69591E-05 | 294827 |

Proposal 2: Prioritize test cases in TDLB100-400Hz fading channel for high reliability for URLLC in BS demodulation requirement

## 2.2	Requirements for low latency

Based on the WI, the following candidate features related to low latency should be further identified and prioritized.

        - PDSCH processing capability 2

        - Self-contained slot and/or non slot for DL

        - PDSCH and PUSCH mapping type A/B

        - Pre-emption indication for DL

        - Other features are not precluded

For latency requirements, it should be related with UE/BS processing timing, response time, timing adjustment, UL schedule (both SR and SPL).

In [2], the timing assumption with different SCS and symbol lengths is given by

| Timing | 15/30kHz SCS |  |  | 120kHz SCS |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| #TTI | 7os TTI | 4os TTI | 2os TTI | 7os TTI | 4os TTI | 2os TTI |
| gNB processing | 1 | 1 | 2 | 2 | 3 | 5 |
| UL timing | 1 | 1 | 2 | 2 | 3 | 5 |

The latency for DL and UL data can be illustrated as

Figure 1, latency for data transmission with DL and UL

Based on this procedure, the latency for FDD based on HARQ retransmission can be analyzed simply as [1]

Table xxx, latency for data transmission with HARQ-based retransmission

| Latency (ms) | HARQ | 15kHz SCS |  |  |  | 30kHz SCS |  |  |  | 120kHz SCS |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 14-os TTI | 7-os TTI | 4-os TTI | 2-os TTI | 14-os TTI | 7-os TTI | 4-os TTI | 2-os TTI | 14-os TTI | 7-os TTI | 4-os TTI | 2-os TTI |
| DL data | 1st transmission | 2.4 | 1.4 | 1.0 | 0.71 | 1.2 | 0.71 | 0.50 | 0.36 | 0.41 | 0.29 | 0.23 | 0.20 |
|  | 1 retx | 5.4 | 2.9 | 1.9 | 1.4 | 2.7 | 1.5 | 0.93 | 0.71 | 0.79 | 0.60 | 0.48 | 0.39 |
|  | 2 retx | 8.4 | 4.4 | 2.7 | 2.1 | 4.2 | 2.2 | 1.4 | 1.1 | 1.2 | 0.91 | 0.73 | 0.59 |
|  | 3 retx | 11 | 5.9 | 3.6 | 2.9 | 5.7 | 3.0 | 1.8 | 1.4 | 1.5 | 1.2 | 0.98 | 0.79 |
| UL data (SR) | 1st transmission | 4.5 | 2.5 | 1.6 | 1.4 | 2.3 | 1.3 | 0.82 | 0.68 | 0.67 | 0.54 | 0.46 | 0.38 |
|  | 1 retx | 8.4 | 4.4 | 2.7 | 2.1 | 4.2 | 2.2 | 1.4 | 1.1 | 1.2 | 0.91 | 0.73 | 0.59 |
|  | 2 retx | 11 | 5.9 | 3.6 | 2.9 | 5.7 | 3.0 | 1.8 | 1.4 | 1.5 | 1.2 | 0.98 | 0.79 |
|  | 3 retx | 14 | 7.4 | 4.4 | 3.6 | 7.2 | 3.7 | 2.2 | 1.8 | 1.9 | 1.5 | 1.2 | 0.98 |
| UL data (SPS) | 1st transmission | 2.4 | 1.4 | 1.0 | 0.71 | 1.2 | 0.71 | 0.50 | 0.36 | 0.41 | 0.29 | 0.23 | 0.20 |
|  | 1 retx | 5.4 | 2.9 | 1.9 | 1.4 | 2.7 | 1.5 | 0.93 | 0.71 | 0.79 | 0.60 | 0.48 | 0.39 |
|  | 2 retx | 8.4 | 4.4 | 2.7 | 2.1 | 4.2 | 2.2 | 1.4 | 1.1 | 1.2 | 0.91 | 0.73 | 0.59 |
|  | 3 retx | 11 | 5.9 | 3.6 | 2.9 | 5.7 | 3.0 | 1.8 | 1.4 | 1.5 | 1.2 | 0.98 | 0.79 |

As indicated in this table, slot based transmission with 120 KHz can meet 1ms target. With 15KHz SCS and higher, mini-slot based transmission can fulfill latency target. With 30KHz SCS and higher, mini-slot HARQ-based retransmission can fulfill latency target.

For Rel-15 NR, The performance requirement for both slot based and not slot based PUSCH with type A and type B have been defined. To verify the latency feature, mini slot based transmission can be regarded to one solution to meeting latency targeting, such as 2, 4 or 7 symbols.

It is agreed that no combined performance requirements will be defined by meeting both reliability of 99.999% and low latency requirements. That means some features with slot repetitions to improve high reliability should not be combined.

Proposal 3: The mini-slot based transmission could be considered to define requirements for meeting latency targeting.

# Conclusion

In this contribution, the initial results are provided to analyze the URLLC with high reliability requirement.

Proposal 1: loose number of errors should be considered for high reliability requirement of URLLC

Proposal 2: Prioritize test cases in TDLB100-400Hz fading channel for high reliability for URLLC in BS demodulation requirement

Proposal 3: The mini-slot based transmission could be considered to define requirements for meeting latency targeting.

# References

R4-1910044, WF on URLLC demodulation performance requirements, Huawei

R1-1806016, Evaluation of UP latency in NR	Ericsson