3GPP TSG-RAN WG4 Meeting #92Bis                                   R4-1911391

Chongqing, China, 14th – 18th Oct, 2019

Agenda item:			8.11.2.2

Source:			Samsung

Title:	Discussion on RRM Requirement for SCell BFR

Document for:		Discussion

# 1	Introduction

The Rel-16 MIMO enhancement work item was approved in RAN#80 including the objective of specifying BFR for SCell [1], i.e., .

<Captured in latest Rel-16 eMIMO WID> - Specify beam failure recovery for SCell with DL/UL as well as DL-only, where PCell can be operating in FR1 as well as FR2

Specifically, in Rel-15, beam failure recovery (BFR) is only supported on SpCell (i.e., PCell in SA, NR-DC, or NE-DC operation mode, and PSCell in NR-DC and EN-DC operation mode) but not on SCell. Provided that BFR mainly addresses FR2, which is often configured as SCell (e.g., it is a common deployment scenario where FR2 SCell(s) in SA provide high-throughput data connection), there was a strong desire to support BFR on SCell as well back in Rel-15. The issue was extensively discussed in RAN1/RAN2 but was terminated due to lack of time to complete this feature in Rel-15 scope, thus making the above-mentioned SCell BFR mechanism as one objective in Rel-16 MIMO enhancement work item.

In this discussion paper, we would like to share our view for how to define RRM requirement for SCell BFR.

# Discussion

As discussed in RAN1, several aspects to support beam failure recovery are included with RAN1 conclusion/agreement to be taken into account in specifying RAN4 RRM requirement.

#### 2.1 Scenarios for SCell BFR

As discussed in RAN1, the following two scenarios need to be considered for SCell BFR, i.e.,

-	Scenario 1: SCell with both uplink and downlink

-	Scenario 2: SCell with downlink only

-	PCell can be in FR1 or FR2 for scenarios above

It should be noted that for Scenario 2, i.e., SCell with downlink only, the Rel-15 BFR mechanism, i.e., RACH-based mechanism which requires UL configured in the SCell, can’t be applied since there is no UL corresponding to SCell DL. Based on that, new mechanism of beam failure recovery request and response with new beam identification is introduced in RAN1, which will be discussed as below.

Observation-1: Following two scenarios should be supported for Rel-16 SCell BFR:

-	Scenario 1: SCell with both uplink and downlink

-	Scenario 2: SCell with downlink only

Note: PCell can be in FR1 or FR2 for scenarios above.

#### 2.1 Beam Failure Detection on SCell

For beam failure detection on SCell, there is no big difference from existing BFD on PCell or PSCell, while the following related RAN1 agreement is achieved:

<RAN1 agreement> Agreement - SCell BFD is based on periodic 1-port CSI-RS, which can be configured explicitly by RRC or implicitly by TCI state. - Down-select one of the following alternatives in RAN1#96: - Alt 1: SCell BFD RS is in current CC - Alt 2: SCell BFD RS is in current CC for explicit configuration and can be in current CC or another CC for implicit configuration - Alt 3: SCell BFD RS can be in current CC or another CC for both explicit and implicit configuration - SCell BFD is measured based on hypothetical BLER Agreement When SCell BFD RS is configured in an implicit manner, BFD RS can be transmitted in active BWP of either current CC or another CC. Agreement Support that the BLER threshold for SCell BFD is the same as the default value of rlmInSyncOutOfSyncThreshold.

As captured in RAN1 agreement, beam failure detection can be based on 1-port periodic CSI-RS. The CSI-RS for BFD can be configured explicitly by RRC or implicitly by TCI state for a CORESET in SCell. Thus, we can have the following observation for SCell BFD’s impact on current TS38.133:

Proposal-1: At least the following impacts on TS38.133 are identified to be changed for introducing BFD on SCell:

- The scenario of SCell BFD needs to be added for the introduction part of link recovery in TS38.133 Section 8.5.1;

- It should be clarified that only CSI-RS based BFD is applied for SCell BFD.

#### 2.3 Candidate Beam Identification

It has been agreed that new beam identification can be performed based on SSB and CSI-RS for beam management. The SSB/CSI-RS can be transmitted within the SCell or another CC within the same band. The gNB can configure N SSB/CSI-RS resources per BWP for new beam identification. The detailed RAN1 agreement is captured as below:

<RAN1 agreement> Agreement Support the configuration of up to 64 candidate beams for BFR by RRC signalling, without introducing additional MAC CE signalling for down-selecting a subset of beams. - The total number of RSs for new beam identification and layer 1 RSRP measurement are part of UE capability signaling This applies per BWP. Agreement Downlink RS for new beam identification can be based on SSB and CSI-RS for BM Agreement Downlink RS for new beam identification can be transmitted in active BWP of the CC which is configured to be monitored for BFR or another CC within the same band Agreement New beam identification threshold is based on L1-RSRP Agreement - When SCell BFR is configured and RS for new beam identification is configured, the threshold for new beam identification should be always configured - If a SCell has failed, when there is no new beam with L1-RSRP higher than configured threshold for SCell BFR, for new beam information reporting, UE reports that there is no new beam identified for the SCell Agreement During a BFRQ procedure, UE reports only 1 beam with corresponding beam index only per SCell Agreement - For SCell BFR, support maximum number of RS for new beam identification per BWP to be 64. - The range of threshold for SCell new beam identification is based on range specified in RSRP-Range.

As analysed as above, for the procedure of candidate beam identification in UE itself, there is not big difference for Rel-16 candidate beam identification for SCell BFR, while the way of reporting candidate beam index can be different from Rel-15 CFRA-based mechanism.

Observation-2: For the procedure of candidate beam identification in UE itself, there is not big difference for Rel-16 candidate beam identification for SCell BFR, while the way of reporting candidate beam index can be different from Rel-15 CFRA-based mechanism.

#### 2.4 Beam Failure Recovery Request (BFRQ)

Based on RAN1 discussion, the mechanism of BFRQ is introduced for SCell BFR, i.e., it has been agreed that after UE declares beam failure, it shall transmit BFRQ. Specifically, the following RAN1 agreements related are achieved:

<RAN1 agreement> Agreement Specification support will be provided for gNB to derive at least the failed CC index during SCell BFR procedure Agreement For SCell BFR - Decide BFRQ solution for BFR on SCell with DL only first, PCell in FR1+FR2 - Above is to facilitate RAN1 discussion but not to prioritize certain scenarios Agreement - For SCell BFR, BFRQ shall be conveyed if UE declares beam failure - UE shall convey new beam information during BFR procedure if new candidate beam RS and corresponding threshold is configured and at least if channel quality of new beam is above or equal to threshold Agreement - For SCell with downlink only, UE reports failed CC index(es) and new beam information (if present) by PUSCH or PUCCH - FFS: whether it is carried by MAC CE or UCI-like PUSCH or PUCCH - Down-select at least one options for BFRQ procedure in RAN1 #97: - Option 1: Failed CC index(es), new beam information (if present) and beam failure event to be reported by a single report by MAC CE - FFS: whether or not to have spec impact on resource for MAC CE - Resource for MAC CE is not triggered by dedicated PUCCH/PRACH for BFR - Option 2: step 1: UE conveys beam failure event, and step 2: UE reports new beam information (if present) and failed CC index(es) - Step 1 is carried by dedicated PUCCH/PRACH resource - Step 2 is carried by MAC CE or UCI - Option 3: step 1: UE conveys beam failure event and failed CC index(es), and step 2: UE reports new beam information (if present) - Step 2 is carried by MAC CE or UCI, e.g. AP-CSI - PUCCH/PRACH is used for step 1 to carry failed CC index(es) implicitly - FFS: whether it is single-bit PUCCH or multi-bit PUCCH - The failed CC index(es) should be selected from up to N_max CCs for SCell BFR - FFS: N_max Agreement - When SCell BFR is configured and RS for new beam identification is configured, the threshold for new beam identification should be always configured - If a SCell has failed, when there is no new beam with L1-RSRP higher than configured threshold for SCell BFR, for new beam information reporting, UE reports that there is no new beam identified for the SCell Agreement During a BFRQ procedure, UE reports only 1 beam with corresponding beam index only per SCell Agreement On BFRQ procedure for SCell - Step 1 can be carried by at least a dedicated SR-like PUCCH resource for BFR over PCell or PSCell - FFS: Details including whether or not it is precluded that MAC CE in step 2 is multiplexed in a PUSCH not triggered by step 1 - (Working Assumption) Step 2 is carried by MAC CE Above applies at least for SCell with downlink only Agreement For SCell with both UL and DL, at least reuse the same BFRQ procedure as SCell with DL only. - Note: Whether to support CBRA/CFRA based BFRQ for both scenarios is a separate issue. - Note: At least from RAN1 perspective, there is no need for introducing restrictions on MAC CE transmission for BFR in Rel-16 - FFS: Whether PUCCH-BFR can be configured on SCells Agreement Support PUCCH-BFR to be configured by either one of PUCCH format 0 and PUCCH format 1 - FFS: details when PUCCH-BFR transmission is to be made in the same slot with other uplink signal(s). Conclusion - The details on MAC CE for BFR, and whether to transmit a MAC CE to carry BFRQ information for 1 SCell or multiple SCells is up to RAN2 - RAN1 identified that beam failure on multiple SCells can occur simultaneously but have not reached consensus on how often this occurs Agreement Send an LS to RAN2 suggesting higher priority for SCell BFR MAC CE than at least UL data, and to have a higher priority for SCell step 1 PUCCH than normal SR - To be included as part of LS to RAN2 The draft LS for RAN2 is modified as follows and endorsed in R1-1909833. Q1: Can the UE transmit BFR MAC CE using UL grant of any serving cell or should there be a restriction not to send it on failed serving cell(s)? R1: At least from RAN1 perspective, there is no need for introducing such restrictions on MAC CE transmission for BFR in Rel-16. Q2: If the UE already has the UL grant on serving cell(s) on which BFR MAC CE can be transmitted based on the answer to question 1, is the UE still required to transmit SR-like indication for BFR? R2: In this case, UE is not required to transmit SR-like indication for SCell BFR. Q3: Is there a case where the SR-like dedicated PUCCH resource for SCell BFR is not configured? If the SR-like dedicated PUCCH resource is not configured, one possible option being considered by RAN2 is that the UE follows the existing framework for requesting uplink resources when no uplink resources are available (i.e. performs CBRA on SpCell). R3: RAN1 did not discuss this case and will leave it to RAN2. RAN1 plans to conclude on this by RAN1#98bis. Q4: Is the SR-like dedicated PUCCH resource for SCell BFR configured for each SCell separately or is it common for all SCell(s) of the same cell group (i.e. MCG/SCG)? R4: The SR-like dedicated PUCCH resource for SCell BFR is not configured separately for each SCell. Q5: What conditions are used for the (successful) completion of the SCell BFR? R5: When UE receives beam failure recovery response (BFRR) to step 2, UE can consider BFR procedure is finished, where the BFRR to step 2 is a normal uplink grant to schedule a new transmission for the same HARQ process as PUSCH carrying the step 2 MAC CE, which is the same as normal “ACK” for PUSCH. Additional information RAN1 would like to provide the following additional information on SCell BFR to RAN2. - RAN1 suggests RAN2 to give higher priority for SCell BFR MAC CE than at least UL data, and also higher priority for SCell BFR PUCCH than normal SR - The details on MAC CE for BFR, and whether to transmit a MAC CE to carry BFRQ information for 1 SCell or multiple SCells is up to RAN2 - RAN1 identified that beam failure on multiple SCells can occur simultaneously but have not reached consensus on how often this occurs Agreement RAN1 will conclude on the following issue in RAN1#98bis Q3: Is there a case where the SR-like dedicated PUCCH resource for SCell BFR is not configured? If the SR-like dedicated PUCCH resource is not configured, one possible option being considered by RAN2 is that the UE follows the existing framework for requesting uplink resources when no uplink resources are available (i.e. performs CBRA on SpCell). Agreement Down-select one of the following alternatives on UE behavior when no new beam RS is configured in RAN1#98bis - Alt 1: UE shall expect gNB to configure at least one new beam RS if BFR for corresponding SCell is configured - Alt 3: If new beam RS is not configured, all SSBs are considered as new beam RS candidates

Based on the above RAN1 agreement, we can conclude that:

(1) For SCell with downlink only, the BFRQ can be conveyed by two steps:

Step 1 is carried by a dedicatedly configured SR-like PUCCH in PCell or PSCell to inform gNB beam failure happens

Step 2 is carried by a MAC CE to report detail information, e.g. failed CC index and new beam index

- If new candidate beam RS and corresponding threshold is configured

  - If at least one new beam is identified (L1-RSRP is higher or equal to the threshold)

    - UE reports 1 new beam index and failed CC index

  - Else

    - UE reports no new beam identified and failed CC index

- Else: No such case since RAN1 agreed that “When SCell BFR is configured and RS for new beam identification is configured, the threshold for new beam identification should be always configured”.

(2) For SCell with both uplink and downlink, at least the above BFRQ procedure as SCell with DL only can be used.

- MAC CE transmission for BFR in Rel-16 is possible on any SCell.

- RACH-based BFRQ is not yet precluded.

With the above analysis, the following observation can be achieved:

Observation-3: In Rel-16 eMIMO WI, RAN1 introduced new mechanism of beam failure recovery request (BFRQ), with two steps relying on SR-like PUCCH and MAC CE on SpCell:

Step 1 is carried by a dedicatedly configured SR-like PUCCH in PCell or PSCell to inform gNB beam failure happens

Step 2 is carried by a MAC CE to report detail information, e.g. failed CC index and new beam index

  - If at least one new beam is identified (L1-RSRP is higher or equal to the threshold)

    - UE reports 1 new beam index and failed CC index

  - Else

    - UE reports no new beam identified and failed CC index

Observation-4: In SCell BFRQ, the procedure of informing gNB beam failure happens and reporting detailed information (failed CC and new beam index (or no beam identified)), has no big impact on existing core requirement of BFD and CBD.

With the above into account, we can reach the following proposal for how to deal with SCell BFR requirement in TS38.133:

Proposal-2: For SCell BFRQ mechanism introduced in Rel-16, RAN4 still define RRM requirement by following Rel-15 specification architecture for BFD and CBD. In other words, the new procedure of informing gNB beam failure happens and reporting detailed information will not have core requirement impact on TS38.133.

#### 2.5 UE Capability of Number of SCells for BFR

It has been agreed that gNB can configure UE to perform BFR for any configured SCells, and the maximum number of SCells for BFR is one UE capability, as captured in following RAN1 agreement.

<RAN1 agreement> Agreement A UE can be configured to perform BFR for any configured SCells - The maximum number of SCells for which the UE performs BFR is a UE capability

Observation-5: The maximum number of SCell for which UE performs BFR is a UE capability.

# Conclusion

In this paper, we provided our view and analysis on SCell BFR which is introduced in Rel-16 MIMO enhancement work item, with the following observations and proposals obtained:

Observation-1: Following two scenarios should be supported for Rel-16 SCell BFR:

-	Scenario 1: SCell with both uplink and downlink

-	Scenario 2: SCell with downlink only

Note: PCell can be in FR1 or FR2 for scenarios above.

Proposal-1: At least the following impacts on TS38.133 are identified to be changed for introducing BFD on SCell:

- The scenario of SCell BFD needs to be added for the introduction part of link recovery in TS38.133 Section 8.5.1;

- It should be clarified that only CSI-RS based BFD is applied for SCell BFD.

Observation-2: For the procedure of candidate beam identification in UE itself, there is not big difference for Rel-16 candidate beam identification for SCell BFR, while the way of reporting candidate beam index can be different from Rel-15 CFRA-based mechanism.

Observation-3: In Rel-16 eMIMO WI, RAN1 introduced new mechanism of beam failure recovery request (BFRQ), with two steps relying on SR-like PUCCH and MAC CE on SpCell:

Step 1 is carried by a dedicatedly configured SR-like PUCCH in PCell or PSCell to inform gNB beam failure happens

Step 2 is carried by a MAC CE to report detail information, e.g. failed CC index and new beam index

  - If at least one new beam is identified (L1-RSRP is higher or equal to the threshold)

    - UE reports 1 new beam index and failed CC index

  - Else

    - UE reports no new beam identified and failed CC index

Observation-4: In SCell BFRQ, the procedure of informing gNB beam failure happens and reporting detailed information (failed CC and new beam index (or no beam identified)), has no big impact on existing core requirement of BFD and CBD.

Proposal-2: For SCell BFRQ mechanism introduced in Rel-16, RAN4 still define RRM requirement by following Rel-15 specification architecture for BFD and CBD. In other words, the new procedure of informing gNB beam failure happens and reporting detailed information will not have core requirement impact on TS38.133.

Observation-5: The maximum number of SCell for which UE performs BFR is a UE capability.

# Reference

[1] RP-192271,	“Revised WID: Enhancements on MIMO for NR”, Samsung.

[2] RP-192270,	“Status Report for WI: Enhancements on MIMO for NR”, Samsung

# Appendix

#### 5.1 RAN1 Agreement for BFR on SCell

Till this RAN4 meeting, RAN1 has concluded the following agreement for BFR mechanism on SCell as below:

<RAN1 agreement> Agreement made in RAN1#95 RAN1 has identified the following scenarios to be important for SCell BFR - Scenario 1: SCell with both uplink and downlink - Scenario 2: SCell with downlink only - PCell can be in FR1 or FR2 for scenarios above Agreement made in RAN1 NR-AH1901 Specification support will be provided for gNB to derive at least the failed CC index during SCell BFR procedure - FFS: Whether the information is implicitly derived or explicitly conveyed by the UE - FFS: Whether new beam information should be included - FFS: Details on triggering for transmitting BFRQ Agreement - SCell BFD is based on periodic 1-port CSI-RS, which can be configured explicitly by RRC or implicitly by TCI state. - Down-select one of the following alternatives in RAN1#96: - Alt 1: SCell BFD RS is in current CC - Alt 2: SCell BFD RS is in current CC for explicit configuration and can be in current CC or another CC for implicit configuration - Alt 3: SCell BFD RS can be in current CC or another CC for both explicit and implicit configuration - SCell BFD is measured based on hypothetical BLER Agreement Down-select at least one of the following alternatives: - Alt 1: For SCell BFR, BFRQ can be transmitted if UE declares beam failure and identifies a new candidate beam. - UE reports new beam information by or after BFRQ - Alt 2: For SCell BFR, BFRQ can be transmitted if UE declares beam failure. - UE only indicates beam failure happens by BFRQ - Note: new beam identification can be done by using DL BM procedure - Alt 3: For SCell BFR, BFRQ can be transmitted if UE declares beam failure - UE may report new beam information during BFR procedure - FFS: impact of new beam identification threshold - Note: It is up to UE whether to do beam failure detection and new beam identification in parallel or not - For Alt1 and Alt3, reference signals for new candidate DL beam(s) are configured, which are based on CSI-RS and/or SSB. - FFS: whether the CSI-RS and/or SSB can be in another CC - FFS: signaling details, e.g. RRC and/or MAC CE Agreement For SCell BFR - Decide BFRQ solution for BFR on SCell with DL only first, PCell in FR1+FR2 - Above is to facilitate RAN1 discussion but not to prioritize certain scenarios Agreement made in RAN1#96 - For SCell BFR, BFRQ shall be conveyed if UE declares beam failure - UE shall convey new beam information during BFR procedure if new candidate beam RS and corresponding threshold is configured and at least if channel quality of new beam is above or equal to threshold - FFS: whether no new beam identified could be included as a state of new beam information - FFS: details if no new beam is above or equal to threshold Agreement made in RAN1#96Bis Support the configuration of up to 64 candidate beams for BFR by RRC signalling, without introducing additional MAC CE signalling for down-selecting a subset of beams. - The total number of RSs for new beam identification and layer 1 RSRP measurement are part of UE capability signaling This applies per BWP. Agreement Downlink RS for new beam identification can be based on SSB and CSI-RS for BM Agreement Downlink RS for new beam identification can be transmitted in active BWP of the CC which is configured to be monitored for BFR or another CC within the same band Agreement New beam identification threshold is based on L1-RSRP Agreement At least for explicit configuration, downlink RS for BFD is in current CC - FFS: Downlink RS for BFD in another CC within the same band for implicit configuration Agreement - For SCell with downlink only, UE reports failed CC index(es) and new beam information (if present) by PUSCH or PUCCH - FFS: whether it is carried by MAC CE or UCI-like PUSCH or PUCCH - Down-select at least one options for BFRQ procedure in RAN1 #97: - Option 1: Failed CC index(es), new beam information (if present) and beam failure event to be reported by a single report by MAC CE - FFS: whether or not to have spec impact on resource for MAC CE - Resource for MAC CE is not triggered by dedicated PUCCH/PRACH for BFR - Option 2: step 1: UE conveys beam failure event, and step 2: UE reports new beam information (if present) and failed CC index(es) - Step 1 is carried by dedicated PUCCH/PRACH resource - Step 2 is carried by MAC CE or UCI - Option 3: step 1: UE conveys beam failure event and failed CC index(es), and step 2: UE reports new beam information (if present) - Step 2 is carried by MAC CE or UCI, e.g. AP-CSI - PUCCH/PRACH is used for step 1 to carry failed CC index(es) implicitly - FFS: whether it is single-bit PUCCH or multi-bit PUCCH - The failed CC index(es) should be selected from up to N_max CCs for SCell BFR - FFS: N_max Agreement made in RAN1#97 When SCell BFD RS is configured in an implicit manner, BFD RS can be transmitted in active BWP of either current CC or another CC. Agreement A UE can be configured to perform BFR for any configured SCells - The maximum number of SCells for which the UE performs BFR is a UE capability Agreement - When SCell BFR is configured and RS for new beam identification is configured, the threshold for new beam identification should be always configured - If a SCell has failed, when there is no new beam with L1-RSRP higher than configured threshold for SCell BFR, for new beam information reporting, UE reports that there is no new beam identified for the SCell Agreement During a BFRQ procedure, UE reports only 1 beam with corresponding beam index only per SCell Agreement On BFRQ procedure for SCell - Step 1 can be carried by at least a dedicated SR-like PUCCH resource for BFR over PCell or PSCell - FFS: Details including whether or not it is precluded that MAC CE in step 2 is multiplexed in a PUSCH not triggered by step 1 - (Working Assumption) Step 2 is carried by MAC CE Above applies at least for SCell with downlink only Agreement made in RAN1#98 Support that the BLER threshold for SCell BFD is the same as the default value of rlmInSyncOutOfSyncThreshold. Agreement - For SCell BFR, support maximum number of RS for new beam identification per BWP to be 64. - The range of threshold for SCell new beam identification is based on range specified in RSRP-Range. Agreement For SCell with both UL and DL, at least reuse the same BFRQ procedure as SCell with DL only. - Note: Whether to support CBRA/CFRA based BFRQ for both scenarios is a separate issue. - Note: At least from RAN1 perspective, there is no need for introducing restrictions on MAC CE transmission for BFR in Rel-16 - FFS: Whether PUCCH-BFR can be configured on SCells Agreement Support PUCCH-BFR to be configured by either one of PUCCH format 0 and PUCCH format 1 - FFS: details when PUCCH-BFR transmission is to be made in the same slot with other uplink signal(s). Conclusion - The details on MAC CE for BFR, and whether to transmit a MAC CE to carry BFRQ information for 1 SCell or multiple SCells is up to RAN2 - RAN1 identified that beam failure on multiple SCells can occur simultaneously but have not reached consensus on how often this occurs Agreement Include the above conclusion as part of LS to RAN2. Agreement For latency/overhead reduction across multiple CCs/BWPs, support single MAC-CE to activate at least the same set of PDSCH TCI state IDs for multiple CCs/BWPs - Example 1: Reuse Rel-15 MAC-CE to activate same set of TCI state IDs for all active BWPs in same band or cell group(s) on FR2 - Support of this mode can be indicated by UE capability - To operate in this mode, UE may expect the same QCL-TypeD RS is configured for same TCI state ID for all BWPs in each band or cell group(s) - For activation MAC-CE received on any active BWP in a band or cell group(s), indicated activated TCI state IDs will be applied to every active BWP in that band or cell group(s) - Example 2: Reuse Rel-15 MAC-CE to activate one set of TCI state IDs (including both QCL Type-A and Type-D RSs) for an active BWP of the CC indicated by the MAC-CE to be applied to all active BWPs in same band or cell group(s) on FR2 - Note: The QCL Type A RS(s) applied to each CC/BWP is that corresponding to the same resource ID(s) indicated by the TCI state IDs - FFS: operation/signaling details including the possibility to activate different sets of PDSCH TCI state IDs for multiple CCs/BWPs - Note: QCL type-A comes from the BWP where the TCI state is applied From RAN1 perspective, the response after UE transmits PUCCH-BFR can be a normal uplink grant with C-RNTI/MCS-C-RNTI - No RAN1 spec impact Agreement Send an LS to RAN2 suggesting higher priority for SCell BFR MAC CE than at least UL data, and to have a higher priority for SCell step 1 PUCCH than normal SR - To be included as part of LS to RAN2 The draft LS for RAN2 is modified as follows and endorsed in R1-1909833. Q1: Can the UE transmit BFR MAC CE using UL grant of any serving cell or should there be a restriction not to send it on failed serving cell(s)? R1: At least from RAN1 perspective, there is no need for introducing such restrictions on MAC CE transmission for BFR in Rel-16. Q2: If the UE already has the UL grant on serving cell(s) on which BFR MAC CE can be transmitted based on the answer to question 1, is the UE still required to transmit SR-like indication for BFR? R2: In this case, UE is not required to transmit SR-like indication for SCell BFR. Q3: Is there a case where the SR-like dedicated PUCCH resource for SCell BFR is not configured? If the SR-like dedicated PUCCH resource is not configured, one possible option being considered by RAN2 is that the UE follows the existing framework for requesting uplink resources when no uplink resources are available (i.e. performs CBRA on SpCell). R3: RAN1 did not discuss this case and will leave it to RAN2. RAN1 plans to conclude on this by RAN1#98bis. Q4: Is the SR-like dedicated PUCCH resource for SCell BFR configured for each SCell separately or is it common for all SCell(s) of the same cell group (i.e. MCG/SCG)? R4: The SR-like dedicated PUCCH resource for SCell BFR is not configured separately for each SCell. Q5: What conditions are used for the (successful) completion of the SCell BFR? R5: When UE receives beam failure recovery response (BFRR) to step 2, UE can consider BFR procedure is finished, where the BFRR to step 2 is a normal uplink grant to schedule a new transmission for the same HARQ process as PUSCH carrying the step 2 MAC CE, which is the same as normal “ACK” for PUSCH. Additional information RAN1 would like to provide the following additional information on SCell BFR to RAN2. - RAN1 suggests RAN2 to give higher priority for SCell BFR MAC CE than at least UL data, and also higher priority for SCell BFR PUCCH than normal SR - The details on MAC CE for BFR, and whether to transmit a MAC CE to carry BFRQ information for 1 SCell or multiple SCells is up to RAN2 - RAN1 identified that beam failure on multiple SCells can occur simultaneously but have not reached consensus on how often this occurs Agreement RAN1 will conclude on the following issue in RAN1#98bis Q3: Is there a case where the SR-like dedicated PUCCH resource for SCell BFR is not configured? If the SR-like dedicated PUCCH resource is not configured, one possible option being considered by RAN2 is that the UE follows the existing framework for requesting uplink resources when no uplink resources are available (i.e. performs CBRA on SpCell). Agreement The BFRR to step 2 is a normal uplink grant to schedule a new transmission for the same HARQ process as PUSCH carrying the step 2 MAC CE - The procedure is the same as normal “ACK” for PUSCH - When UE receives BFRR to step 2, UE can consider BFR procedure is finished - No RAN1 spec impact - Included as part of LS to RAN2 Agreement Down-select one of the following alternatives on UE behavior when no new beam RS is configured in RAN1#98bis - Alt 1: UE shall expect gNB to configure at least one new beam RS if BFR for corresponding SCell is configured - Alt 3: If new beam RS is not configured, all SSBs are considered as new beam RS candidates Agreement Continue discussion on the support of updating pathloss RSs for power control for PUSCH and SRS via MAC-CE, including the following candidates until RAN1#98bis: - Option 1: For codebook based PUSCH transmission, the pathloss RS follows DL RS in spatial relation associated with SRI indicated in scheduling DCI, if the pathloss RS is not configured and periodic DL RS is configured in the spatial relation. - FFS: the cases of non-codebook based PUSCH, SRS - Option 2: Pathloss RS is associated/configured for downlink RS in spatial relation info. - gNB can configure more than 4 pathloss RSs. - Option 3: At least the pathloss RSs for SRS or PUSCH can be explicitly activated/updated by the MAC-CE - FFS: The other power control parameters including P0, alpha, and a closed loop process index are also activated by the MAC-CE - FFS on whether to support the number of configured pathloss RSs are more than four. - Note: The MAC-CE is the activation MAC CE for ap-SRS/sp-SRS. - Option 4: Support updating TCI state for periodic CSI-RS by MAC CE. - Note: The periodic CSI-RS is used for pathloss reference. - Option 5: Support semi-persistent CSI-RS for pathloss reference RS. - Note: Baseline is that the same transmission power is applied within SRS resource set (same as Rel-15). Agreement At least for UEs supporting beam correspondence, if spatial relation info for dedicated-PUCCH/SRS, except for SRS with usage = 'BeamManagement', is not configured in FR2, the applied default spatial relation for the dedicated-PUCCH/SRS is down-selected from the followings in RAN1#98bis - Alt.1: default TCI state or QCL assumption of PDSCH (e.g. the most recent slot and the lowest CORESET ID) - Alt.2: one of an active TCI state of CORESET - FFS: details of which TCI state - Alt.3: TCI state of scheduling PDCCH for A-SRS/PUCCH, and default TCI state or QCL assumption of PDSCH for other than A-SRS/PUCCH - Alt.4: CORESET#0 QCL assumption - Alt.5: pathloss reference RS - FFS: details of which pathloss reference RS - FFS: whether to apply the above for UEs not supporting beam correspondence Conclusion On the issue of beam management enhancements to address MPE, there is no consensus in RAN1 to introduce additional specification support in RAN1. No further discussion for Rel-16 from RAN1 perspective. Conclusion On the issue of uplink transmit beam selection that facilitates UE panel specific uplink beam selection, there is no consensus in RAN1 to introduce additional specification support in RAN1. No further discussion for Rel-16 from RAN1 perspective.