3GPP TSG-RAN WG4 Meeting #92bis	R4-1911422

# Chongqing, China, 14 – 18 October, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | DRAFT | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | [draft] CR on introducing UE demodulation tests for single tap HST with 500km/h |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_high_speed_enh2-Perf |  |  |  |  |  | Date: |  |  | 2019-10-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Add the agreed UE demodulation tests for single tap HST with 500km/h. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the UE demodulation tests for single tap HST with 500km/h and modify the corresponding HST conditions. Modofy the definition of high speed train scenarios |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE demodulation performance for single tap HST with 500km/h cannot be verified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.2.6.1, 8.2.1.9.2, 8.2.2.9.2, 8.10.1.1.14, 8.10.1.2.14, B.3, B.3A, B.3C(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 36.521-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------------------------------------- Beginning of Change ------------------------------------------------------------

##### 8.1.2.6.1	Applicability rule and antenna connection for single carrier tests with 2Rx

------------------------------------------------- Unchanged sections omitted --------------------------------------------------------

Table 8.1.2.6.1-1: Applicability rules for single carrier tests with 2Rx

| 4Rx test lists | 2Rx test lists |
| --- | --- |
| 8.10.1.1.1 Test 1 | 8.2.1.2.1 Test 1 |
| 8.10.1.1.2 Test 1 | 8.2.1.3.1 Test 1 |
| 8.10.1.1.3 Test 1 | 8.2.1.4.1B Test 1 |
| 8.10.1.1.4 Test 1 | 8.2.1.4.2 Test 1 |
| 8.10.1.1.4 Test 2 | 8.2.1.4.2 Test 3 |
| 8.10.1.1.5 Test 1 | 8.3.1.1A Test 1 |
| 8.10.1.1.5A Test 1 | 8.3.1.1 Test 2 |
| 8.10.1.1.5B Test 1 | 8.3.1.1H Test 1 |
| 8.10.1.1.6 Test 1 | 8.3.1.2 Test 1 |
| 8.10.1.1.14 Test 1 | 8.2.1.9.2 Test 1 |
| 8.10.1.1.14 Test 2 | 8.2.1.9.2 Test 2 |
| 8.10.1.2.1 Test 1 | 8.2.2.2.1 Test 1 |
| 8.10.1.2.2 Test 1 | 8.2.2.3.1 Test 1 |
| 8.10.1.2.3 Test 1 | 8.2.2.4.1B Test 1 |
| 8.10.1.2.4 Test 1 | 8.2.2.4.2 Test 1 |
| 8.10.1.2.4 Test 2 | 8.2.2.4.2 Test 3 |
| 8.10.1.2.5 Test 1 | 8.3.2.1B Test 1 |
| 8.10.1.2.5A Test 1 | 8.3.2.1A Test 2 |
| 8.10.1.2.5B Test 1 | 8.3.1.1I Test 1 |
| 8.10.1.2.6 Test 1 | 8.3.2.2 Test 2 |
| 8.10.1.2.14 Test 1 | 8.2.2.9.2 Test 1 |
| 8.10.1.2.14 Test 2 | 8.2.2.9.2 Test 2 |
| 8.10.2.1.1 Test 1 | 8.4.1.1 Test 1 |
| 8.10.2.1.2 Test 1 | 8.4.1.2.1 Test 1 |
| 8.10.2.1.3 Test 1 | 8.4.1.2.2 Test 1 |
| 8.10.2.2.1 Test 1 | 8.4.2.1 Test 1 |
| 8.10.2.2.2 Test 1 | 8.4.2.2.1 Test 1 |
| 8.10.2.2.3 Test 1 | 8.4.2.2.2 Test 1 |
| 8.10.3.1.1 Test 1 | 8.5.1.1 Test 1 |
| 8.10.3.1.2 Test 1 | 8.5.1.2.1 Test 1 |
| 8.10.3.1.3 Test 1 | 8.5.1.2.2 Test 1 |
| 8.10.3.2.1 Test 1 | 8.5.2.1 Test 1 |
| 8.10.3.2.2 Test 1 | 8.5.2.2.1 Test 1 |
| 8.10.3.2.3 Test 1 | 8.5.2.2.2 Test 1 |
| 8.10.4.1.1 Test 1 | 8.8.1.1 Test 1 |
| 8.10.4.1.1 Test 2 | 8.8.1.1 Test 2 |
| 8.10.4.1.2 Test 1 | 8.8.1.2 Test 1 |
| 8.10.4.1.2 Test 2 | 8.8.1.2 Test 2 |
| 8.10.4.2.1 Test 1 | 8.8.2.1 Test 1 |
| 8.10.4.2.1 Test 2 | 8.8.2.1 Test 2 |
| 8.10.4.2.2 Test 1 | 8.8.2.2 Test 1 |
| 8.10.4.2.2 Test 2 | 8.8.2.2 Test 2 |

------------------------------------------------- Unchanged sections omitted --------------------------------------------------------

8.2.1.9.2	Minimum Requirement for Rel-16 further enhanced HST

The requirements are specified in Table 8.2.1.9.2-2, with the addition of the parameters in Table 8.2.1.9.2-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests are to verify UE performance in the HST-SFN-500 and HST-500 scenario. The test for HST-SFN-500 scenario defined in B.3B is applied when highSpeedEnhancedDemodulationFlag-Rel16 [7] is received. The test for HST-500 scenario defined in B.3C is applied when highSpeedEnhancedDemodulationFlag-Rel16 [7] is not received.

Table 8.2.1.9.2-1: Test Parameters for UE performance in HST-SFN-500 and HST-500 scenario (FRC)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 (NOTE 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| PDSCH transmission mode |  |  | 3 |
| NOTE 1: . |  |  |  |

Table 8.2.1.9.2-2: Minimum performance UE in HST-SFN-500 and HST-500 scenario (FRC)

| Test number | Band-width | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 NOTE 2 | 10 MHz | R.87-1 FDD | OP.1 FDD | HST-SFN-500 | 2x2 | 70 | TBD | ≥1 |
| 2 | 10 MHz | R.87 FDD | OP.1 FDD | HST-500 | 2x2 | 70 | TBD | ≥1 |
| NOTE 1: Test case applicability is defined in 8.1.2.6. NOTE 2: The requirement defined is based on the normarliazed channel model, i.e.the power of each tap is normalized to the instantaneous total received power from four taps. |  |  |  |  |  |  |  |  |

------------------------------------------------- Unchanged sections omitted --------------------------------------------------------

8.2.2.9.2	Minimum Requirement for Rel-16 further enhanced HST

The requirements are specified in Table 8.2.2.9.2-2, with the addition of the parameters in Table 8.2.2.9.2-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests are to verify UE performance in the HST-SFN-500 and HST-500 scenario. The test for HST-SFN-500 scenario defined in B.3B is applied when highSpeedEnhancedDemodulationFlag-Rel16 [7] is received. The test for HST-500 scenario defined in B.3C is applied when highSpeedEnhancedDemodulationFlag-Rel16 [7] is not received.

Table 8.2.2.9.2-1: Test Parameters for UE performance in HST-SFN-500 and HST-500 scenario (FRC)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 |  |
|  |  | dB | -3 (NOTE 1) |  |
|  |  | dB | 0 |  |
| at antenna port |  | dBm/15kHz | -98 |  |
| ACK/NACK feedback mode |  |  | Multiplexing |  |
| PDSCH transmission mode |  |  | 3 |  |
| NOTE 1: . |  |  |  |  |

Table 8.2.2.9.2-2: Minimum performance UE in HST-SFN scenario (FRC)

| Test number | Band-width | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 NOTE 2 | 10 MHz | R.87-1 TDD | OP.1 TDD | HST-SFN-500 | 2x2 | 70 | TBD | ≥1 |
| 2 | 10 MHz | R.87 TDD | OP.1 TDD | HST-500 | 2x2 | 70 | TBD | ≥1 |
| NOTE 1: Test case applicability is defined in 8.1.2.1 and 8.1.2.6. NOTE 2: The requirement defined is based on the normarliazed channel model, i.e.the power of each tap is normalized to the instantaneous total received power from four taps. |  |  |  |  |  |  |  |  |

------------------------------------------------- Unchanged sections omitted --------------------------------------------------------

8.10.1.1.14	HST-SFN performance

8.10.1.1.14.1	Minimum Requirement for Rel-16 further enhanced HST

The requirements are specified in Table 8.10.1.1.14.1-2, with the addition of the parameters in Table 8.10.1.1.14.1-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests are to verify UE performance in the HST-SFN-500 and HST-500 scenario. The test for HST-SFN-500 scenario defined in B.3B is applied when highSpeedEnhancedDemodulationFlag-Rel16 [7] is received. The test for HST-500 scenario defined in B.3C is applied when highSpeedEnhancedDemodulationFlag-Rel16 [7] is not received.

Table 8.10.1.1.14.1-1: Test Parameters for UE performance in HST-SFN-500 and HST-500 scenario (FRC)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 (NOTE 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| PDSCH transmission mode |  |  | 3 |
| NOTE 1: . |  |  |  |

Table 8.10.1.1.14.1-2: Minimum performance UE in HST-SFN-500 and HST-500 scenario (FRC)

| Test number | Band-width | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 NOTE 2 | 10 MHz | R.87-1 FDD | OP.1 FDD | HST-SFN-500 | 2x4 | 70 | TBD | ≥1 |
| 2 | 10 MHz | R.87 FDD | OP.1 FDD | HST-500 | 2x4 | 70 | TBD | ≥1 |
| NOTE 1: Test case applicability is defined in 8.1.2.6. NOTE 2: The requirement defined is based on the normarliazed channel model, i.e.the power of each tap is normalized to the instantaneous total received power from four taps. |  |  |  |  |  |  |  |  |

------------------------------------------------- Unchanged sections omitted --------------------------------------------------------

8.10.1.2.14	HST-SFN performance

8.10.1.2.14	Minimum Requirement for Rel-16 further enhanced HST

The requirements are specified in Table 8.10.1.2.14-2, with the addition of the parameters in Table 8.10.1.2.14-1 and the downlink physical channel setup according to Annex C.3.2. The purpose of these tests are to verify UE performance in the HST-SFN-500 and HST-500 scenario. The test for HST-SFN-500 scenario defined in B.3B is applied when highSpeedEnhancedDemodulationFlag-Rel16 [7] is received. The test for HST-500 scenario defined in B.3C is applied when highSpeedEnhancedDemodulationFlag-Rel16 [7] is not received.

Table 8.10.1.2.14-1: Test Parameters for UE performance in HST-SFN-500 and HST-500 scenario (FRC)

| Parameter |  | Unit | Test 1 |
| --- | --- | --- | --- |
| Downlink power allocation |  | dB | -3 |
|  |  | dB | -3 (NOTE 1) |
|  |  | dB | 0 |
| at antenna port |  | dBm/15kHz | -98 |
| ACK/NACK feedback mode |  |  | Multiplexing |
| PDSCH transmission mode |  |  | 3 |
| NOTE 1: . |  |  |  |

Table 8.10.1.2.14-2: Minimum performance UE in HST-SFN-500 and HST-500 scenario (FRC)

| Test number | Band-width | Reference Channel | OCNG Pattern | Propagation Condition | Correlation Matrix and Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1NOTE 2 | 10 MHz | R.87-1 TDD | OP.1 TDD | HST-SFN-500 | 2x4 | 70 | TBD | ≥1 |
| 2 | 10 MHz | R.87 TDD | OP.1 TDD | HST-500 | 2x4 | 70 | TBD | ≥1 |
| NOTE 1: Test case applicability is defined in 8.1.2.1 and 8.1.2.6. NOTE 2: The requirement defined is based on the normarliazed channel model, i.e.the power of each tap is normalized to the instantaneous total received power from four taps. |  |  |  |  |  |  |  |  |

------------------------------------------------- Unchanged sections omitted --------------------------------------------------------

B.3	High speed train scenario

The high speed train condition for the test of the baseband performance is a non fading propagation channel with one tap. Doppler shift is given by

(B.3.1)

where  is the Doppler shift and  is the maximum Doppler frequency. The cosine of angle is given by

, 	(B.3.2)

, 	(B.3.3)

, 	(B.3.4)

where  is the initial distance of the train from eNodeB, and  is eNodeB Railway track distance, both in meters;  is the velocity of the train in m/s,  is time in seconds.

Doppler shift and cosine angle are given by equation B.3.1 and B.3.2-B.3.4 respectively, where the required input parameters listed in table B.3-1 and the resulting Doppler shift shown in Figure B.3-1 are applied for all frequency bands.

Table B.3-1: High speed train scenario

| Parameter | Value |
| --- | --- |
|  | 300 m |
|  | 2 m |
|  | 300 km/h |
|  | 750 Hz |

NOTE 1:	Parameters for HST conditions in table B.3-1 including  and Doppler shift trajectories presented on figure B.3-1 were derived from Band 7 and are applied for performance verification in all frequency bands.

Figure B.3-1: Doppler shift trajectory

For 1x2 antenna configuration, the same h(t,τ) is used to describe the channel between every pair of Tx and Rx.

For 2x2 antenna configuration, the same h(t,τ) is used to describe the channel between every pair of Tx and Rx with phase shift according to .

For 2x4 antenna configuration, the same h(t,τ) is used to describe the channel between every pair of Tx and Rx with phase shift according to .

------------------------------------------------- Unchanged sections omitted --------------------------------------------------------

B.3A	HST-SFN scenario

There is an infinite number of RRHs distributed equidistantly along the track with the same Cell ID as depicted in figure B.3A-1.

Figure B.3A-1: Deployment of HST-SFN

The location of RRH k is given as:

(B.3A.1)

where:	,  and is the distance between the RRHs and railway track, while  is the distance of two RRHs, both in meters.

The train location is denoted as:

(B.3A.2)

where:	 and a means distance in meters, which means the train is right on the track.

The HST-SFN scenario for the test of the baseband performance is a non fading propagation channel with four taps, namely the four nearest RRHs. Thus RRH k is visible for the train only in the range:

(B.3A.3)

Power level  (dB) for the signal from kth RRH, normalized to the total power received from all visible RRHs, is given by:

for 	(B.3A.4)

Doppler shift (Hz) from kth RRH is given by:

for 	(B.3A.5)

The relative delay  (s) for the signal from kth RRH can be derived as:

for 	(B.3A.6)

In the above v (m/s) is the moving speed of the train, fC (Hz) is the center frequency, and C (m/s) is the velocity of light.

Power level, Doppler shift and relative delay are given by equations B.3A.4 ~ B.3A.6 respectively, where the required input parameters listed in table B.3A-1 and the resulting Doppler shift shown in Figure B.3A-3 are applied for all frequency bands.

Table B.3A-1: HST-SFN scenario

| Parameter | Value |
| --- | --- |
|  | 1000 m |
|  | 50m |
|  | 350 km/h |
|  | 872 Hz |

NOTE 1:	Parameters for HST-SFN scenario in Table B.3A-1 includingand Doppler shift trajectories presented in Figure B.3A-2 were derived from Band 7 and are applied for performance verification in all frequency bands. And the trajectories of ralative power, Doppler shifts and relative delay presented in Figures B.3A-2~ B.3A-4 are derived from the equations B.3A.4 ~ B.3A.6 respectively.

Figure B.3A-2 Ralative power level trajectories

Figure B.3A-3 Doppler shifts trajectories

Figure B.3A-4 Relative delay trajectories

For 2x2 antenna configuration, the same h(t,τ) is used to describe the channel between every pair of Tx and Rx with phase shift according to .

For 2x4 antenna configuration, the same h(t,τ) is used to describe the channel between every pair of Tx and Rx with phase shift according to .

------------------------------------------------- Unchanged sections omitted --------------------------------------------------------

B.3B	HST-SFN scenario for 500km/h speed

The channel model for this scenario is the same as B.3A, with the following parameters replacing Table B.3A-1:

Table B.3B-1: HST-SFN-500 scenario for higher speed

| Parameter | Value |
| --- | --- |
|  | 1000 m |
|  | 50 m |
|  | 500 km/h |
|  | 972 Hz |

B.3C	HST scenario for 500km/h speed

The channel model for this scenario is the same as B.3, with the following parameters replacing Table B.3-1:

Table B.3C-1: HST-500 scenario for higher speed

| Parameter | Value |
| --- | --- |
|  | 300 m |
|  | 2 m |
|  | 500 km/h |
|  | 972 Hz |

------------------------------------------------------------- End of change ------------------------------------------------------------