3GPP TSG RAN WG4 #92BIS Meeting                  	R4-1911448

Chongqing, China, 14th – 18th October 2019

Agenda Item:	9.8.2

Source:	LG Electronics

Title:	TP on MSD results for EN-DC LTE x bands DL and 1UL (x=1,2,3,4) + NR 2 bands DL and 1UL in REl-16

Document for:	Approval

- Introduction

In the last RAN #84 and #85 meeting, new EN-DC basket WI are approved for LTE x bands DL and 1UL (x=1,2,3,4) + NR 2 bands DL and 1UL DC band combinations [1].

Base on summary paper [2], we merge the DC band combination in Table 1 which need to analyse the additional self-interference problem

Table1. Remaining DC band combination lists by self-interference analysis [2]

| EN-DC Downlink band configuration | Uplink EN-DC Configuration | Harmonic relation issues | intermodulation to own rx band (3rd band) | interference due to small frequency separation | MSD |
| --- | --- | --- | --- | --- | --- |
| DC_39_n41-n79 | DC_39A_n41A | 2nd harmonic from n41 into n79 | 2nd I& 5th MDs | - | - Over 4992MHz frequency in n79 was impacted by 2nd harmonic, but not used - FFS |
|  | DC_39A_n79A | - | 2nd & 5th IMDs | - | - FFS |
| DC_40_n41-n79 | DC_40A_n41A | 2nd harmonic from n40 into n79 2nd harmonic from n41 into n79 | 2nd & 4th IMDs | - | - FFS by 2nd harmonic from B40 - Over 4992MHz frequency in n79 was impacted by 2nd harmonic, but not used - FFS - No impact to china band since over 4800MHz is considered in NR |
| DC_12_n7-n78 | DC_12A_n7A | 5th harmonic from B12 into n78 | 4th IMD |  | - 5th harmonic issue will be covered in DC_12A_n78A - FFS |
|  | DC_12A_n78A | - | 2nd IMD | Yes | - FFS - The cross band isolation issue already covered in Table 7.3B.2.3.4-1 |
| DC_2_n66-n78 | DC_2A_n66A | 2nd harmonic from B2 into n78 2nd harmonic from n66 inton78 | 2nd & 4th IMDs | - | - These harmonic problems already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - FFS |
| DC_12_n66-n78 | DC_12A_n66A | 2nd harmonic from n66 inton78 5th harmonic from B12 into n78 | 5th IMD |  | - 2nd harmonic is same as DC_66A_n78A and 5th harmonic will be covered in DC_12A_n78A - FFS |
|  | DC_12A_n78A | 3rd harmonic from B12 into n66 | 3rd IMD |  | - The harmonic problem already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - FFS |
| DC_7_n66-n78 | DC_7A_n66A | 2nd harmonic from n66 into n78 | 3rd IMD |  | - Harmonic problem already covered in DC_66A_n78A - FFS |
| DC_66_n66-n78 | DC_66A_n66A | 2nd harmonic from B66 into n78 | 2nd & 4th IMDs |  | - Harmonic problem already covered in DC_66A_n78A - FFS |
| DC_20_n1-n28 | DC_20A_n28A | 3rd harmonic from n28 into n1 | 5th IMD | - | - Harmonic problem already covered in DC_1A_n28A - FFS |
| DC_20_n7-n28 | DC_20A_n7A | - | 3rd & 5th IMDs | - | - FFS |
| DC_20_n7-n78 | DC_20A_n7A | 4th harmonic from B20 into n78 | 2nd & 4th IMDs | - | - The harmonic problem already specified in Table 7.3B.2.3.1-1 in TS38.101-3 - FFS |
| DC_3_n7-n28 | DC_3A_n7A, DC_3C_n7A |  | 2nd IMD | - | - FFS |
| DC_2_n41-n71 | DC_2A_n41A | - | 2nd & 5th IMDs | - | - FFS |
|  | DC_2A_n71A | 4th harmonic from n71 into n41 | 2nd IMD | - | The harmonic problem will be treated CA_n41A_n71A - FFS |
| DC_18_n3-n78 | DC_18A_n3A | 2nd harmonic from n3 into n78 4th harmonic from B18 into n78 | 3rd & 5th IMDs | - | These harmonic problem already specified in in Table 7.3B.2.3.1-1 in TS38.101-3 - FFS |
| DC_8_n1-n78 | DC_8A_n1A | 4th harmonic from B8 into n78 | 3rd IMD | - | - The harmonic problem already specified in in Table 7.3B.2.3.1-1 in TS38.101-3 - FFS |
| DC_3_n40-n79 | DC_3A_n40A | 2nd harmonic from n40 into n79 | 5th IMD | - | - The 2nd harmonic problem will be studied in CA_n40-n79. - FFS |
|  | DC_3A_n79A | - | 5th IMD | - | - FFS |
| DC_3_n41-n79 | DC_3A_n41A | 2nd harmonic from n41 into n79 | 2nd & 5th IMDs | Yes | - The 2nd harmonic problem will be studied in CA_n41-n79. - FFS - The cross band isolation issue already covered in Table 7.3B.2.3.4-1 |
| DC_8_n40-n79 | DC_8A_n40A | 2nd harmonic from n40 into n79 5th harmonic from B8 into n79 | 4th IMD | - | - These harmonic problem will be treated lower order DC or CA band combos. - FFS |
|  | DC_8A_n79A | - | 4th IMD | - | - FFS |
| DC_8_n41-n79 | DC_8A_n41A | 2nd harmonic from n41 into n79 5th harmonic from B8 into n79 | 3rd IMD | - | - These harmonic problem will be treated lower order DC or CA band combos. - FFS |
|  | DC_8A_n79A | 3rd harmonic from B8 into n41 | 3rd IMD | - | - FFS |
| DC_39_n40-n79 | DC_39A_n40A | 2nd harmonic from n40 into n79 | 4th IMD | - | - The harmonic problem will be treated CA_n40-n79 band combos. - FFS |
| DC_8_n3-n28 | DC_8A_n3A | - | 2nd & 5th IMDs | - | - FFS |
| DC_7-7_n66-n78 | DC_7A_n66A | 2nd harmonic from n66 into n78 | 3rd IMD | - | -The harmonic problem already covered in Table 7.3B.2.3.1-1 in TS38.101-3 - FFS |

In this paper, we provide our MSD analysis results to support EN-DC operation by dual transmission.

- MSD analysis

In rel-16 NR new WI, RAN4 also consider shared antenna RF architectures for NSA UE in sub-6GHz as LTE system. So we consider shared antenna RF architecture for general NSA DC UE to derive MSD levels.

For the MSD analysis of these 3DL/2UL EN-DC NR UE, we assume the parameters and attenuation levels based on current UE RF FE components as shown in Table 1 and 2.

Table 2 show the RF component isolation parameters to derive MSD level at sub-6GHz.

Table 2: UE RF Front-end component parameters

| UE ref. architecture Component | Triplexer-Diplexer Architecture w/ single ant. |  |  |  | Cascaded Diplexer Architecture w/ single ant. |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | DC_39A_n41A-n79A, DC_40A_n41A_n79A, DC_12A_n7A-n78A, DC_2A_n66A-n78A DC_12A_n66A-n78A, DC_7A_n66A-n78A DC_66A_n66A-n78A, DC_20A_n7A-n78A DC_18A_n3A-n78A, DC_28A_n3A-n78A DC_8A_n1A-n78A, DC_3A_n40A-n79A DC_3A_n41A-n79A, DC_8A_n40A-n79A DC_8A_n41A-n79A, DC_39A_n40-n79A DC_7A-7A_n66A-n78A |  |  |  | DC_20A_n1A-n28A DC_20A_n7A-n28A DC_3A_n7A-n28A DC_2A_n41A-n71A DC_8A_n3A-n28A |  |  |  |
|  | IP2 (dBm) | IP3 (dBm) | IP4 (dBm) | IP5 (dBm) | IP2 (dBm) | IP3 (dBm) | IP4 (dBm) | IP5 (dBm) |
| Ant. Switch | 112 | 68 | 55 | 55 | 112 | 68 | 55 | 55 |
| Triplexer | 110 | 72 | 55 | 52 |  |  |  |  |
| Quadplexer |  |  |  |  | 112 | 72 | 55 | 52 |
| Diplexer | 115 | 87 | 55 | 55 | 115 | 87 | 55 | 55 |
| Duplexer | 100 | 75 | 55 | 53 | 100 | 75 | 55 | 53 |
| PA Forward | 28.0 | 32 | 30 | 28 | 28.0 | 32 | 30 | 28 |
| PA Reversed | 40 | 30.5 | 30 | 30 | 40 | 30.5 | 30 | 30 |
| LNA | 10 | 0 | 0 | -10 | 10 | 0 | 0 | -10 |

Table 3 show the isolation levels according to the RF component.

Table 3: UE RF Front-end component isolation parameters

| Isolation Parameter | Value (dB) | Comment |
| --- | --- | --- |
| Antenna to Antenna | 10 | Main antenna to diversity antenna |
| PA (out) to PA (in) | 60 | PCB isolation (PA forward mixing) |
| Triplexer | 20 | High/low band isolation |
| Diplexer | 25 | High/low band isolation |
| PA (out) to PA (out) | 60 | L-H/H-L cross-band |
| PA (out) to PA (out) | 50 | H-H cross-band |
| LNA (in) to PA (out) | 60 | L-H/H-L cross-band |
| LNA (in) to PA (out) | 50 | H-H cross-band |
| Duplexer | 50 | Tx band rejection at Rx band |

For the EN-DC_66A_n66A-78A with dual uplink, we need more information how simultaneous transmission between LTE 66 and NR 66 will be allocated each CC with sub-band gap.  Currently, RAN4 do not specified the general RF requirements for intra-band non-contiguous CA or DC.

So, we recommend the uplink intra-band non-contiguous CA will be treated after specify the general RF core requirements for intra-band non-contiguous CA or DC in rel-16.

Based on these assumptions, we proposed the MSD levels as below.

Table 4: Proposed MSD test configuration and results by IMD problems

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | DL BW (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_39A_n41A-n79A | 39 | IMD2 | ¦fB39-fn41¦ | 1900 | 5 | 25 | 1900 | 5 | N/A |
|  | n41 |  |  | 2620 | 10 | 50 | 2620 | 10 |  |
|  | n79 |  |  | 4520 | 40 | 216 | 4520 | 40 | 29.8 |
|  | 39 | IMD5 | ¦4*fB39 -fn41¦ | 1885 | 5 | 25 | 1885 | 5 | N/A |
|  | n41 |  |  | 2660 | 10 | 50 | 2660 | 10 |  |
|  | n79 |  |  | 4880 | 40 | 216 | 4880 | 40 | 4.4 |
|  | 39 | IMD2 | ¦fB39 -fn79¦ | 1900 | 5 | 25 | 1900 | 5 | N/A |
|  | n79 |  |  | 4520 | 40 | 216 | 4520 | 40 |  |
|  | n41 |  |  | 2620 | 10 | 50 | 2620 | 10 | 30.2 |
|  | 39 | IMD5 | ¦4*fB39 -fn79¦ | 1885 | 5 | 25 | 1885 | 5 | N/A |
|  | n79 |  |  | 4880 | 40 | 216 | 4880 | 40 |  |
|  | n41 |  |  | 2660 | 10 | 50 | 2660 | 10 | 4.3 |
| DC_40A_n41A-n79A | 40 | IMD2 | ¦fB40+fn41¦ | 2340 | 5 | 25 | 2340 | 5 | N/A |
|  | n41 |  |  | 2600 | 10 | 50 | 2600 | 10 |  |
|  | n79 |  |  | 4940 | 40 | 216 | 4940 | 10 | 30.5 |
| DC_12A_n7A-n78A | 12 | IMD4 | ¦2*fB12 -2*fn7¦ | 708 | 5 | 25 | 738 | 5 | N/A |
|  | n7 |  |  | 2520 | 5 | 25 | 2640 | 5 |  |
|  | n78 |  |  | 3624 | 10 | 50 | 3624 | 10 | 8.4 |
|  | 12 | IMD2 | ¦fB12 -fn78¦ | 708 | 5 | 25 | 738 | 5 | N/A |
|  | n78 |  |  | 3360 | 10 | 50 | 3360 | 10 |  |
|  | n7 |  |  | 2542 | 5 | 25 | 2662 | 5 | 28.7 |
| DC_2A_n66A-n78A | 2 | IMD2 | ¦fB2+fn66¦ | 1880 | 5 | 25 | 1960 | 5 | N/A |
|  | n66 |  |  | 1740 | 5 | 25 | 2140 | 5 |  |
|  | n78 |  |  | 3620 | 10 | 50 | 3620 | 10 | 29.4 |
|  | 2 | IMD4 | ¦fB2 -3*fn66¦ | 1880 | 5 | 25 | 1960 | 5 | N/A |
|  | n66 |  |  | 1740 | 5 | 25 | 2140 | 5 |  |
|  | n78 |  |  | 3340 | 10 | 50 | 3340 | 10 | 8.9 |
| DC_12A_n66A-n78A | 12 | IMD5 | ¦2*fB12-3*fn66¦ | 703 | 5 | 25 | 733 | 5 | N/A |
|  | n66 |  |  | 1720 | 5 | 25 | 2120 | 5 |  |
|  | n78 |  |  | 3754 | 10 | 50 | 3754 | 10 | 4.1 |
|  | 12 | IMD3 | ¦2*fB12 -fn78¦ | 703 | 5 | 25 | 733 | 5 | N/A |
|  | n78 |  |  | 3546 | 10 | 50 | 3546 | 10 |  |
|  | n66 |  |  | 1740 | 5 | 25 | 2140 | 5 | 16.5 |
| DC_7A_n66A-n78A | 7 | IMD3 | ¦2*fB7-fn66¦ | 2542 | 5 | 25 | 2662 | 5 | N/A |
|  | n66 |  |  | 1740 | 5 | 25 | 2140 | 5 |  |
|  | n78 |  |  | 3344 | 10 | 50 | 3344 | 10 | 16.0 |
| DC_66A_n66A-n78A | 66 | IMD2 | ¦fB66+fn66¦ | TBD | 5 | 25 | TBD | 5 | N/A |
|  | n66 |  |  | TBD | 5 | 25 | TBD | 5 |  |
|  | n78 |  |  | TBD | 10 | 50 | TBD | 10 | TBD |
|  | 66 | IMD4 | ¦3*fB66 -fn66¦ | TBD | 5 | 25 | TBD | 5 | N/A |
|  | n66 |  |  | TBD | 5 | 25 | TBD | 5 |  |
|  | n78 |  |  | TBD | 10 | 50 | TBD | 10 | TBD |
| DC_20A_n1A-n28A | 20 | IMD5 | ¦fB20-4*fn28¦ | 837 | 5 | 25 | 796 | 5 | N/A |
|  | n28 |  |  | 738 | 5 | 25 | 793 | 5 |  |
|  | n1 |  |  | 1925 | 5 | 25 | 2115 | 5 | 3.3 |
| DC_20A_n7A-n28A | 20 | IMD3 | ¦2*fB20-fn7¦ | 857 | 5 | 25 | 816 | 5 | N/A |
|  | n7 |  |  | 2512 | 5 | 25 | 2632 | 5 |  |
|  | n28 |  |  | 743 | 5 | 25 | 798 | 5 | 15.7 |
|  | 20 | IMD5 | ¦4*fB20-fn7¦ | 837 | 5 | 25 | 796 | 5 | N/A |
|  | n7 |  |  | 2555 | 5 | 25 | 2675 | 5 |  |
|  | n28 |  |  | 738 | 5 | 25 | 793 | 5 | 2.8 |
| DC_20A_n7A-n78A | 20 | IMD2 | ¦fB20+fn7¦ | 837 | 5 | 25 | 796 | 5 | N/A |
|  | n7 |  |  | 2555 | 5 | 25 | 2675 | 5 |  |
|  | n78 |  |  | 3392 | 10 | 50 | 3392 | 10 | 28.8 |
|  | 20 | IMD4 | ¦2*fB20-2*fn7¦ | 837 | 5 | 25 | 796 | 5 | N/A |
|  | n7 |  |  | 2555 | 5 | 25 | 2675 | 5 |  |
|  | n78 |  |  | 3436 | 10 | 50 | 3436 | 10 | 8.5 |
| DC_3A_n7A-n28A | 3 | IMD2 | ¦fB3-fn7¦ | 1730 | 5 | 25 | 1825 | 5 | N/A |
|  | n7 |  |  | 2518 | 5 | 25 | 2638 | 5 |  |
|  | n28 |  |  | 733 | 5 | 25 | 788 | 5 | 29.0 |
| DC_2A_n41A-n71A | 2 | IMD2 | ¦fB2-fn41¦ | 1900 | 5 | 25 | 1980 | 5 | N/A |
|  | n41 |  |  | 2530 | 10 | 50 | 2530 | 10 |  |
|  | n71 |  |  | 676 | 5 | 50 | 630 | 5 | 28.7 |
|  | 2 | IMD5 | ¦3*fB2 -2*fn41¦ | 1900 | 5 | 25 | 1980 | 5 | N/A |
|  | n41 |  |  | 2530 | 10 | 50 | 2530 | 10 |  |
|  | n71 |  |  | 686 | 5 | 50 | 640 | 5 | 3.8 |
|  | 2 | IMD2 | ¦fB2+fn71¦ | 1900 | 5 | 25 | 1980 | 5 | N/A |
|  | n71 |  |  | 686 | 5 | 50 | 640 | 5 |  |
|  | n41 |  |  | 2586 | 10 | 50 | 2586 | 10 | 29.2 |
| DC_18A_n3A-n78A | 18 | IMD3 | ¦2*fB18+fn3¦ | 820 | 5 | 25 | 865 | 5 | N/A |
|  | n3 |  |  | 1750 | 5 | 25 | 1845 | 5 |  |
|  | n78 |  |  | 3390 | 10 | 50 | 3390 | 10 | 15.2 |
|  | 18 | IMD5 | ¦2*fB18 -3*fn3¦ | 820 | 5 | 25 | 865 | 5 | N/A |
|  | n3 |  |  | 1750 | 5 | 25 | 1845 | 5 |  |
|  | n78 |  |  | 3610 | 10 | 50 | 3610 | 10 | 4.0 |
| DC_8A_n1A-n78A | 8 | IMD3 | ¦2*fB8+fn1¦ | 900 | 5 | 25 | 945 | 5 | N/A |
|  | n1 |  |  | 1945 | 5 | 25 | 2135 | 5 |  |
|  | n78 |  |  | 3745 | 10 | 50 | 3745 | 10 | 14.9 |
| DC_3A_n40A-n79A | 3 | IMD5 | ¦4*fB3-fn40¦ | 1720 | 5 | 25 | 1815 | 5 | N/A |
|  | n40 |  |  | 2330 | 5 | 25 | 2330 | 5 |  |
|  | n79 |  |  | 4550 | 40 | 216 | 4550 | 40 | 4.7 |
|  | 3 | IMD5 | ¦4*fB3-fn79¦ | 1720 | 5 | 25 | 1815 | 5 | N/A |
|  | n79 |  |  | 4550 | 40 | 216 | 4550 | 40 |  |
|  | n40 |  |  | 2330 | 5 | 25 | 2330 | 5 | 3.2 |
| DC_3A_n41A-n79A | 3 | IMD2 | ¦fB3-fn41¦ | 1770 | 5 | 25 | 1865 | 5 | N/A |
|  | n41 |  |  | 2670 | 10 | 50 | 2670 | 10 |  |
|  | n79 |  |  | 4440 | 40 | 216 | 4440 | 40 | 30.8 |
|  | 3 | IMD5 | ¦4*fB3-fn41¦ | 1770 | 5 | 25 | 1865 | 5 | N/A |
|  | n41 |  |  | 2550 | 10 | 50 | 2550 | 10 |  |
|  | n79 |  |  | 4530 | 40 | 216 | 4530 | 40 | 5.2 |
| DC_8A_n40A-n79A | 8 | IMD4 | ¦3*fB8+fn40¦ | 885 | 5 | 25 | 930 | 5 | N/A |
|  | n40 |  |  | 2305 | 5 | 25 | 2305 | 5 |  |
|  | n79 |  |  | 4960 | 40 | 216 | 4960 | 40 | 10.7 |
|  | 8 | IMD4 | ¦3*fB8-fn79¦ | 885 | 5 | 25 | 930 | 5 | N/A |
|  | n79 |  |  | 4960 | 40 | 216 | 4960 | 40 |  |
|  | n40 |  |  | 2305 | 5 | 25 | 2305 | 5 | 9.2 |
| DC_8A_n41A-n79A | 8 | IMD3 | ¦2*fB8+fn41¦ | 910 | 5 | 25 | 955 | 5 | N/A |
|  | n41 |  |  | 2650 | 10 | 50 | 2650 | 10 |  |
|  | n79 |  |  | 4470 | 40 | 216 | 4470 | 40 | 16.3 |
|  | 8 | IMD3 | ¦2*fB8-fn79¦ | 910 | 5 | 25 | 955 | 5 | N/A |
|  | n79 |  |  | 4470 | 40 | 216 | 4470 | 40 |  |
|  | n41 |  |  | 2650 | 10 | 50 | 2650 | 10 | 15.5 |
| DC_39A_n40A-n79A | 39 | IMD4 | ¦fB39-3*fn40¦ | 1917.5 | 5 | 25 | 1917.5 | 5 | N/A |
|  | n40 |  |  | 2302.5 | 5 | 25 | 2302.5 | 5 |  |
|  | n79 |  |  | 4980 | 40 | 216 | 4980 | 40 | 5.8 |
| DC_8A_n3A-n28A | 8 | IMD2 | ¦fB8-fn3¦ | 912.5 | 5 | 25 | 957.5 | 5 | N/A |
|  | n3 |  |  | 1712.5 | 5 | 25 | 1807.5 | 5 |  |
|  | n28 |  |  | 745 | 5 | 25 | 800 | 5 | 30.4 |
|  | 8 | IMD5 | ¦3*fB8 -2*fn3¦ | 910 | 5 | 25 | 955 | 5 | N/A |
|  | n3 |  |  | 1750 | 5 | 25 | 1845 | 5 |  |
|  | n28 |  |  | 715 | 5 | 25 | 770 | 5 | 2.6 |
| DC_7A-7A_n66A-n78A | 7 | IMD3 | ¦2*fB7 -fn66¦ | 2555 | 5 | 25 | 2675 | 5 | N/A |
|  | n66 |  |  | 1740 | 5 | 25 | 2140 | 5 |  |
|  | n78 |  |  | 3370 | 10 | 50 | 3370 | 10 | 15.1 |

- Conclusions

In this contribution, we provide the required MSD levels based on shared antenna RF architectures to support NSA DC operation in sub-6GHz. Based on the analysis in session 2, we proposed as below

Proposal 1: The proposed MSD test configuration and MSD levels should be considered to specify the MSD requirements in related TR and TS for EN-DC band combinations.

- ======== Text Proposal for TR37.716-21-21 section 6.X ========

6.39	DC_40A_n41A-n79A

----- Unchanged sections omitted -----

### 6.39.5		MSD

Based on the analysis in the UE-coexistence studies, compared to its fall back mode, there is additional MSD issue due to dual uplink DC_40A_n41A operation falling into n79 DL of the third band for this band combination.  The required MSD are shown in the following table.

Table 6.39.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_40A_n41A-n79A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_40A_n41A-n79A | 40 | IMD2 | ¦fB40+fn41¦ | 2340 | 5 | 25 | 2340 | N/A |
|  | n41 |  |  | 2600 | 10 | 50 | 2600 |  |
|  | n79 |  |  | 4940 | 40 | 216 | 4940 | 30.5 |

----- Unchanged sections omitted -----

6.x DC_39A_n41A-n79A

----- Unchanged sections omitted -----

### 6.x.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there are IMD2 & IMD5 products produced by Band 39 and n41 that impact the reference sensitivity of NR band n79.

In addition, there are IMD2 & IMD5 products produced by Band 39 and n79 that impact the reference sensitivity of NR band n41. The required MSD are shown in the following table.

Table 6.x.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_39A_n41A-n79A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_39A_n41A-n79A | 39 | IMD2 | ¦fB39-fn41¦ | 1900 | 5 | 25 | 1900 | N/A |
|  | n41 |  |  | 2620 | 10 | 50 | 2620 |  |
|  | n79 |  |  | 4520 | 40 | 216 | 4520 | 29.8 |
|  | 39 | IMD5 | ¦4*fB39 -fn41¦ | 1885 | 5 | 25 | 1885 | N/A |
|  | n41 |  |  | 2660 | 10 | 50 | 2660 |  |
|  | n79 |  |  | 4880 | 40 | 216 | 4880 | 4.4 |
|  | 39 | IMD2 | ¦fB39 -fn79¦ | 1900 | 5 | 25 | 1900 | N/A |
|  | n79 |  |  | 4520 | 40 | 216 | 4520 |  |
|  | n41 |  |  | 2620 | 10 | 50 | 2620 | 30.2 |
|  | 39 | IMD5 | ¦4*fB39 -fn79¦ | 1885 | 5 | 25 | 1885 | N/A |
|  | n79 |  |  | 4880 | 40 | 216 | 4880 |  |
|  | n41 |  |  | 2660 | 10 | 50 | 2660 | 4.3 |

6.X	DC_12_n7-n78

----- Unchanged sections omitted -----

### 6.X.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD4 products produced by Band 12 and n7 that impact the reference sensitivity of NR band n78.

In addition, there is IMD2 product produced by Band 12 and n78 that impact the reference sensitivity of NR Band n7. The required MSD are shown in the following table.

Table 6.X.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_12A_n7A-n78A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_12A_n7A-n78A | 12 | IMD4 | ¦2*fB12 -2*fn7¦ | 708 | 5 | 25 | 738 | N/A |
|  | n7 |  |  | 2520 | 5 | 25 | 2640 |  |
|  | n78 |  |  | 3624 | 10 | 50 | 3624 | 8.4 |
|  | 12 | IMD2 | ¦fB12 -fn78¦ | 708 | 5 | 25 | 738 | N/A |
|  | n78 |  |  | 3360 | 10 | 50 | 3360 |  |
|  | n7 |  |  | 2542 | 5 | 25 | 2662 | 28.7 |

6.X	DC_2_n66-n78

----- Unchanged sections omitted -----

### 6.X.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there are IMD2 & IMD4 products produced by Band 2 and n66 that impact the reference sensitivity of NR band n78. The required MSD are shown in the following table.

Table 6.X.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_2A_n66A-n78A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_2A_n66A-n78A | 2 | IMD2 | ¦fB2+fn66¦ | 1880 | 5 | 25 | 1960 | N/A |
|  | n66 |  |  | 1740 | 5 | 25 | 2140 |  |
|  | n78 |  |  | 3620 | 10 | 50 | 3620 | 29.4 |
|  | 2 | IMD4 | ¦fB2 -3*fn66¦ | 1880 | 5 | 25 | 1960 | N/A |
|  | n66 |  |  | 1740 | 5 | 25 | 2140 |  |
|  | n78 |  |  | 3340 | 10 | 50 | 3340 | 8.9 |

6.X	DC_12_n66-n78

----- Unchanged sections omitted -----

### 6.X.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD5 products produced by Band 12 and n66 that impact the reference sensitivity of NR band n78.

In addition, there is IMD3 product produced by Band 12 and n78 that impact the reference sensitivity of NR Band n66. The required MSD are shown in the following table.

Table 6.X.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_12A_n66A-n78A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_12A_n66A-n78A | 12 | IMD5 | ¦2*fB12-3*fn66¦ | 703 | 5 | 25 | 733 | N/A |
|  | n66 |  |  | 1720 | 5 | 25 | 2120 |  |
|  | n78 |  |  | 3754 | 10 | 50 | 3754 | 4.1 |
|  | 12 | IMD3 | ¦2*fB12 -fn78¦ | 703 | 5 | 25 | 733 | N/A |
|  | n78 |  |  | 3546 | 10 | 50 | 3546 |  |
|  | n66 |  |  | 1740 | 5 | 25 | 2140 | 16.5 |

6.X	DC_7_n66-n78

----- Unchanged sections omitted -----

### 6.X.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD3 products produced by Band 7 and n66 that impact the reference sensitivity of NR band n78. The required MSD is shown in the following table.

Table 6.X.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_7A_n66A-n78A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_7A_n66A-n78A | 7 | IMD3 | ¦2*fB7-fn66¦ | 2542 | 5 | 25 | 2662 | N/A |
|  | n66 |  |  | 1740 | 5 | 25 | 2140 |  |
|  | n78 |  |  | 3344 | 10 | 50 | 3344 | 16.0 |

6.X	DC_20_n1-n28

----- Unchanged sections omitted -----

### 6.X.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD5 products produced by Band 20 and n28 that impact the reference sensitivity of NR band n1. The required MSD is shown in the following table.

Table 6.X.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_20A_n1A-n28A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_20A_n1A-n28A | 20 | IMD5 | ¦fB20-4*fn28¦ | 837 | 5 | 25 | 796 | N/A |
|  | n28 |  |  | 738 | 5 | 25 | 793 |  |
|  | n1 |  |  | 1925 | 5 | 25 | 2115 | 3.3 |

6.X	DC_20_n7-n28

----- Unchanged sections omitted -----

### 6.X.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there are IMD3 & IMD5 products produced by Band 20 and n7 that impact the reference sensitivity of NR band n28. The required MSD are shown in the following table.

Table 6.X.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_20A_n7A-n28A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_20A_n7A-n28A | 20 | IMD3 | ¦2*fB20-fn7¦ | 857 | 5 | 25 | 816 | N/A |
|  | n7 |  |  | 2512 | 5 | 25 | 2632 |  |
|  | n28 |  |  | 743 | 5 | 25 | 798 | 15.7 |
|  | 20 | IMD5 | ¦4*fB20-fn7¦ | 837 | 5 | 25 | 796 | N/A |
|  | n7 |  |  | 2555 | 5 | 25 | 2675 |  |
|  | n28 |  |  | 738 | 5 | 25 | 793 | 2.8 |

6.X	DC_20_n7-n78

----- Unchanged sections omitted -----

### 6.X.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there are IMD2 & IMD4 products produced by Band 20 and n7 that impact the reference sensitivity of NR band n78. The required MSD are shown in the following table.

Table 6.X.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_20A_n7A-n78A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_20A_n7A-n78A | 20 | IMD2 | ¦fB20+fn7¦ | 837 | 5 | 25 | 796 | N/A |
|  | n7 |  |  | 2555 | 5 | 25 | 2675 |  |
|  | n78 |  |  | 3392 | 10 | 50 | 3392 | 28.8 |
|  | 20 | IMD4 | ¦2*fB20-2*fn7¦ | 837 | 5 | 25 | 796 | N/A |
|  | n7 |  |  | 2555 | 5 | 25 | 2675 |  |
|  | n78 |  |  | 3436 | 10 | 50 | 3436 | 8.5 |

6.X	DC_3_n7-n28

----- Unchanged sections omitted -----

### 6.X.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD2 products produced by Band 3 and n7 that impact the reference sensitivity of NR band n28. The required MSD is shown in the following table.

Table 6.X.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_3A_n7A-n28A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A_n7A-n28A | 3 | IMD2 | ¦fB3-fn7¦ | 1730 | 5 | 25 | 1825 | N/A |
|  | n7 |  |  | 2518 | 5 | 25 | 2638 |  |
|  | n28 |  |  | 733 | 5 | 25 | 788 | 29.0 |

6.X	DC_2_n41-n71

----- Unchanged sections omitted -----

### 6.X.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there are IMD2 & IMD5 products produced by Band 2 and n41 that impact the reference sensitivity of NR band n71.

In addition, there is IMD2 product produced by Band 2 and n71 that impact the reference sensitivity of NR Band n41. The required MSD are shown in the following table.

Table 6.X.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_2A_n41A-n71A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_2A_n41A-n71A | 2 | IMD2 | ¦fB2-fn41¦ | 1900 | 5 | 25 | 1980 | N/A |
|  | n41 |  |  | 2530 | 10 | 50 | 2530 |  |
|  | n71 |  |  | 676 | 5 | 50 | 630 | 28.7 |
|  | 2 | IMD5 | ¦3*fB2 -2*fn41¦ | 1900 | 5 | 25 | 1980 | N/A |
|  | n41 |  |  | 2530 | 10 | 50 | 2530 |  |
|  | n71 |  |  | 686 | 5 | 50 | 640 | 3.8 |
|  | 2 | IMD2 | ¦fB2+fn71¦ | 1900 | 5 | 25 | 1980 | N/A |
|  | n71 |  |  | 686 | 5 | 50 | 640 |  |
|  | n41 |  |  | 2586 | 10 | 50 | 2586 | 29.2 |

6.X	DC_18_n3-n78

----- Unchanged sections omitted -----

### 6.X.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD3 & IMD5 products produced by Band 18 and n3 that impact the reference sensitivity of NR band n78. The required MSD are shown in the following table.

Table 6.X.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_18A_n3A-n78A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_18A_n3A-n78A | 18 | IMD3 | ¦2*fB18+fn3¦ | 820 | 5 | 25 | 865 | N/A |
|  | n3 |  |  | 1750 | 5 | 25 | 1845 |  |
|  | n78 |  |  | 3390 | 10 | 50 | 3390 | 15.2 |
|  | 18 | IMD5 | ¦2*fB18 -3*fn3¦ | 820 | 5 | 25 | 865 | N/A |
|  | n3 |  |  | 1750 | 5 | 25 | 1845 |  |
|  | n78 |  |  | 3610 | 10 | 50 | 3610 | 4.0 |

6.X	DC_8_n1-n78

----- Unchanged sections omitted -----

### 6.X.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD3 products produced by Band 8 and n1 that impact the reference sensitivity of NR band n78. The required MSD is shown in the following table.

Table 6.X.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_8A_n1A-n78A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_8A_n1A-n78A | 8 | IMD3 | ¦2*fB8+fn1¦ | 900 | 5 | 25 | 945 | N/A |
|  | n1 |  |  | 1945 | 5 | 25 | 2135 |  |
|  | n78 |  |  | 3745 | 10 | 50 | 3745 | 14.9 |

6.X	DC_3_n40-n79

----- Unchanged sections omitted -----

### 6.X.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD5 products produced by Band 3 and n40 that impact the reference sensitivity of NR band n79.

In addition, there is IMD5 product produced by Band 3 and n79 that impact the reference sensitivity of NR Band n40. The required MSD are shown in the following table.

Table 6.X.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_3A_n40A-n79A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A_n40A-n79A | 3 | IMD5 | ¦4*fB3-fn40¦ | 1720 | 5 | 25 | 1815 | N/A |
|  | n40 |  |  | 2330 | 5 | 25 | 2330 |  |
|  | n79 |  |  | 4550 | 40 | 216 | 4550 | 4.7 |
|  | 3 | IMD5 | ¦4*fB3-fn79¦ | 1720 | 5 | 25 | 1815 | N/A |
|  | n79 |  |  | 4550 | 40 | 216 | 4550 |  |
|  | n40 |  |  | 2330 | 5 | 25 | 2330 | 3.2 |

6.X	DC_3_n41-n79

----- Unchanged sections omitted -----

### 6.X.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there are IMD2 & IMD5 products produced by Band 3 and n41 that impact the reference sensitivity of NR band n79. The required MSD are shown in the following table.

Table 6.X.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_3A_n41A-n79A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_3A_n41A-n79A | 3 | IMD2 | ¦fB3-fn41¦ | 1770 | 5 | 25 | 1865 | N/A |
|  | n41 |  |  | 2670 | 10 | 50 | 2670 |  |
|  | n79 |  |  | 4440 | 40 | 216 | 4440 | 30.8 |
|  | 3 | IMD5 | ¦4*fB3-fn41¦ | 1770 | 5 | 25 | 1865 | N/A |
|  | n41 |  |  | 2550 | 10 | 50 | 2550 |  |
|  | n79 |  |  | 4530 | 40 | 216 | 4530 | 5.2 |

6.X	DC_8_n40-n79

----- Unchanged sections omitted -----

### 6.X.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD4 products produced by Band 8 and n40 that impact the reference sensitivity of NR band n79.

In addition, there is IMD4 product produced by Band 8 and n79 that impact the reference sensitivity of NR Band n40. The required MSD are shown in the following table.

Table 6.X.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_8A_n40A-n79A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_8A_n40A-n79A | 8 | IMD4 | ¦3*fB8+fn40¦ | 885 | 5 | 25 | 930 | N/A |
|  | n40 |  |  | 2305 | 5 | 25 | 2305 |  |
|  | n79 |  |  | 4960 | 40 | 216 | 4960 | 10.7 |
|  | 8 | IMD4 | ¦3*fB8-fn79¦ | 885 | 5 | 25 | 930 | N/A |
|  | n79 |  |  | 4960 | 40 | 216 | 4960 |  |
|  | n40 |  |  | 2305 | 5 | 25 | 2305 | 9.2 |

6.X	DC_8_n41-n79

----- Unchanged sections omitted -----

### 6.X.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD3 products produced by Band 8 and n41 that impact the reference sensitivity of NR band n79.

In addition, there is IMD3 product produced by Band 8 and n79 that impact the reference sensitivity of NR Band n41. The required MSD are shown in the following table.

Table 6.X.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_8A_n41A-n79A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_8A_n41A-n79A | 8 | IMD3 | ¦2*fB8+fn41¦ | 910 | 5 | 25 | 955 | N/A |
|  | n41 |  |  | 2650 | 10 | 50 | 2650 |  |
|  | n79 |  |  | 4470 | 40 | 216 | 4470 | 16.3 |
|  | 8 | IMD3 | ¦2*fB8-fn79¦ | 910 | 5 | 25 | 955 | N/A |
|  | n79 |  |  | 4470 | 40 | 216 | 4470 |  |
|  | n41 |  |  | 2650 | 10 | 50 | 2650 | 15.5 |

6.X	DC_39_n40-n79

----- Unchanged sections omitted -----

### 6.X.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD4 products produced by Band 39 and n40 that impact the reference sensitivity of NR band n79. The required MSD is shown in the following table.

Table 6.X.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_39A_n40A-n79A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_39A_n40A-n79A | 39 | IMD4 | ¦fB39-3*fn40¦ | 1917.5 | 5 | 25 | 1917.5 | N/A |
|  | n40 |  |  | 2302.5 | 5 | 25 | 2302.5 |  |
|  | n79 |  |  | 4980 | 40 | 216 | 4980 | 5.8 |

6.X	DC_8_n3-n28

----- Unchanged sections omitted -----

### 6.X.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there are IMD2 & IMD5 products produced by Band 8 and n3 that impact the reference sensitivity of NR band n28. The required MSD are shown in the following table.

Table 6.X.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_8A_n3A-n28A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_8A_n3A-n28A | 8 | IMD2 | ¦fB8-fn3¦ | 912.5 | 5 | 25 | 957.5 | N/A |
|  | n3 |  |  | 1712.5 | 5 | 25 | 1807.5 |  |
|  | n28 |  |  | 745 | 5 | 25 | 800 | 30.4 |
|  | 8 | IMD5 | ¦3*fB8 -2*fn3¦ | 910 | 5 | 25 | 955 | N/A |
|  | n3 |  |  | 1750 | 5 | 25 | 1845 |  |
|  | n28 |  |  | 715 | 5 | 25 | 770 | 2.6 |

6.X	DC_7-7_n66-n78

----- Unchanged sections omitted -----

### 6.X.5	MSD

Based on Table 5.3-1 in TR 37.716-21-21 [2], there is IMD3 products produced by Band 7 and n66 that impact the reference sensitivity of NR band n78. The required MSD is shown in the following table.

Table 6.X.5-1: MSD exception for Scell due to dual uplink operation for EN-DC_7A-7A_n66A-n78A

| DC bands | UL DC | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | DL Fc (MHz) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_7A-7A_n66A-n78A | 7 | IMD3 | ¦2*fB7 -fn66¦ | 2555 | 5 | 25 | 2675 | N/A |
|  | n66 |  |  | 1740 | 5 | 25 | 2140 |  |
|  | n78 |  |  | 3370 | 10 | 50 | 3370 | 15.1 |

----- End of changed -----

- References

[1] RP-191744, “Revised WID on EN-DC LTE x bands DL and 1UL (x=1,2,3,4) + NR 2 bands DL and 1UL in REl-16” LG Electronics

[2] R4-191xxxx,  “TP on summary of self-interference studies”, LG Electronics

[3] Way Forward MSD for below 6GHz, Huawei, Vivo, LG Electronics

[4] TR37.716-02-01 v0.6.0

[5] TS38.101-3 v15.5.0