3GPP TSG-RAN WG4 Meeting #92bis	R4-1911466

Chongqing, China, 14th – 18th October, 2019

Source:	Huawei, HiSilicon

Title:	TP for TR 36.716-04-01 for CA_2A-7A-13A-66A, CA_2A-7C-13A-66A

Agenda item:	7.4.2

Document for:	Approval

Background

This contribution provides text proposal on the LTE CA configuration CA_2A-7A-13A-66A, CA_2A-7C-13A-66A as defined in the revised WID [1].

Reference

[1]	RP-191730, “Rel-16 LTE inter-band CA for x bands DL (x=4, 5) with 1 band UL”, Nokia, Nokia Shanghai Bell

Text Proposal

---Start of changes---

5.X	CA_2-7-13-66

5.X.1	Channel bandwidths per operating band for CA

Table 5.x.1-1: Supported E-UTRA bandwidths per CA configuration for 4DL inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration | Uplink CA configurations | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| CA_2A-7A-13A-66A | - | 2 |  |  | Yes | Yes | Yes | Yes | 70 | 0 |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 13 |  |  | Yes | Yes |  |  |  |  |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_2A-7C-13A-66A | - | 2 |  |  | Yes | Yes | Yes | Yes | 90 | 0 |
|  |  | 7 | See CA_7C Bandwidth combination set 1 in Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | 13 |  |  | Yes | Yes |  |  |  |  |
|  |  | 66 |  |  | Yes | Yes | Yes | Yes |  |  |

5.X.2		ΔTIB,c and ΔRIB,c values

ΔRIB,c values for CA_2-7-13-66 are derived from CA_2-7-66 and CA_7-13-66.  For the ΔTIB,c the  values which derived from CA_2-7-66 and CA_7-13-66 are reused. Values will be defined in the following tables in TS 36.101.

Table 5.x.2-1: ΔTIB,c (three bands)

| E-UTRA operating band combination | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_2-7-13-66 | 2 | 0.5 |
|  | 7 | 0.5 |
|  | 13 | 0.3 |
|  | 66 | 0.5 |

Table 5.x.2-2: ΔRIB,c (three bands)

| E-UTRA operating band combination | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_2-7-13-66 | 2 | 0.3 |
|  | 7 | 0.5 |
|  | 13 | 0 |
|  | 66 | 0.5 |

5.X.3	REFSENS requirements

There are no reference sensitivity requirements needed.

---End of changes---