3GPP RAN WG4 Meeting #92bis		R4-1911513

Chongqing, China, 14 – 18th October 2019

Agenda item:	9.16.1

Source:	Apple Inc.

Title:	Derivation of peak EIRP and REFSENS for band n259

WI/SI:	NR_n259-Core

Release:	Rel-16

Document for:	Discussion

# 1	Introduction

The work item to introduce the new Band n259 [1] has reached the following agreements [2]:

Agreement:

-	Band plan for n259 is 39.5GHz – 43.5GHz

-	Intra-band CA requirements shall apply for inter-band CA configurations between n260 and n259 including intra-band capabilities.

-	Requirements for single band n259 and associated multiband requirements shall be agreed as a single CR pack for n259 WI.

-	The scope of the multiband requirements shall be further discussed.

According to the Rapporteur’s work plan [3] a suggested work plan refinement [4], the work on deriving multi-band requirements for band n259 has progressed during the RAN4 #92 meeting with the following agreements [5]:

WF

-	To define the maximum output power and Refsens OTA requirements for n259, companies are encouraged to provide the link budget parameters for Tx and Rx in the next meeting according to the tables in the WF.

-	PC3 is considered as the highest priority

-	For the analysis and simulations, packaging environment shall be considered (plastic, glass)

Multi-band relaxation

-	Companies are encouraged to submit analysis on multiband relaxations for band configurations including n259 with respect to provided parameters in the previous slide.

The WF in [5] also contains exemplary tables of parameters for the peak EIRP and REFSENS derivation.

This contribution provides derivations for peak EIRP and REFSENS based on conducted component and antenna array capabilities for band n259.

# 2	Discussion

## 2.1	Beam forming losses

As observed in [6], the assumptions on the conducted transmitter and receiver compontents’ performance have a direct relationship with the peak EIRP and REFSENS requirements for band n259.  At least in comparison with band n260, these components are expected to achieve lower peformance metrics.  This effect can be quantified as beam forming losses.

Drawing upon the experience of defining beam correspondence requirements in Rel-15, we can refer to Clause 7.8.3 of TR38.817-01 [7] for the methodology used in the evaluation.  For the purpose of quantifying beam forming losses, we reuse most of the methodology and adapt it to the receive and transmit operations.

When considering the beam forming losses for peak EIRP and peak EIS, we calculate the difference in the 100%-tile CDF value between the ideal CDF and the impaired CDF, where the impairments include phase shifter error, phase shifter loss, gain variation, and RSRP estimation accuracy.  TS38.101-2 [8] defines all requirements for extreme temperature conditions (ETC), and with the approval of the study on enhanced test methods for FR2 UEs [9] we should anticipate beam peak search procedures to be feasible under ETC in the Rel-16 conformance test specification.  Thus, the peak EIRP and REFSENS requirements for band n259 should take beam forming losses under ETC into account.

Toward this end, three spherical coverage simulation were performed according to the following parameters:

-	2 panels (array of 4 dual polarized patch antennas) positioned on opposite sides of the UE (see TR38.803 [10])

-	Sector codebook: 9 beams (15 degree step)

-	Mutual coupling, form factor integration and implementation losses not modeled

-	Reference coverage pattern: no impairments

-	Impaired coverage pattern

-	Amplitude error δak = [1, 2] dB

-	Phase error δpk = [16, 32]˚

-	RSRP error ∆k = [1.5, 3.0] dB

The analysis steps are illustrated in Figure 1 below.

Figure 1: Analysis steps to quantify beam forming losses for peak EIRP and REFSENS

The impaired and reference CDFs for one simulation set are shown in Figure 2 below.

Figure 2: Analysis of beam forming losses

The results are summarized in Table 1 below.

Table 1: Summary of beam forming losses simulations

| Phase err | Mag err | RSRP err | 50-tile CDF degradation | Peak CDF degradation |
| --- | --- | --- | --- | --- |
| 16 | 1 | 1.5 | -1.5 | -0.8 |
| 32 | 2 | 1.5 | -2.9 | -1.6 |
| 32 | 2 | 3.0 | -3.5 | -1.6 |

Observation 1:	The total beam forming loss for band n259 is 1.6 dB.

## 2.2	Antenna rolloff vs. frequency

As further observed in [6], the TE1 mode onset can be taken into account in the peak EIRP and REFSENS budgeting as antenna rolloff vs. frequency.

a)

b)

Figure 2: Analytical results for patch array + glass (steered to boresight, f=39 GHz)

Although there are several different ways to account for this design effect in the 3GPP requirement, the simplest way is to use the antenna rolloff loss versus frequency and to set this value to a practical number based on the patch array + glass analysis above.

Observation 2:	The antenna rolloff loss versus frequency for band n259 is 3.0 dB.

## 2.3	Peak EIRP

Collecting the beam forming loss and antenna rolloff loss versus frequency terms into the common EIRP link budget table from [5], we obtain the budget illustrated in Table 2 below.

Table 2: Peak EIRP budget for band n259

| Parameter | Unit | Nominal value | Contribution to tolerance | Comments |
| --- | --- | --- | --- | --- |
| Pout per element | dBm | 13 |  | Nominal value does not account for variation over PVT |
| # of antennas in an array |  | 4 |  | Reuse n260 value |
| Total conducted power per polarization | dBm | 19 | -1 | Account for PVT |
| Avg antenna element gain | dBi | 4 |  | Reuse n260 value |
| Antenna rolloff loss versus frequency | dB | -3 |  | Rolloff is greater due to TE1 onset (see R4-1908011) |
| Realized antenna array gain | dBi | 7 | 0 |  |
| Polarization gain | dB | 2.8 |  | Reuse n260 value |
| Mismatch and transmission line loss including load pull | dB | -2.3 | -0.7 |  |
| Beam forming loss (phase shifter and amplitude error) | dB | -1 | -0.6 | Model phase shifter and amplitude error under ETC |
| Finite beam table | dB |  |  | Simulated with BF loss above |
| Beam forming loss (one beam table fits all) | dB |  |  | Simulated with BF loss above |
| Form factor integration losses | dB | -4 | -2 | Update relative to n260 value |
| Total implementation loss | dB | -7.3 | -3.3 |  |
| Peak EIRP (Nominal) | dBm | 21.5 |  | For information only (not a requirement) |
| Tolerance (+/-) | dB |  | 4.3 | For information only (not a requirement) |
| Peak EIRP (Minimum) | dBm | 17.2 |  |  |
| Peak EIRP (Maximum) | dBm | 25.8 |  | For information only (not a requirement) |

Proposal 1:	The single-band minimum peak EIRP requirement for band n259 is 17.2 dBm.

## 2.4	REFSENS

Collecting the beam forming loss and antenna rolloff loss versus frequency terms into the common REFSENS link budget table from [5], we obtain the budget illustrated in Table 2 below.

Table 2: REFSENS budget for band n259

| Parameter | Unit | Value |  |  |  | Comments |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth | MHz | 50 | 100 | 200 | 400 |  |
| 10log(Rx BW) | dB | 76.99 | 80 | 83.01 | 86.02 |  |
| Thermal noise | dBm/Hz | -174 | -174 | -174 | -174 | Assume ETC |
| Noise Figure | dB | 12 | 12 | 12 | 12 | Accounts for variation over PVT |
| Realized array gain | dB | 7 | 7 | 7 | 7 | from peak EIRP calculation |
| Beamforming loss | dB | -1.6 | -1.6 | -1.6 | -1.6 | Model phase shifter and amplitude error under ETC |
| Mismatch and transmission line loss | dB | -3 | -3 | -3 | -3 | from peak EIRP calculation (with tolerance) |
| Form factor integration losses | dB | -6 | -6 | -6 | -6 | from peak EIRP calculation (with tolerance) |
| Total implementation loss | dB | -10.6 | -10.6 | -10.6 | -10.6 |  |
| SNR | dB | -1 | -1 | -1 | -1 | reuse n260 value |
| REFSENS | dBm | -82.41 | -79.4 | -76.39 | -73.38 |  |

Proposal 2:	The single-band maximum peak EIS requirement for band n259 is -82.4 dBm/50 MHz, -79.4 dBm/100 MHz, -76.4 dBm/200 MHz, and -73.4 dBm/400 MHz.

# 3	Conclusions

This contribution has provided derivations for peak EIRP and REFSENS based on conducted component and antenna array capabilities for band n259 and has made the following observations and proposals:

Observation 1:	The total beam forming loss for band n259 is 1.6 dB.

Observation 2:	The antenna rolloff loss versus frequency for band n259 is 3.0 dB.

Proposal 1:	The single-band minimum peak EIRP requirement for band n259 is 17.2 dBm.

Proposal 2:	The single-band maximum peak EIS requirement for band n259 is -82.4 dBm/50 MHz, -79.4 dBm/100 MHz, -76.4 dBm/200 MHz, and -73.4 dBm/400 MHz.

# 4	References

- RP-190765, “New WID: introduction of Band n259”

- R4-1907901, “RAN4 #91 Meeting report”

- R4-1904379, "Scope and workplan for introduction of Band n259 WI"

- R4-1903072, "Recommendations for the work plan on Band n259"

- R4-1910511, “WF on link budget parameters for Rx/Tx of band n259,” Huawei, Apple Inc., 3GPP RAN4 #92, August 2019

- R4-1908011, “Multi-band requirement framework for Band n259,” Apple Inc., 3GPP RAN4 #92, August 2019

- 3GPP TR38.817-01, “General aspects for User Equipment (UE) Radio Frequency (RF) for NR”

- 3GPP TS38.101-2, “User Equipment (UE) radio transmission and reception; Part 2: Range 2 Standalone”

- RP-192322, “New SID: Study on enhanced test methods for FR2,” Apple Inc., CAICT, 3GPP RAN #85, September 2019

- 3GPP TR38.803, “Study on new radio access technology: Radio Frequency (RF) and co-existence aspects”