3GPP TSG-RAN WG4 Meeting #92bis	R4-1911703

# Chongqing, China, 14th– 18th October, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-3 | CR | - | rev | - | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for correction on NW signalling architecture for TS 38.101-3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO, INC. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There is a misalignment about “modifiedMPR-Behaviour” between RAN2 and RAN4 specification. To solve this misalignment issue, associating modified MPR for intra-band non-contiguous n41 EN-DC with new NS_01M and NS_04M while having no impact on RAN2 specification at late stage of Rel-15. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Use “modifiedMPR-Behaviour” capability for both A-MPR and modified MPR Associate Modified MPR requirements specified in TS 38.101-3 with NS index versioning. Specify new NS of NS_04M for n41 to distinguish if the UE has modified MPR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misalignment about “modifiedMPR-Behaviour” between RAN2 and RAN4 specification still exist. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | ANNEX H.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR ... CR ... 38.521-3 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

---Start of changes---

H.1	Indication of modified MPR behavior

This annex contains the definitions of the bits in the field modifiedMPRbehavior indicated in the IE RF-Parameters [7] by a UE supporting an MPR or A-MPR modified in a later release of this specification. modifiedMPRbehavior is indicated in TS 36.211 [7] by an 8-bit bitmap per NR band.

Table H.1-1: Definitions of the bits in the field modifiedMPRbehavior

| NR Band | Index of field (bit number) | Definition (description of the supported functionality if indicator set to one) | Notes |
| --- | --- | --- | --- |
| n41 | 0 (leftmost bit) | - For NS_01, EN-DC contiguous intraband MPR as defined in clausue 6.2B.2.1 of 38.101-3 v15.5.0 | - This bit may be set to 1 by a UE supporting DC_(n)41AA UE EN-DC |
|  | 1 | - For_NS_04, EN-DC contiguous intraband MPR as defined in clause 6.2B.2.2 of 38.101-3 v15.5.0 |  |
|  | 2 | - For_NS_01M, EN-DC non-contiguous intraband MPR as defined in clause 6.2B.2.2 of 38.101-3 v15.5.0 | - This bit may be set to 1 by a UE supporting DC_41A_n41A EN-DC |
|  | 3 | - For_NS_04M, EN-DC non-contiguous intraband MPR as defined in clause 6.2B.2.2 of 38.101-3 v15.5.0 |  |
| n71 | 0 (leftmost bit) | - For NS_01, EN-DC contiguous intraband MPR as defined in clausue 6.2B.2.1 of 38.101-3 v15.5.0 | - This bit may be set to 1 by a UE supporting DC_(n)71AA UE EN-DC |
|  | 1 | - For NS_35, EN-DC contiguous intraband MPR as defined in clausue 6.2B.2.1 of 38.101-3 v15.5.0 |  |

---End of changes---