3GPP TSG-RAN WG4 Meeting #92bis	R4-1911772

Chongqing, China, 14 – 18 October 2019

Agenda Item:	7.14.3.1

Source:	Ericsson

Title:	Discussion on channel quality reporting for eMTC

Document for:	Discussion

# 1	Introduction

RAN4#91 discussed the channel quality reporting in MSG3 but agreed to wait further RAN1/RAN2 progress [1]. We will discuss further on the channel quality reporting for eMTC for both in CONNECTED mode and in MSG3, based on the RAN1/RAN2 agreements listed in Appendix.

# 2	Discussion

RAN1 has agreed to define the DL quality as follows:

- DL quality is defined as the repetition level and/or aggregation level of hypothetical MPDCCH BLER of 1%.

This definition is similar to the DL quality used for the DL quality reporting on anchor carriers introduced in Rel-14 NB-IoT. For the DL channel quality report for the BL/CE UEs, unlike NB-IoT DL channel quality report, it is agreed to report more information such as preferred narrowband(s) and aggregation level in addition to the MPDCCH repetition level.

## 2.1	Quality report with MSG3 in IDLE mode

Figure 1 illustrates an example of measurement period and measured resources for MSG3-based channel quality report, where MPDCCH is transmitted on two narrowbands for RAR scheduling.

According to RAN1 agreements, the narrowband(s) for downlink quality measurement include the narrowband(s) on which MPDCCH of RAR is monitored. This means RAN4 should assume UE measures the channel quality in T2 in Figure 1.

Figure 1	DL channel quality reporting for BL/CE UEs in MSG3.

## 2.2	Quality report in CONNECTED mode

According to RAN1 agreements, the channel quality in CONNECTED mode is supported for both CE modes A and B, and the reported values are based at least on narrowband(s) where the User-specific search space is monitored. We expect the quality reporting in CONNECTED mode will reuse the quality reporting in IDLE mode.

## 2.3	Summary of reported value and bandwidth for DL channel quality report

Table 1, Table 2, and Table 3 summarize the RAN1/RAN2 agreements so far from RAN4 RRM requirement point of view. It is observed from the summary that RAN4 needs more inputs from RAN1/RAN2, for example,

- RAN1 agreed that the MSG3-based channel quality information is up to 8 bits for non-EDT case. RAN1 also agreed to report the channel quality information with 2 bits for non-EDT case. RAN4 need to wait further the reported values using both 8 bits and 2 bits formats.

- RAN4 need to wait for the RAN1 agreement especially for DL channel quality in CONNECTED mode.

Table 1	Reported DL channel quality value.

| UE state | CE Mode | Repetition number | Reported value |
| --- | --- | --- | --- |
| IDLE | Mode A | =1 | Rep1 together with aggregation level |
|  |  | >1 | Repetition number with fixed aggregation level 24 |
|  | Mode B | N/A | Repetition number with fixed aggregation level 24 |
| CONNECTED | Mode A | =1 | TBD (Wait for RAN1 agreements) |
|  |  | >1 | Repetition number with fixed aggregation level 24 |
|  | Mode B | N/A | Repetition number with fixed aggregation level 24 |

Table 2	Reported bandwidth for DL channel quality.

| UE state | CE Mode | Frequency hopping | Reported RBs |
| --- | --- | --- | --- |
| IDLE | Mode A | Enabled | Wideband only |
|  |  | Disabled | One narrowband only |
|  | Mode B | Enabled | TBD (Wait for RAN1 agreements) |
|  |  | Disabled | One narrowband only |
| CONNECTED | Mode A | Enabled | TBD (Wait for RAN1 agreements) |
|  |  | Disabled | One narrowband only |
|  | Mode B | Enabled | TBD (Wait for RAN1 agreements) |
|  |  | Disabled | One narrowband only |

Table 3	Measurement resources

| UE state | Measurement resources |
| --- | --- |
| IDLE | Narrowband(s) on which MPDCCH of RAR is monitored. |
| CONNECTED | At least the narrowband(s) on which USS MPDCCH is monitored. RAN4 need to wait for RAN1 agreements further because RAN1 may agree to allow measurements on other narrowband(s) than USS narrowband. |

# 3	RRM core requirements for MSG3-based channel quality report for Cat-M UEs.

Since it is observed RAN4 can start to specify the RRM core requirements for MSG3-based downlink channel quality report in IDLE, we provide our initial thoughts how to specify the core requirements. Note RAN4 need wait further for the information elements from RAN1/RAN2.

The DL channel quality provides the serving eNB with information shown in Table 4 to satisfy the hypothetical MPDCCH block error rate of 1% with the parameters specified in Table 5. Table 4 Downlink channel quality in MSG3-based channel quality reporting. CE Mode; Values. CE Mode A; MPDCCH aggregation level if the minimum repetition number is 1. Otherwise Minimum MPDCCH repetition number with fixed MPDCCH aggregation level 24.. CE Mode B; Minimum MPDCCH repetition number with fixed MPDCCH aggregation level 24.. Table 5 MPDCCH transmission parameters for MSG3-based channel quality reporting. Parameters; Values. CE Modes; CE Mode A; CE Mode B. DCI format; 6-1A; 6-1B. Starting OFDM symbols; 2; Bandwidth ≥ 10MHz 3; 3MHz ≤ Bandwidth < 10MHz 4; Bandwidth = 1.4MHz; 2; Bandwidth ≥ 10MHz 3; 3MHz ≤ Bandwidth < 10MHz 4; Bandwidth = 1.4MHz. MPDCCH transmission type; Distributed; Distributed. Reference signal; DMRS; DMRS. The reported MPDCCH aggregation level and repetition level shall be derived from the channel quality measured in the period in the carrier where the random access response is transmitted.

# 4	Conclusion

Proposal 1: RAN4 specifies the RRM core requirements for MSG3-based channel quality reporting as shown in Section 3.

Proposal 2: For DL quality reporting in CONNECTED mode, RAN4 waits for RAN1/RAN2 decision.

# References

- R4-1907733, “Way forward on Rel-16 MTC RRM enhancement”, Ericsson, Nokia, Nokia Shanghai Bell, Qualcomm Incorporated, Huawei, HiSilicon, Sierra Wireless, S.A.

# Appendix

## RAN1 agreements

RAN1#94 Agreement Prioritize the following alternatives for DL quality report in Msg3 in MTC, for CE Mode A and CE Mode B separately: - CQI (for CE Mode A) - Repetition number related to UE decoding of actual or hypothetical MPDCCH/PDSCH - FFS if aggregation level needs to be reported when repetition number equal to 1 - RSRP/RSRQ Companies are encouraged to provide details and/or performance evaluation results Agreement Whether the DL quality report is included in Msg3 is indicated in SIB and/or RAR. Above applies in case the UE supports DL quality report in Msg3. RAN1#94bis Agreement For CE mode A (PRACH CE level 0, 1), the downlink channel quality is down-selected among the following in RAN1#95: - CQI - The repetition number and/or aggregation level that the UE needs to decode hypothetical MPDCCH with BLER of 1% - Support both CQI and repetition number and/or aggregation level that the UE needs to decode hypothetical MPDCCH with BLER of 1% Agreement For CE Mode B, the downlink channel quality reported in Msg3 is denoted as the repetition number that the UE recommends to achieve a hypothetical MPDCCH decoding BLER of 1% Agreement CRS may be used as the reference signal for measurement of DL quality metric for measurement report in Msg3. Agreement Enabling of DL quality report is indicated in SIB. RAN1#95 Agreement For CE mode A (PRACH CE level 0, 1), the downlink channel quality is the repetition number and/or aggregation level that the UE needs to decode hypothetical MPDCCH with BLER of 1% Agreement DL quality report is transmitted via higher layer signaling, e.g. MAC CE or RRC message RAN1#96 Agreement For DL quality report in Msg3: - If frequency hopping for MPDCCH is enabled, at least wideband DL quality is reported. - Wideband DL quality is calculated assuming transmission on all narrowband(s) to which the derived repetition number and/or aggregation level relates. - FFS if additionally reports DL quality on a preferred narrowband and/or position of the preferred narrowband. - If frequency hopping for MPDCCH is disabled, DL quality based on one narrowband is reported. Agreement For DL quality report in connected mode - DL quality is with respect to USS for both CE mode A and CE mode B. - DL quality is measured at least on narrowband(s) on which MPDCCH of USS is monitored. Agreement: For DL quality report in Msg3 in CE mode A (PRACH CE level 0, 1): - If the repetition number in DL quality information is larger than 1, only repetition number is reported with pre-defined maximum aggregation level; - If the repetition number in DL quality information equals to 1, FFS between: - Only aggregation level is reported with pre-defined repetition number equals to 1 - Repetition number equals to 1 is reported with pre-defined [maximum] aggregation level RAN1#96bis Agreement - DL quality report in Msg3 in IDLE mode and DL quality report in connected mode are configured separately. - DL quality report in Msg3 in IDLE mode is configured per PRACH CE level or per CE mode in the SI. - DL quality report in connected mode is configured by UE-specific RRC signaling. Agreement - For DL quality report in connected mode, DL quality is transmitted via higher layer signaling, e.g. MAC CE or RRC message. Working Assumption - For DL quality report in Msg3 for IDLE mode UEs, the narrowband(s) for downlink quality measurement includes at least the narrowband(s) on which MPDCCH of RAR is monitored. Agreement For channel quality report in Msg3, RAN1 send LS to RAN2 requesting the following information: - Number(s) of available bits for the quality reporting in Msg3 - Whether the quality report in Msg3 is transmitted in RRC or MAC CE Send LS to RAN2 and CC RAN4. Ask, from RAN2, perspective, whether the above might be different for different cases. Include as part of LS to be handled in the NB-IoT agenda item. Working Assumption For DL quality report in CE mode A (PRACH CE level 0, 1), the pre-defined maximum aggregation level is fixed to 24. RAN1#97 Agreement - Specify DL quality information with up to 8 bits for DL quality report in at least - Msg3 report for EDT case - Msg3 report for connected mode - Fixed range of DL quality information is defined for DL quality report with up to 8 bits Agreement For both DL quality report in Msg3 in IDLE mode and DL quality report in connected mode: - Confirm the following working assumption: - For DL quality report in CE mode A (PRACH CE level 0, 1), the pre-defined maximum aggregation level is fixed to 24. - For DL quality report in CE mode B (PRACH CE level 2, 3), the pre-defined maximum aggregation level is fixed to 24. Agreement Confirm the following working assumption: - For DL quality report in Msg3 for IDLE mode UEs, the narrowband(s) for downlink quality measurement includes at least the narrowband(s) on which MPDCCH of RAR is monitored. Agreement For DL quality report in connected mode, the mechanism for triggering is based on one or more of the following: - DCI (e.g. UL grant, PDCCH order) - MAC CE Agreement For IDLE mode DL quality report in Msg3 in CE mode A (PRACH CE level 0, 1) with up to 8 bits, if the repetition number in DL quality information equals to 1, select one between the following alternatives in RAN1#98 meeting: - Alternative 1: Repetition number =1 is reported with aggregation level - Alternative 2: Repetition number =1 is reported assuming aggregation level of 24 Agreement For IDLE mode DL quality report in Msg3 in CE mode A (PRACH CE level 0, 1) with up to 8 bits, if frequency hopping for MPDCCH is enabled, select one between the following alternatives in RAN1#98 meeting: - Alternative 1: Only wideband DL quality is reported. - Alternative 2: DL quality on a preferred narrowband and position of the preferred narrowband are reported in addition to wideband DL quality. - The preferred narrowband is selected within the set of narrowband(s) on which wideband DL quality is measured. RAN1#98 Agreement CQI report and DL quality report are separate features and it is up to eNB configuration/triggering (UE can be configured to report both CQI and DL quality) Agreement - Specify DL quality information with up to 8 bits for DL quality report in Msg3 report non-EDT case. - In addition to the 8-bit DL quality report, DL quality report with 2 bits is supported for non-EDT case - FFS: Details on the range of DL quality information for the 2-bit case - If the quality reporting in Msg-3 is configured in SIB - If the scheduled TBS is large enough for inclusion of the 8-bit DL quality report, the 8-bit DL quality report is used - Otherwise, the 2-bit DL quality report is used. Agreement For IDLE mode DL quality report in Msg3 in CE mode A with up to 8 bits, if the repetition number in DL quality information equals to 1 - Repetition number =1 is reported with aggregation level - A common table is used for CE mode A and B Agreement For DL quality report in Msg3 for IDLE mode UEs, downlink quality is measured on the narrowband(s) on which MPDCCH of RAR is monitored. No additional downlink quality measurement on other narrowband(s) is introduced. Agreement The draft LS is endorsed in R1-1909793. Agreement For IDLE mode DL quality report in Msg3 in CE mode A with up to 8 bits, if frequency hopping for MPDCCH is enabled, - Only wideband DL quality is reported. Agreement: - For DL quality report not in Msg3 in connected mode, DL quality report is configured by UE-specific RRC signaling. - For DL quality report in Msg3 in connected mode, the configuration of DL quality report is selected between in RAN1#98bis: - SIB - Reuse the configuration of DL quality report in Msg3 in IDLE mode - UE-specific RRC signaling - Reuse the configuration of DL quality report not in Msg3 in connected mode Working Assumption: If DL quality report in Msg3 in connected mode is enabled by higher layer configuration, UE always report DL quality in Msg3 in connected mode

## RAN2 agreements

RAN2#103bis agreements: Quality report in Msg3 is introduced for EDT. FFS for non-EDT. RAN2#104 agreements: RAN2 waits for progress on the discussion of MT-EDT before deciding on whether channel quality report in Msg3 is introduced for EDT. Channel quality report in Msg3 is introduced for non-EDT. RAN2#105 agreements: UE reports at most one DL quality measurement in Msg3 transmission. This is pending RAN1 agreement. For EDT, new MAC CE will be defined to report the channel quality in Msg3. FFS whether an LCID (lowest priority) or eLCID is used. Channel quality in Msg3 is reported with no explicit differentiation on whether the measurement was made in T1 or T2. RAN2#105bis agreements: eLCID based solution is not supported. A new LCID is used for eMTC pending approval in the main room. IoT informs the main room regarding the pending agreement above and the alternatives, e.g. to use one of LCID values reserved for NB-IoT or sidelink. RAN2#106 agreements: None RAN2#107 agreements: Agreements - Explicit signaling is introduced to trigger the aperiodic DL quality report in connected mode. FFS whether DCI or MAC CE. - MAC CE is used to provide the aperiodic DL quality report in connected mode. - DL quality report in Msg3 in idle mode is enabled via system information broadcast. - UE capability reporting is not needed for the DL quality report in Msg3 in idle mode. - UE capability reporting is introduced for the aperiodic DL quality report in connected mode.