TSG-RAN Working Group 4 (Radio) meeting #92-bis	R4-1911843

Chongqing, China, 14th – 18th October 2019

Source:	Ericsson

Title:	TP to TR 38.820: Improvement of example frequency range in subclause 5.2

Agenda item:	10.4.3

Document for:	Approval

# Introduction

At the last RAN4 meeting (RAN4#92 in Ljubljana) technical background information was added in TR 38.820. To capture technical background information a concept of identifying three example frequencies to cover was adopted.

In this contribution a text proposal to resolve the last open issues related to the definition of the example frequencies have been created. The text proposal is attached at the end of this contribution.

# Discussion

The technology characteristics around 24 GHz are likely to resemble the lower region of FR2 and around 7 GHz to be more similar to the upper region of FR1. Some level of differentiation is required for the analysis to harmonize the transition between FR1 and FR2. In order to have a reasonable work load considering the technology feasibility, there is a need to break down the 7-24 GHz into few smaller sub-ranges and limit the investigation for these frequency ranges to an “example frequency” per sub-range. The use of “example frequency” instead of “proxy frequency” is used to avoid possible confusion as “proxy frequency” was used when RAN4 studied the technology potential, limits and feasibility aspects related to FR2. At that time, the possible frequency bands were clearly defined for WRC-19 whereas for the frequency range 7-24 GHz, possible frequency bands have not been identified and any implication that they have should be avoided.

Traditionally, the spectrum within the frequency range 7 to 24 GHz have been divided in to frequency ranges defined by IEEE in [3]. The relevant IEEE bands for the frequency range 7 to 24 GHz are listed in Table 2-1.

Table 2-1: IEEE bands

| IEEE Band | Frequency Range (GHz) | Width (GHz) |
| --- | --- | --- |
| C | 4 to 8 | 4 |
| X | 8 to 12 | 4 |
| Ku | 12 to 18 | 6 |
| K | 18 to 27 | 9 |

It can be noticed that C, X, Ku and K covers frequencies from 4 to 27 GHz, so adaptations are required to fit the frequency range 7.125 GHz to 24.250 GHz. A possible break-down to sub-ranges is presented in Figure 2-1 where the boundaries in the proposed ranges are aligned with the regulatory landscape survey outlining current possible allocation to mobile systems, summarized in [2].

Using the IEEE definitions, the example frequency ranges can be created as:

- Example range 1 is constituted by the X-Band, which is extended to start at 7.125 GHz.

- Example range 2 is constituted by the Ku-Band.

- Example range 3 is constituted by the K-band, which is limited to 24.250 GHz.

In Figure 2-1, the IEEE band definitions are mapped towards the example frequencies determined for the frequency range 7 to 24 GHz to be used by RAN4.

# Figure 2-1: Example ranges

Using the IEEE definitions, the example frequency ranges can be created as:

- Example range 1 is constituted by the X-Band, which is extended to start at 7.125 GHz.

- Example range 2 is constituted by the Ku-Band.

- Example range 3 is constituted by the K-band, which is limited to 24.250 GHz.

The current version of TR 38.820 defines the example frequencies according to Table 2-2 as a result of an extensive discussion in RAN4..

Table 2-2: Example ranges

| Example range | Frequency (GHz) | Example frequency (GHz) |
| --- | --- | --- |
| 1 | 7.125 to X | 10 |
| 2 | X to Y | 15 |
| 3 | Y to 24.250 | 20 |

, where X and Y is open parameters definition frequency ranges as described in Table 2.3.

Table 2-3: Unresolved parameters

| Parameter | Frequency range (GHz) |
| --- | --- |
| X | 10 to 13 |
| Y | 16 to 18 |

The example frequencies have thus no relation to possible future frequency bands in this range but the feasibility studies around performance, and limitations could be useful in conjunction with co-existence studies when frequency bands in the future are considered in different regions and administrations.

The following aspects shall be noticed during further discussions;

- The above example ranges were selected only for purpose of RF technology analyses during SI and shall not be confused with potential new Frequency Ranges and band plan which could be defined by RAN4 for NR in future.

- The above example ranges are not fixed and could be modified based on RF technology findings during the SI phase.

- In case outcomes of the RF/requirement analyses will be found as sufficiently similar across adjacent example ranges, RAN4 will consider merging them towards SI conclusion on FR1/FR2 extension and/or new FR introduction.

The intension with the attached text proposal is to resolve and remove [] around the ranges in Table 5.2-1 in TR 38.820. In Table 2-4, an updated table have been created, where two notes have been added to describe the ranges between the example ranges.

Table 2-4: Example ranges

| Example range | Frequency (GHz) | Example frequency (GHz) |
| --- | --- | --- |
| 1 | 7.125 to X | 10 |
| 2 | X to Y | 15 |
| 3 | Y to 24.250 | 20 |
| Note 1: X is within the frequency range 10 to 13 GHz Note 2: Y is within the frequency range 16 to 18 GHz |  |  |

# Conclusion

A text proposal has been created to resolve the preliminary frequency ranges related to the definition of example frequencies. At the end on this contribution a text proposal is attached for approval.

# References

[1]	R4-1903455, “TP to TR 38.820: Addition of example frequencies in subclause 6.2”, Ericsson

[2]	R4-1906173, “TP to TR 38.820: Addition of example frequencies in subclause 6.2”, Ericsson

[3]	IEEE Std 521-2002, “Standard Letter Designations for Radar-Frequency Bands”, IEEE

TEXT PROPOSAL:

# 5	Study on general aspects

Editor’s note: consider aspects common to BS and UE

## 5.1	General

## 5.2	Selection of example frequencies

The performance trends related to vital components and technologies to be used within the frequency range 7 - 24 GHz indicates that the frequency range needs to be split up into sub-ranges, each with one example frequency. The intension with the example frequencies is that the technology evaluation is supposed to be performed per example frequency. The example frequencies are listed in table 5.2-1.

Table 5.2-1: Example ranges

| Example range | Frequency (GHz) | Example frequency (GHz) |
| --- | --- | --- |
| 1 | 7.125 to X | 10 |
| 2 | X to Y | 15 |
| 3 | Y to 24.250 | 20 |
| Note 1: X is within the frequency range 10 to 13 GHz Note 2: Y is within the frequency range 16 to 18 GHz |  |  |

The above example ranges will be selected only for purpose of RF technology analyses during SI and shall not be confused with potential new frequency ranges (as compared to the FR1 or FR2 defined for NR) and band plan which could be defined by RAN4 in harmonization with other services for NR in future.

The above example ranges are not fixed and could be modified based on RF technology findings during the SI phase.

In case outcomes of the RF requirement analyses will be found as sufficiently similar across adjacent example ranges, RAN4 will consider merging them towards SI conclusion on FR1/FR2 extension and/or new FR introduction.