3GPP TSG-RAN WG4 Meeting #92bis	R4-1911916

Chongqing , China, 14th – 18th Oct, 2019

Title: 	Discussion on synchronization related requirements for NR V2X

Source: 	Huawei, HiSilicon

Agenda item:	8.4.5.2

Document for:	Discussion

Introduction

In last RAN4 meeting, the RRM impacts for NR V2X was discussed and some RRM requirements for NR V2X were identified. In this contribution, we provide the discussion on synchronization related requirements for NR V2X based on RAN1 and RAN2 agreements.

Discussion

Currently, only Band n47 has been defined in TS38.101 for NR V2X. Hence, the discussion in this contribution is focused on the ITS band.

Selection/Reselection to gNB or eNB

In RAN4, the following agreements were achieved on the priority order for NR V2X synchronization

| GNSS-based synchronization | gNB/eNB-based synchronization |
| --- | --- |
| • P0: GNSS | • P0’: gNB/eNB |
| • P1: UE directly synchronized to GNSS | • P1’: UE directly synchronized to gNB/eNB |
| • P2: UE indirectly synchronized to GNSS | • P2’: UE indirectly synchronized to gNB/eNB |
| • P3: gNB/eNB | • P3’: GNSS |
| • P4: UE directly synchronized to gNB/eNB | • P4’: UE directly synchronized to GNSS |
| • P5: UE indirectly synchronized to gNB/eNB | • P5’: UE indirectly synchronized to GNSS |
| • P6: the remaining UEs have the lowest priority. | • P6’: the remaining UEs have the lowest priority. |

Based on the RAN1’s agreements, it can be observed that gNB and eNB are defined as the same priority. When UE is configured with EN-DC or NE-DC, both gNB and eNB are available. However, RAN1 has not clarified which one would be used as synchronization source. In RAN4, it was agreed [], to study this issue from RAN4 perspective and the potential RRM impact for following sub-bullets

    - How to choose gNB or eNB as synchronization reference source when they’re in the same priority, and

    - How to choose serving cell as synchronization reference source when multiple active serving cells under the same RAT(NR or LTE) are configured as candidate synchronization reference sources

    - How to facilitate the communication between UEs that synchronized to asynchronized synchronization reference sources.

In last RAN4 meeting, some companies provided solutions on synchronization source selection from gNB and eNB. It was proposed that the selection of gNB or eNB depends on whether the PCell and PSCell are synchronized or depends on which one has a better RSRP result. However, the purpose of synchronization source selection is to choose a reliable reference timing for V2X communication. The sidelink transmit timing error (Te) requirements have achieved some agreements in [2], and the Te requirements for gNB as reference timing and eNB as reference timing are defined as follows:

Table 1: Te requirements for gNB as reference timing

| Frequency Range of NR Uu | DL SSB SCS (kHz) | SL SCS in FR1 (kHz) | Te |
| --- | --- | --- | --- |
| FR1 | 15 | 15 | 14*64*Tc |
|  |  | 30 | 12*64*Tc |
|  |  | 60 | 12*64*Tc |
|  | 30 | 15 | 10*64*Tc |
|  |  | 30 | 10*64*Tc |
|  |  | 60 | 9*64*Tc |

Table 2: Te requirements for eNB as reference timing

| Frequency Range of NR SL | LTE DL CBW (MHz) | Te |
| --- | --- | --- |
| FR1 | >=3 | 14*64*Tc |

It can be observed that gNB could provide a better timing accuracy than eNB specifically for higher SCS. When gNB is used as synchronization source, the UE could be provided with a more reliable reference timing for V2X communication. Hence, gNB is suggested to be used as reference timing for NR sidelink when gNB and eNB are in the same priority.

Proposal 1: In EN-DC or NE-DC operation, the UE select gNB as synchronization source for NR V2X sidelink transmission when gNB and eNB are in the same priority.

In CA operation, there are multiple active serving cells under the same RAT and UE needs choose one of them as reference timing for NR sidelink transmission. There is similar issue for uplink transmission. The approach of selecting reference timing for uplink transmission can be reused for NR sidelink transmission. Hence, PCell or PSCell (if PCell is not configure) can be used as synchronization source.

Proposal 2: When there are multiple active serving cells under the same RAT, PCell or PSCell (if PCell is not configure) can be used as synchronization source.

Due to synchronization selection procedure, NR V2X UE may select different synchronization source in different scenario.

Figure 1: Example of synchronization source selection

As shown in figure 1, UE1 in EN-DC mode, UE2 in NR SA mode and UE3 in LTE SA mode are all (pre-)configured with gNB/eNB-based synchronization priority. UE4 is (pre-)configured with GNSS-based synchronization priority. According to the synchronization selection procedure in RAN1, UE2 will use gNB as synchronization source and UE3 will use eNB as synchronization source. UE4 will use GNSS as synchronization source. UE1 will use gNB or eNB as synchronization source. When gNB and eNB are asynchronized, UE1 with eNB as synchronization source could not have NR sidelink communication with UE2, or UE1 with gNB as synchronization source could not have NR sidelink communication UE3. However, how to solve sidelink reference timing misalignment between two UEs which need to communicate with each other is the synchronization selection procedure issue which shall be discussed in RAN1.

Observation 1: The timing misalignment between UEs with asynchronized synchronization reference sources cannot be solved from RRM perspective.

Selection/Reselection to SyncRef UE

RAN4 need to evaluate SyncRef UE detection performance, which includes the side condition of detectable SyncRef UE, SyncRef UE detection time and dropping rate of V2X sidelink communication. In RAN1, the S-SSB has been introduced for NR V2X sidelink synchronization. In RAN1, the following agreements on S-SSB design were reached in [3].

Agreements: - In NR V2X, S-SSB bandwidth is 11RBs. - PSBCH spans 11RBs. - The S-SSB is designed following combination 1. - Length-127 M-sequences for S-PSS and length-127 Gold sequences for S-SSS - Two symbols are used for each of S-PSS and S-SSS, respectively. Agreements: - For the evaluation of PSBCH performance, the payload size of NR V2X PSBCH is 56 bits including 24 bits of CRC. Working assumption: - For the NR SLSS, - Same sequence is used for both symbols of S-PSS - Same sequence is used for both symbols of S-SSS Agreements: - NR S-SSB structure for NCP is as follows: - For the case of ECP, the structure is the same as the above except that the number of PSBCH symbols after S-SSS is only 6

The purpose of SyncRef UE detection is to identify SLSS ID and obtain the timing information. UE could identify SLSS ID and obtain slot timing boundary information based on S-PSS/S-SSS detection. Based on RAN1’s design, the number of S-SSB transmissions within one S-SSB period is (pre)configurable and the following agreements were achieved.

Agreements: - Note: the values in bracket are subject to further discussion regarding potential removal all-together For FR1: - For 15kHz SCS, {1, [2]} - For 30kHz SCS, {1, 2, [4]} - For 60kHz SCS, {1, 2, 4, [8]} For FR2: - For 60kHz SCS, {1, 2, 4, 8, 16, 32} - For 120kHz SCS, {1, 2, 4, 8, 16, 32, 64}

If multiple S-SSBs are (pre)configured within one S-SSB period, UE needs to perform S-SSB index reading in order to obtain further timing information. For the SyncRef UEs with the same priority, UE selects the synchronization source based on the S-RSRP measurement results. Hence, the SyncRef UE identification delay includes S-PSS/S-SSS detection time, S-SSB index reading time and one S-RSRP measurement period.

Proposal 2: RAN4 shall evaluate the SyncRef UE identification delay which includes the following three aspects:

- S-PSS/S-SSS detection time

- S-SSB index reading time

- S-RSRP measurement period

For SyncRef UE selection/reselection, UE may need to drop V2X data and/or S-SSB transmission. According to synchronization source reselection procedure, SyncRef UEs are divided into different priority group. The SyncRef UEs to be identified can be synchronized or asynchronized with the current reference source. UE may need to drop V2X data and/or SLSS transmission for identifying SyncRef UE with same or higher priority than the current reference source.

Proposal 3: UE is allowed to drop NR V2X data/SLSS transmissions for detecting SyncRef UE with same or higher priority than the current reference source.

If all of them will be synchronized with the current reference source, then UE only need to drop V2X SLSS transmission. If some of them may be asynchronized with the current reference source, then UE should be allowed to drop V2X data and SLSS transmission. The dropping rate of V2X data/SLSS is related to NR SyncRef UE identification performance. The dropping rate of V2X data/SLSS transmission defined for LTE V2X can be used as the starting point. In LTE V2X, UE needs three SLSS occasions for identifying SyncRef UE. UE is allowed to drop 6% of its V2X data/SLSS transmissions during 8 seconds (SyncRef UE identification delay), which allows three SLSS searching windows for identifying both synchronized and asynchronized SyncRef UE. If UE need >3 SLSS occasions for identifying SyncRef UE in NR V2X, then RAN4 need to study whether to reuse the same dropping rate or reuse the same SyncRef UE identification delay.

Observation 2: The drop rate of NR V2X data/SLSS transmissions depends on NR SyncRef UE identification performance

Conclusions

This contribution provides our analysis on synchronization related RRM requirements for NR V2X sidelink. The following are provided:

Proposal 1: In EN-DC or NE-DC operation, the UE select gNB as synchronization source for NR V2X sidelink transmission when gNB and eNB are in the same priority.

Proposal 2: When there are multiple active serving cells under the same RAT, PCell or PSCell (if PCell is not configure) can be used as synchronization source.

Observation 1: The timing misalignment between UEs with asynchronized synchronization reference sources cannot be solved from RRM perspective.

Proposal 2: RAN4 shall evaluate the SyncRef UE indentification delay which includes the following three aspects:

- S-PSS/S-SSS detection time

- S-SSB index reading time

- S-RSRP measurement period

Proposal 3: UE is allowed to drop NR V2X data/SLSS transmissions for detecting SyncRef UE with same or higher priority than the current reference source

Observation 2: The drop rate of NR V2X data/SLSS transmissions depends on NR SyncRef UE identification performance.

Reference

- R4-1909994, “WF on NR V2X RRM requirement scope”, LG Electronics

- R4-1909993, “Way forward on UE sidelink timing requirements for NR V2X”, Huawei, HiSilicon

- R1-1909874, “LS to RAN4 on NR V2X S-SSB design”, 3GPP TSG RAN WG1 Meeting #98