3GPP TSG-RAN WG4 Meeting #92bis	R4-1911921

Chongqing , China, 14th – 18th Oct, 2019

Title: 	Discussion on CSI-RS based measurement performance for NR mobility

Source: 	Huawei, HiSilicon

Agenda item:	8.16.1.1

Document for:	Discussion

Introduction

RAN4 needs to study the CSI-RS measurement requirements for L3 mobility, including measurement period, CSI-RSRP measurement accuracy and the corresponding side condition. Based on the simulation assumptions in RAN4 email discussion, we provide the simulation results and analysis for CSI-RS based measurements in NR.

Discussion

Simulation Results

In this section, we provide the simulation results of CSI-RSRP measurement accuracy with different measurement bandwidth. The results in Tables 1-2 show 5%, 50% and 95% CDF of the delta measurements of CSI-RSRP.

Table 1: Delta measurements of measured CSI-RSRP (24RBs)

| SCS=15kHz |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Propagation condition | Samples | Density = 1 |  |  |  |  | Density = 3 |  |  |  |  |
|  |  | 5% | 50% | 95% | 50% - 5% | 95% -50% | 5% | 50% | 95% | 50% - 5% | 95% -50% |
| AWGN | 3 | -2.80 | -0.29 | 1.47 | 2.51 | 1.76 | -1.34 | -0.08 | 1.00 | 1.26 | 1.08 |
|  | 5 | -2.16 | -0.36 | 1.11 | 1.80 | 1.48 | -1.13 | -0.06 | 0.76 | 1.08 | 1.82 |
|  | 10 | -1.49 | -0.24 | 0.73 | 1.25 | 0.97 | -0.81 | -0.13 | 0.50 | 0.68 | 0.63 |
| EPA5 | 3 | -3.38 | -0.32 | 1.63 | 3.06 | 1.95 | -1.68 | -0.11 | 1.05 | 1.57 | 1.16 |
|  | 5 | -2.45 | -0.23 | 1.30 | 2.22 | 1.53 | -1.25 | -0.13 | 0.84 | 1.12 | 0.97 |
|  | 10 | -1.55 | -0.33 | 0.79 | 1.22 | 1.12 | -0.85 | -0.06 | 0.57 | 0.79 | 0.63 |
| ETU30 | 3 | -5.03 | -1.42 | 0.77 | 3.61 | 2.19 | -2.19 | -0.60 | 0.47 | 1.59 | 1.07 |
|  | 5 | -3.86 | -1.33 | 0.28 | 2.53 | 1.61 | -1.75 | -0.54 | 0.39 | 1.21 | 0.93 |
|  | 10 | -2.70 | -1.27 | -0.09 | 1.43 | 1.18 | -1.30 | -0.54 | 0.14 | 0.76 | 0.68 |
| ETU70 | 3 | -4.87 | -1.21 | 0.75 | 3.66 | 1.96 | -1.99 | -0.60 | 0.66 | 1.39 | 1.26 |
|  | 5 | -3.84 | -1.34 | 0.50 | 2.50 | 1.84 | -1.81 | -0.53 | 0.37 | 1.28 | 0.90 |
|  | 10 | -2.88 | -1.24 | -0.01 | 1.64 | 1.23 | -1.30 | -0.54 | 0.11 | 0.76 | 0.65 |
| SCS=30KHz |  |  |  |  |  |  |  |  |  |  |  |
| Propagation condition | Samples | Density = 1 |  |  |  |  | Density = 3 |  |  |  |  |
|  |  | 5% | 50% | 95% | 50% - 5% | 95% -50% | 5% | 50% | 95% | 50% - 5% | 95% -50% |
| AWGN | 3 | -2.79 | -0.31 | 1.39 | 2.48 | 1.71 | -1.45 | -0.09 | 0.89 | 1.36 | 0.98 |
|  | 5 | -2.13 | -0.24 | 1.15 | 1.89 | 1.39 | -1.04 | -0.09 | 0.70 | 0.95 | 0.79 |
|  | 10 | -1.53 | -0.25 | 0.73 | 1.28 | 0.97 | -0.76 | -0.09 | 0.52 | 0.67 | 0.61 |
| EPA5 | 3 | -3.80 | -0.38 | 1.54 | 3.42 | 1.92 | -1.74 | -0.12 | 1.06 | 1.62 | 1.18 |
|  | 5 | -2.47 | -0.27 | 1.27 | 2.20 | 1.54 | -1.27 | -0.07 | 0.84 | 1.10 | 0.91 |
|  | 10 | -1.61 | -0.26 | 0.77 | 1.35 | 1.03 | -0.84 | -0.11 | 0.60 | 0.73 | 0.71 |
| SCS= 60KHz |  |  |  |  |  |  |  |  |  |  |  |
| Propagation condition | Samples | Density = 1 |  |  |  |  | Density = 3 |  |  |  |  |
|  |  | 5% | 50% | 95% | 50% - 5% | 95% -50% | 5% | 50% | 95% | 50% - 5% | 95% -50% |
| AWGN | 3 | -2.85 | -0.23 | 1.47 | 2.61 | 1.71 | -1.39 | -0.13 | 0.96 | 1.26 | 1.09 |
|  | 5 | -2.16 | -0.32 | 1.08 | 1.84 | 1.41 | -1.14 | -0.08 | 0.78 | 1.06 | 0.86 |
|  | 10 | -1.49 | -0.29 | 0.80 | 1.20 | 1.09 | -0.76 | -0.09 | 0.53 | 0.67 | 0.62 |
| EPA5 | 3 | -3.26 | -0.39 | 1.49 | 2.87 | 1.88 | -1.47 | -0.10 | 0.98 | 1.37 | 1.08 |
|  | 5 | -2.15 | -0.34 | 1.16 | 1.81 | 1.50 | -1.18 | -0.14 | 0.77 | 1.04 | 0.91 |
|  | 10 | -1.52 | -0.29 | 0.76 | 1.23 | 1.05 | -0.77 | -0.11 | 0.56 | 0.66 | 0.67 |
| TDL-A 83Hz | 3 | -3.21 | -0.30 | 1.56 | 2.91 | 1.86 | -1.55 | -0.14 | 1.03 | 1.41 | 1.17 |
|  | 5 | -2.27 | -0.32 | 1.25 | 1.95 | 1.57 | -1.16 | -0.12 | 0.74 | 1.04 | 0.86 |
|  | 10 | -1.49 | -0.32 | 0.77 | 1.17 | 1.09 | -0.76 | -0.07 | 0.58 | 0.69 | 0.65 |
| TDL-C 83Hz | 3 | -3.20 | -0.46 | 1.33 | 2.74 | 1.79 | -1.42 | -0.11 | 0.97 | 1.31 | 1.08 |
|  | 5 | -2.32 | -0.35 | 1.10 | 1.96 | 1.45 | -1.10 | -0.07 | 0.84 | 1.03 | 0.91 |
|  | 10 | -1.68 | -0.35 | 0.64 | 1.33 | 0.99 | -0.78 | -0.09 | 0.55 | 0.69 | 0.64 |
| SCS= 120KHz |  |  |  |  |  |  |  |  |  |  |  |
| Propagation condition | Samples | Density = 1 |  |  |  |  | Density = 3 |  |  |  |  |
|  |  | 5% | 50% | 95% | 50% - 5% | 95% -50% | 5% | 50% | 95% | 50% - 5% | 95% -50% |
| TDL-A 83Hz | 3 | -3.88 | -0.76 | 1.27 | 3.12 | 2.03 | -1.61 | -0.09 | 0.95 | 1.52 | 1.04 |
|  | 5 | -2.78 | -0.70 | 0.73 | 2.08 | 1.43 | -1.12 | -0.12 | 0.69 | 1.00 | 0.81 |
|  | 10 | -2.09 | -0.68 | 0.36 | 1.41 | 1.04 | -0.79 | -0.08 | 0.58 | 0.71 | 0.66 |
| TDL-C 83Hz | 3 | -3.37 | -0.48 | 1.34 | 2.89 | 1.81 | -1.52 | -0.11 | 1.05 | 1.41 | 1.16 |
|  | 5 | -2.56 | -0.58 | 0.93 | 1.98 | 1.51 | -1.12 | -0.08 | 0.76 | 1.04 | 0.84 |
|  | 10 | -1.83 | -0.47 | 0.58 | 1.36 | 1.05 | -0.77 | -0.09 | 0.55 | 0.68 | 0.64 |

Table 2: Delta measurements of measured CSI-RSRP (48RBs/96RBs)

| SCS=15kHz |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Propagation condition | Samples | 48RBs (Density = 1) |  |  |  |  | 96RBs (Density = 1) |  |  |  |  |
|  |  | 5% | 50% | 95% | 50% - 5% | 95% -50% | 5% | 50% | 95% | 50% - 5% | 95% -50% |
| AWGN | 3 | -1.85 | -0.17 | 1.15 | 1.67 | 1.32 | -1.18 | -0.08 | 0.86 | 1.10 | 0.94 |
|  | 5 | -1.38 | -0.15 | 0.87 | 1.23 | 1.02 | -0.91 | -0.07 | 0.67 | 0.84 | 0.74 |
|  | 10 | -1.00 | -0.14 | 0.60 | 0.86 | 0.74 |  |  |  |  |  |
| EPA5 | 3 | -2.10 | -0.17 | 1.29 | 1.93 | 1.46 | -1.28 | -0.07 | 0.92 | 1.20 | 0.99 |
|  | 5 | -1.48 | -0.15 | 0.95 | 1.33 | 1.10 | -0.97 | 0.08 | 0.69 | 0.89 | 0.77 |
|  | 10 | -1.06 | -0.16 | 0.63 | 0.90 | 0.79 |  |  |  |  |  |
| ETU30 | 3 | -3.28 | -1.12 | 1.41 | 2.16 | 1.53 | -2.54 | -1.03 | 0.11 | 1.51 | 1.14 |
|  | 5 | -2.74 | -1.10 | 0.13 | 1.64 | 1.23 | -2.15 | -1.02 | -0.13 | 1.13 | 0.89 |
|  | 10 | -2.17 | -1.11 | -0.22 | 1.06 | 0.89 |  |  |  |  |  |
| ETU70 | 3 | -3.39 | -1.15 | 0.39 | 2.24 | 1.54 | -2.50 | -1.04 | 0.12 | 1.46 | 1.16 |
|  | 5 | -2.73 | -1.11 | 0.09 | 1.62 | 1.20 | -2.12 | -1.03 | -0.12 | 1.09 | 0.90 |
|  | 10 | -2.18 | -1.11 | -0.23 | 1.07 | 0.88 |  |  |  |  |  |
| SCS=30KHz |  |  |  |  |  |  |  |  |  |  |  |
| Propagation condition | Samples | 48RBs (Density = 1) |  |  |  |  | 96RBs (Density = 1) |  |  |  |  |
|  |  | 5% | 50% | 95% | 50% - 5% | 95% -50% | 5% | 50% | 95% | 50% - 5% | 95% -50% |
| AWGN | 3 | -1.89 | -0.16 | 1.14 | 1.73 | 1.30 | -1.18 | -0.09 | 0.86 | 1.09 | 0.95 |
|  | 5 | -1.36 | -0.15 | 0.90 | 1.21 | 1.05 | -0.94 | -0.08 | 0.65 | 0.85 | 0.73 |
|  | 10 | -0.99 | -0.14 | 0.59 | 0.85 | 0.73 |  |  |  |  |  |
| EPA5 | 3 | -1.98 | -0.18 | 1.21 | 1.80 | 1.39 | -1.29 | -0.11 | 0.89 | 1.18 | 1.00 |
|  | 5 | -1.47 | -0.17 | 0.89 | 1.30 | 1.06 | -0.97 | -0.10 | 0.67 | 0.87 | 0.77 |
|  | 10 | -1.00 | -0.14 | 0.61 | 0.86 | 0.75 |  |  |  |  |  |
| SCS= 60KHz |  |  |  |  |  |  |  |  |  |  |  |
| Propagation condition | Samples | 48RBs (Density = 1) |  |  |  |  | 96RBs (Density = 1) |  |  |  |  |
|  |  | 5% | 50% | 95% | 50% - 5% | 95% -50% | 5% | 50% | 95% | 50% - 5% | 95% -50% |
| AWGN | 3 | -1.80 | -0.16 | 1.16 | 1.64 | 1.32 | -1.19 | -0.08 | 0.86 | 1.12 | 0.94 |
|  | 5 | -1.37 | -0.14 | 0.89 | 1.23 | 1.03 | -0.92 | -0.07 | 0.68 | 0.85 | 0.75 |
|  | 10 | -0.98 | -0.15 | 0.59 | 0.83 | 0.74 |  |  |  |  |  |
| EPA5 | 3 | -1.98 | -0.21 | 1.16 | 1.76 | 1.37 | -1.33 | -0.13 | 0.87 | 1.20 | 1.00 |
|  | 5 | -1.46 | -0.18 | 0.88 | 1.28 | 1.06 | -1.00 | -0.11 | 0.65 | 0.89 | 0.76 |
|  | 10 | -1.05 | -0.18 | 0.57 | 0.87 | 0.75 |  |  |  |  |  |
| TDL-A 83Hz | 3 | -2.03 | -0.18 | 1.20 | 1.85 | 1.38 | -1.26 | -0.09 | 0.90 | 1.17 | 0.99 |
|  | 5 | -1.48 | -0.13 | 0.96 | 1.35 | 1.09 | -0.94 | -0.08 | 0.67 | 0.86 | 0.75 |
|  | 10 | -1.01 | -0.14 | 0.62 | 0.87 | 0.75 |  |  |  |  |  |
| TDL-C 83Hz | 3 | -2.14 | -0.24 | 1.13 | 1.90 | 1.37 | -1.38 | -0.16 | 0.83 | 1.22 | 0.99 |
|  | 5 | -1.50 | -0.20 | 0.86 | 1.30 | 1.06 | -1.05 | -0.15 | 0.60 | 0.90 | 0.76 |
|  | 10 | -1.07 | -0.21 | 0.56 | 0.86 | 0.77 |  |  |  |  |  |
| SCS= 120KHz |  |  |  |  |  |  |  |  |  |  |  |
| Propagation condition | Samples | 48RBs (Density = 1) |  |  |  |  | 96RBs (Density = 1) |  |  |  |  |
|  |  | 5% | 50% | 95% | 50% - 5% | 95% -50% | 5% | 50% | 95% | 50% - 5% | 95% -50% |
| TDL-A 83Hz | 3 | -2.52 | -0.50 | 0.91 | 2.02 | 1.41 | -1.81 | -0.43 | 0.60 | 1.38 | 1.03 |
|  | 5 | -1.91 | -0.49 | 0.64 | 1.42 | 1.12 | -1.40 | -0.41 | 0.36 | 0.99 | 0.78 |
|  | 10 | -1.40 | -0.47 | 0.33 | 0.94 | 0.80 |  |  |  |  |  |
| TDL-C 83Hz | 3 | -2.27 | -0.36 | 1.00 | 1.91 | 1.36 | -1.49 | -0.27 | 0.75 | 1.22 | 1.01 |
|  | 5 | -1.65 | -0.34 | 0.74 | 1.31 | 1.08 | -1.18 | -0.26 | 0.52 | 0.92 | 0.78 |
|  | 10 | -1.24 | -0.34 | 0.45 | 0.90 | 0.79 |  |  |  |  |  |

Summary

The simulation results of CSI-RSRP measurement performance with 24RBs measurement bandwidth (Density=1/3) are provided in Table 1. The simulation results of CSI-RSRP measurement performance with 48 RBs/96RBs measurement bandwidth (Density=1) are provided in Table 2.

It can be observed that the CSI-RSRP measurement accuracy is impacted by measurement bandwidth, CSI-RS resource density and the number of measurement samples.

Observation 1: When measurement bandwidth is 24RBs, compared with density=3, there is 1~2dB performance loss of CSI-RS based measurements with density=1.

Observation 2: There would bring 1~2dB performance gain for CSI-RS based measurements by increasing measurement bandwidth from 24RBs to 48RBs.

Observation 3: When CSI-RS is configured with density=1, the CSI-RSRP measurement accuracy by using 10 measurement samples are within 2dB.

Observation 4: When CSI-RS is configured with density=3, the CSI-RSRP measurements accuracies by using 3 measurement samples are within 2dB for both FR1 and FR2.

The CSI-RS based L3 measurement requirements could be derived from the existing SSB based L3 measurement requirements. The current accuracy requirements for SSB based measurements and the side condition can be reused for CSI-RS based measurement. In order to achieve the same measurement accuracy performance, more measurement samples shall be used for filtering when a sparser CSI-RS resource is configured or when a smaller measurement bandwidth is used.

Proposal 1: The CSI-RS based measurement period for L3 mobility can be determined by both CSI-RS resource density and measurement bandwidth, which can be expressed in Table 3.

Table 3: TCSI-RS_measurement_period for different configurations

| Configuration | Measurement bandwidth [RB] | CSI-RS resource Density | TCSI-RS_measurement_Period |
| --- | --- | --- | --- |
| 0 | ≥24 | 1 | [10]*CSI-RS period |
| 1 | ≥96 | 1 | [5]*CSI-RS period |
| 2 | ≥24 | 3 | [5]*CSI-RS period |

Conclusions

This contribution provides the simulation results and the analysis on the CSI-RS based measurements requirements in NR. The followings are provided:

Observation 1: When measurement bandwidth is 24RBs, compared with density=3, there is 1~2dB performance loss of CSI-RS based measurements with density=1.

Observation 2: There would bring 1~2dB performance gain for CSI-RS based measurements by increasing measurement bandwidth from 24RBs to 48RBs.

Observation 3: When CSI-RS is configured with density=1, the CSI-RSRP measurement accuracy by using 10 measurement samples are within 2dB.

Observation 4: When CSI-RS is configured with density=3, the CSI-RSRP measurements accuracies by using 3 measurement samples are within 2dB for both FR1 and FR2.

Proposal 1: The CSI-RS based measurement period for L3 mobility can be determined by both CSI-RS resource density and measurement bandwidth, which can be expressed in Table 3.

Table 3: TCSI-RS_measurement_period for different configurations

| Configuration | Measurement bandwidth [RB] | CSI-RS resource Density | TCSI-RS_measurement_Period |
| --- | --- | --- | --- |
| 0 | ≥24 | 1 | 10 measurement samples |
| 1 | ≥96 | 1 | 5 measurement samples |
| 2 | ≥24 | 3 | 5 measurement samples |

Reference

- R4-1909994, “WF on NR V2X RRM requirement scope”, LG Electronics