3GPP TSG-RAN WG4 Meeting #92bis	R4-1912058

Chongqing, China, 14 - 18 October, 2019

Title: 					High speed requirements for NR RRM

Source: 				Ericsson

Agenda item:	8.17.1

Document for:	Discussion

# Introduction

A work item for NR high speed train scenarios, including RRM requirements was agreed. Regarding the speed to be supported, the WID says:

- The feasibility of supporting speeds of up to a maximum of 500km/h will be investigated. The actual maximum supported velocity at 3.6GHz will be decided in this WI.

In this contribution we evaluate RRM requirements for support of both 350km/h and 500km/h. In the event that 500km/h is not feasible then 350km/h requirements are of interest as a possible fallback solution, noting that for release 16 the complete package of requirements needs to be understood before deciding on the actual supported velocity. The scope of the WI is for NR SA single carrier scenarios, so carrier aggregation (Scell enhanced requirements) are not considered.

# Discussion

We begin by reviewing the requirements introduced for LTE high speed operation at 350km/h and 500 hm/h. Only intrafrequency requirements have been enhanced, and a summary is provided in table 1.

| Intra requirement | LTE non high speed | LTE release 14 Pcell up to 350hm/h | LTE release 16 Pcell up to 500km/h | LTE release 16 Scell requirements |
| --- | --- | --- | --- | --- |
| Idle mode Tdetect | 36 DRX cycles (0.32s) 28 DRX cycles (0.64s) 25 DRX cycles (1.28s) 23 DRX cycles (2.56s) | 10 DRX cycles except 2.56s DRX cycle which was not enhanced | 8 DRX cycles (0.32s) 8 DRX cycles (0.64s) 7 DRX cycles (1.28s) 23 DRX cycles (2.56s) (not enhanced) | N/A |
| Idle mode Tervaluate | 16 DRX cycles (0.32s) 8 DRX cycles (0.64s) 5 DRX cycles (1.28s) 3 DRX cycles (2.56s) | 3 DRX cycles | 3 DRX cycles | N/A |
| Measurement period RRC connected non DRX | 200ms | 200ms | 200mss | N/A |
| Cell identification RRC connected non DRX | 800ms | 800ms | 800ms | N/A |
| Measurement period RRC connected DRX |  |  |  |  |
| Cell identification RRC connected DRX | 0.04<DRX-cycle≤0.08; 40. 0.128; 25. 0.128<DRX-cycle≤2.56; 20. | 0.04<DRX-cycle≤0.08; 15. 0.08<DRX-cycle≤1.28; 10. 1.28<DRX-cycle≤2.56; 20. | 0.04<DRX-cycle≤0.08; 15. 0.08<DRX-cycle<1.28; 10. DRX=1.28; 8. 1.28<DRX-cycle Note3≤2.56; 20. | 0.04<DRX-cycle≤0.08; 15. 0.08<DRX-cycle≤1.28; 10. 1.28<DRX-cycle≤2.56; 20. |
| Measurement period RRC connected deactivated Scell | 5 measCycleSCell | N/A | N/A | 3 measCycleSCell |
| Cell identification RRC connected deactivated SCell | 20 measCycleSCell | N/A | N/A | 10 measCycleSCell |

Table 1 : Legacy and high speed requirements in LTE

Generally speaking, very similar procedures are used for reselection, handover and Scell management between LTE and NR, and therefore the system needs for measurement delays are very similar between the RATs. The principal difference between NR and LTE from a measurement perspective is that in LTE, PSS/SSS are transmitted every 5ms and CRS have very high availability. In NR, the UE may only assume that SSB is transmitted in the intra-frequency SMTC, which has a configurable periodicity between 5ms and 160ms, with 20ms being assumed for initial search. Hence, it is not reasonable to expect that high speed train operation at 350km/h or 500km/h could work with the longest SMTC periods, and we have a companion contribution on high speed requirements applicability. The other difference of note is that intra-frequency measurements may be performed with or without gaps depending on whether the active reception bandwidth includes the SSB for measurement.

| Intra requirement | NR release 15 non high speed |
| --- | --- |
| Idle mode Tevaluate | 16M2 samples (0.32s) 8 M2 samples (0.64s) 5 M2 samples (1.28s) 3 M2 samples (2.56s) |
| Idle mode Tdetect | 36 M2 samples (0.32s) 28 M2 samples (0.64s) 25 M2 samples (1.28s) 23 M2 samples (2.56s) |
| Measurement period RRC connected non DRX | 5 samples |
| PSS/SSS sync RRC connected non DRX | 5 samples |
| Time index detection non DRX | 3 samples |
| Measurement period RRC connected DRX | 5 samples |
| PSS/SSS sync RRC connected DRX | 1.5*5 samples (≤320ms) 5 samples (>320ms) |
| Time index detection non DRX | 1.5*3 samples (≤320ms) 3 samples (>320ms) |
| Measurement period RRC connected deactivated SCell | 1.5*5 samples (≤320ms) 5 samples (>320ms) |
| PSS/SSS sync RRC connected deactivated Scell | 5 measCycleSCell (Non DRX) 5 measCycleSCell (≤320ms) 5 measCycleSCell (>320ms) 20 measCycleSCell |

Table 2 : Release 15 NR requirements

Since FR1 NR trackside deployments and LTE trackside deployments can be expected to be very similar or identical, and also because the frequency bands being used are comparable, it follows that the system needs for RRM delays would also be very similar or identical between FR1 NR and LTE. Hence, it is possible to make a direct comparison between table 1 and table 2 to identify suitable enhancements for NR operation at either 350km/h or 500km/h target speed. To ensure that a like-for-like comparison is made, we note that in NR, the total time to detect an unknown cell is the sum of PSS/SSS sync, possible time index reading and measurement period whereas for LTE identification includes both detection and measurement.

Idle Mode

Comparing table 1 and table 2, the non-high speed LTE idle mode requirements are identical to the NR non-high speed requirements in cases that M2=1 (SMTC≤20ms). For other cases, the NR performance is relaxed by a factor of 1.5 compared with LTE non-high speed.

It follows that the release 15 NR idle mode performance is not sufficient to support high speed operation. We propose that the RRM requirements are enhanced with the following attributes

Proposal 1 : For high speed scenarios, idle mode intra frequency Tdetect and Tevaluate shall be enhanced except for 2.56s DRX cycle

Proposal 2 : M2 relaxation in idle mode shall not be considered for high speed scenarios

Proposal 3 : The Idle enhanced requirements would be Tevaluate=3 DRX cycles and Tdetect=10DRX cycles for speed up to 350km/h or Tdetect=8 DRX cycles (0.32s), Tdetect=8 DRX cycles (0.64s), Tdetect=7 DRX cycles (1.28s) for speeds up to 500km/h

RRC connected Mode

Again, comparing table 1 and table 2, the measurement period in DRX for NR should be enhanced to 3 samples, supporting both up to 350km/h and up to 500km/h. Similarly to idle mode, no 1.5x relaxation factor should be allowed for with DRX cycles ≤320ms

Proposal 4: The RRC connected intra frequency DRX measurement period for high speed scenarios in NR is enhanced to 3 samples without any 1.5x relaxation factor

If proposal 4 is done, we can see that the overall cell identification delay becomes 8 DRX cycles without time index reading, or 11 DRX cycles with time index reading. Comparing with the enhancements for LTE, our view is that this level is on a par with the LTE enhanced requirement, except for the 1.5x relaxation factor, so we propose

Proposal 5 : The 1.5x relaxation factor is removed from RRC connected pss/sss sync and time index reading requirements for high speed scenarios

A further justification for proposal 5 is that in release 15 the NR requirements for PSS/SSS sync and time index reading are the outcome of a link level simulation campaign, and it may be difficult to implement enhanced DRX requirements without making more than 1 measurement per DRX cycle. In this case, it would be better that the NW just configures a shorter DRX cycle to get the necessary performance, which also improves the schedulablity of the UE.

# Conclusions

In this paper we evaluate NR high speed requirements for speeds of up to 350km/h or speeds of up to 500km/h. Based on the evaluation we propose

Proposal 1 : For high speed scenarios, idle mode intra frequency Tdetect and Tevaluate shall be enhanced except for 2.56s DRX cycle

Proposal 2 : M2 relaxation in idle mode shall not be considered for high speed scenarios

Proposal 3 : The Idle enhanced requirements would be Tevaluate=3 DRX cycles and Tdetect=10DRX cycles for speed up to 350km/h or Tdetect=8 DRX cycles (0.32s), Tdetect=8 DRX cycles (0.64s), Tdetect=7 DRX cycles (1.28s) for speeds up to 500km/h

Proposal 4: The RRC connected intra frequency DRX measurement period for high speed scenarios in NR is enhanced to 3 samples without any 1.5x relaxation factor

Proposal 5 : The 1.5x relaxation factor is removed from RRC connected pss/sss sync and time index reading requirements for high speed scenarios

Scell requirements are not considered as the scope of the WI is for SA single carrier NR.

# References

- RP-191512, “New WID on NR support for high speed train scenario”, CMCC