3GPP TSG-RAN WG4 Meeting #92bis	R4-1912067

# Chongqing, China,  14 October, 2019 – 18 October, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | Draft | rev | - | Current version: | 16.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | <Requirements for conditional HO> |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_feMob-Core |  |  |  |  |  | Date: |  |  | 2019/4/10 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Addition of new feature, conditional HO in LTE further mobility enhancements |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add two new requirements; a handover delay/interruption requirement which applies once the HO condition has been satisfied and covers HO execution, and a handover condition evaluation requirement which is similar to event triggered reporting requirements except that the trigger is used internally to start HO execution. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No requirements for conditional HO |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Conditional PSCell addition/change requirements are also needed for the LTE dual connectivity case |  |  |  |  |  |  |  |  |

Change 1

### 5.1.2	Requirements for conditional handover

#### 5.1.2A.1	E-UTRAN – E-UTRAN

The requirements in this clause are applicable to both intra-frequency and inter-frequency conditional handovers. The requirements are applicable to FDD-FDD, FDD-TDD, TDD-FDD and TDD-TDD handovers.

##### 5.1.2A.1.1	Handover delay

The RRC UE processing delay is specified as [TBD]ms.

When the UE evaluates a condition implying handover the UE shall be ready to start the transmission of the new uplink PRACH channel within Dhandover seconds from the evaluation of the condition.

Where:

Dhandover equals the maximum UE processing delay plus the interruption time stated in clause  5.1.2A.1.2.

##### 5.1.2A.1.2	Interruption time

The interruption time is the time between evaluation by the UE that a preconfigured handover condition has been satisfied and the time the UE starts transmission of the new PRACH, excluding the 10ms RRC processing delay.

When intra-frequency or inter-frequency handover is commanded, the interruption time shall be less than Tinterrupt

When intra-frequency or inter-frequency handover is commanded, the interruption time shall be less than Tinterrupt

Tinterrupt = TIU + 20 ms

Where:

TIU is the interruption uncertainty in acquiring the first available PRACH occasion in the new cell. TIU can be up to 30 ms.

NOTE: The actual value of TIU shall depend upon the PRACH configuration used in the target cell.

Change 2

8.1.2.2.1.2.1.3A	Conditional handover evaluation

Reported RSRP, RSRQ, and RS-SINR measurements contained in conditional handover evaluation shall meet the requirements in sections 9.1.2.1, 9.1.2.2, 9.1.5.1, and 9.1.17.2.1, respectively.

The UE shall not trigger any handover, as long as no reporting criteria are fulfilled.

The conditional handover evaluation delay is defined as the time between an event that will trigger a conditional handover and and the point when the UE starts to execute the handover according to the requirementsin section 5.1.2A..

The conditional handover evaluation delay, measured without L3 filtering shall be less than T identify_intra defined in Clause 8.1.2.2.1.2. When L3 filtering is used or IDC autonomous denial is configured or the UE is performing reception and/or transmission for ProSe Direct Discovery and/or ProSe Direct Communication, or the UE is configured to perform SRS carrier based switching, an additional delay can be expected.

If a cell which has been detectable at least for the time period Tidentify_intra defined in clause 8.1.2.2.1.2 becomes undetectable for a period ≤ 5 seconds and then the cell becomes detectable again and triggers an event, the conditional handover evaluation delay shall be less than Tmeasure_intra provided the timing to that cell has not changed more than  50 Ts and the L3 filter has not been used. When L3 filtering is used or IDC autonomous denial is configured or the UE is performing reception and/or transmission for ProSe Direct Discovery and/or ProSe Direct Communication, or the UE is configured to perform SRS carrier based switching, an additional delay can be expected.

Change 3

8.1.2.2.2.2.1.3A	Conditional handover evaluation

Reported RSRP, RSRQ, and RS-SINR measurements contained in conditional handover evaluation shall meet the requirements in sections 9.1.2.1, 9.1.2.2, 9.1.5.1, and 9.1.17.2.1, respectively.

The UE shall not trigger any handover, as long as no reporting criteria are fulfilled.

The conditional handover evaluation delay is defined as the time between an event that will trigger a conditional handover and and the point when the UE starts to execute the handover according to the requirementsin section 5.1.2A..

The conditional handover evaluation delay, measured without L3 filtering shall be less than T identify_intra defined in Clause 8.1.2.2.2.2. When L3 filtering is used or IDC autonomous denial is configured or the UE is performing reception and/or transmission for ProSe Direct Discovery and/or ProSe Direct Communication, or the UE is configured to perform SRS carrier based switching, an additional delay can be expected.

If a cell which has been detectable at least for the time period Tidentify_intra defined in clause 8.1.2.2.2.2 becomes undetectable for a period ≤ 5 seconds and then the cell becomes detectable again and triggers an event, the conditional handover evaluation delay shall be less than Tmeasure_intra provided the timing to that cell has not changed more than  50 Ts and the L3 filter has not been used. When L3 filtering is used or IDC autonomous denial is configured or the UE is performing reception and/or transmission for ProSe Direct Discovery and/or ProSe Direct Communication, or the UE is configured to perform SRS carrier based switching, an additional delay can be expected.