3GPP TSG-RAN WG4 #92bis                                                                      		R4-1912098
Chongqing, China, 14 – 18 October 2019

Agenda item: 	10.2.1

Source: 	Keysight Technologies

Title: 	Reference Measurement Channels for NR FR1 MIMO OTA

Document for: 	Approval

# Introduction

This contribution is introducing typical gNodeB emulator settings for FR1 as suggested as next action in last meeting’s WF.

# Discussion

In the WF [1] from the last RAN4#92 meeting, the following next action was captured

Typical gNodeB emulator setting for FR1 2x2 and 4x4 MIMO shall be studied next meeting

The starting point for gNodeB emulator settings for the MIMO OTA test cases should be the technical specification on demodulation requirements [2] and the RMCs defined there should be re-used as much as possible for MIMO OTA.

For FR1 2x2 MIMO, Clause 5.2.2 (2 RX requirements) of [2] should serve as reference. For FDD and TDD, three different RMCs are specified each, as tabulated in Table 1.

Table 1: RMCs specified for FDD and TDD and Rank 2

| RMC | Technology | Bandwidth (MHz) / Subcarrier spacing (kHz) | DL Modulation | Receiver Type |
| --- | --- | --- | --- | --- |
| R.PDSCH.1-3.1 FDD | FDD | 10 / 15 | 64QAM | Baseline Receiver |
| R.PDSCH.2-1.1 FDD | FDD | 20 / 30 | 64QAM | Baseline Receiver |
| R.PDSCH.1-2.2 FDD | FDD | 10 / 15 | 16QAM | Enhanced Receiver |
| R.PDSCH.2-3.1 TDD | TDD | 40 / 30 | 64QAM | Baseline Receiver |
| R.PDSCH.2-9.1 TDD | TDD | 20 / 30 | 64QAM | Baseline Receiver |
| R.PDSCH.2-2.2 TDD | TDD | 40 / 30 | 16QAM | Enhanced Receiver |

To address the best coverage of NR FR1 MIMO OTA testing, it is suggested to only focus on the baseline receiver, i.e., 64QAM which also corresponds to the DL modulation used for LTE MIMO OTA testing, and to select the lowest respective channel BW, i.e., 10MHz for FDD (R.PDSCH.1-3.1 FDD) and 20MHz for TDD (R.PDSCH.2-9.1 TDD).

Proposal 1: For NR FR1 2x2 MIMO Testing, focus on the following RMCs: R.PDSCH.1-3.1 for FDD and R.PDSCH.2-9.1 for TDD

For FR1 4x4 MIMO, Clause 5.2.3 (4 RX requirements) of [2] should serve as reference. For FDD and TDD, one RMC is specified each, as tabulated in Table 2.

Table 2: RMCs specified for FDD and TDD and Rank 4

| RMC | Technology | Bandwidth (MHz) / Subcarrier spacing (kHz) | DL Modulation | Receiver Type |
| --- | --- | --- | --- | --- |
| R.PDSCH.1-2.4 FDD | FDD | 10 / 15 | 16QAM | Baseline Receiver |
| R.PDSCH.2-2.4 TDD | TDD | 40 / 30 | 16QAM | Baseline Receiver |

Given the lack of options and the objective to re-use RMCs, it is proposed to re-use the RMCs from [2] for rank 4, i.e., R.PDSCH.1-2.4 for FDD and R.PDSCH.2-2.4 for TDD.

Proposal 2: For NR FR1 4x4 MIMO Testing, focus on the following RMCs: R.PDSCH.1-2.4 for FDD and R.PDSCH.2-2.4 for TDD

# Conclusion

The following observations and proposals were made in this contribution

Proposal 1: For NR FR1 2x2 MIMO Testing, focus on the following RMCs: R.PDSCH.1-3.1 for FDD and R.PDSCH.2-9.1 for TDD

Proposal 2: For NR FR1 4x4 MIMO Testing, focus on the following RMCs: R.PDSCH.1-2.4 for FDD and R.PDSCH.2-2.4 for TDD

# References

- R4-1910609, WF on NR MIMO OTA, CAICT, 3GPP TSG-RAN WG4 Meeting #92, August 2019

- TS 38.521-4: User Equipment (UE) conformance specification; Radio transmission and reception; Part 4: Performance requirements, V16.0.0 (2019-06)