3GPP TSG-RAN WG4 Meeting #92-Bis	R4-1912357

Chongqing, China, 14 – 18 October, 2019

Agenda item:	7.10.3.4

Source: 	Qualcomm Incorporated

Title: 	On RSRP measurements for MTC PUR

Document for:	Discussion

# Introduction

In RAN4#92 meeting, a Way Forward on Release 16 eMTC enhancements was agreed [1]. On RSRP measurements for TA validation of PUR, the following agreement was reached:

- Both in serving cell relaxed monitoring mode and in serving cell non-relaxed monitoring mode, if UE is configured with RSRP change for TA validation, the first and the second measurements used for validating the TA shall satisfy the following conditions

  - The first measurement (RSRP1) shall be performed within following time range: T1 – N ≤ T1’ ≤ T1 + N;

  - The second measurement (RSRP2) shall be performed within following time range: T2 – M ≤ T2’<T2;

  - Where T1’ is the time when RSRP1 becomes available, T1 is the time when TA is obtained, N is TBD, T2’ is the time when RSRP2 becomes available, T2 is the time when TA is validated, M TBD.

- Relaxation on serving cell monitoring is allowed regardless of TA validation mechanism.

In this paper, we discuss the values of N and M, i.e., how far or old the RSRP measurements should be with respect to reception of PUR configuration or PUR occasion, respectively.

# Discussion

The first measurement of RSRP (RSRP1) is intended to establish a reference measurement for RSRP change. If PUR configuration has RSRP change as a method for TA validation, the change with respect to RSRP1 is computed and compared with a threshold to validate the TA and subsequently allow transmission in a PUR occasion. Given the relatively low accuracy of RSRP in eMTC due to narrowband signal, it is important for UE to be able to do sufficient averaging of RSRP measurement to arrive at an RSRP1 value that accounts for both decent processing gain and fading channel conditions.

It is observed that while receiving PUR configuration, UE is in RRC_CONNECTED state and hence performing radio link monitoring (RLM) and therefore periodically evaluating the in-sync (IS) and out-of-sync (OOS) conditions. Evaluating IS/OOS is tantamount to making RSRP measurement. In IS state, UE is required to make an evaluation (i.e., measurement) at least once every Tevaluate_Qin_CatM1 in non-DRX mode and Tevaluate_Qin_DRX_CatM1 in DRX mode as defined in clause 7.19.2 of TS 36.133. In the worst-case scenario, the PUR configuration can be received just in the middle of two successive IS evaluations. Hence, it is proposed to define N as half of Tevaluate_Qin_CatM1 in non-DRX mode and Tevaluate_Qin_DRX_CatM1 in DRX mode. It is noted that by defining N as such, no additional measurement needs to be performed just for RSRP1 and UE simply takes advantage of RLM measurements it already needs to perform.

Observation 1. While receiving PUR configuration, UE is in RRC_CONNECTED state and hence performing radio link monitoring (RLM) and therefore periodically evaluating the in-sync (IS) and out-of-sync (OOS) conditions. Evaluating IS/OOS is tantamount to making RSRP measurement. In IS state, UE is required to make an evaluation (i.e., measurement) at least once every Tevaluate_Qin_CatM1 in non-DRX mode and Tevaluate_Qin_DRX_CatM1 in DRX mode as defined in clause 7.19.2 of TS 36.133.

Proposal 1.  For RSRP1, define N = 0.5* Tevaluate_Qin_CatM1 in non-DRX mode and N = 0.5* Tevaluate_Qin_DRX_CatM1 in DRX mode.

We now discuss the requirements for the second measurement RSRP2. While it is yet not ruled out whether PUR transmission can also occur in RRC_CONNECTED state, RAN1/2 have not discussed this topic so far. The most likely application of PUR transmission is in RRC_IDLE mode. Hence, after UE receives the PUR configuration, it is likely released into RRC_IDLE state. As agreed in RAN4#92, UE must meet the transmit timing accuracy requirements when it is supposed to transmit in an upcoming PUR occasion. A CR for eMTC PUR was approved [2].

In eMTC, the transmit timing accuracy requirements is specified in clause 7.24 of TS 36.133. The transmit timing error Te should not exceed 24Ts in CE mode A and 48Ts  in CE mode B. The question then becomes how long UE can remain asleep and not do any measurements and be still able to meet the transmit timing accuracy requirements. To answer this, some assumptions on UE XO drift rate in idle mode should be made. In our view, in idle mode and without running tracking loops, XO drift is significantly higher than 0.1-0.2 ppm. Drift rates as high as 10ppm is not unreasonable given the low-cost components that is expected to be used in MTC solutions. With 10ppm drift rate, M can be calculated as:

M= 48Ts30720Ts.10-5=156.2 ms

For CE mode A with 24Ts , M = 78.2ms. Hence, it is proposed M = 0.128 s for CE mode B and M = 0.064s for CE mode A. In our view, if a measurement was done more than M seconds ago, UE needs to perform a fresh measurement to sync with serving cell, estimate the pathloss, and be ready for PUR transmission if it has data to sent.

Observation 2. For second RSRP measurement, UE is most likely in RRC_IDLE state and must meet the transmit timing accuracy requirements. How old RSRP2 can be with respect to an upcoming PUR occasion depends on the UE XO drift rate in idle mode among other factors.

Proposal 2. For RSRP2, define M = 0.064s for CE mode A and M = 0.128s for CE mode B.

# Conclusions

Observation 1. While receiving PUR configuration, UE is in RRC_CONNECTED state and hence performing radio link monitoring (RLM) and therefore periodically evaluating the in-sync (IS) and out-of-sync (OOS) conditions. Evaluating IS/OOS is tantamount to making RSRP measurement. In IS state, UE is required to make an evaluation (i.e., measurement) at least once every Tevaluate_Qin_CatM1 in non-DRX mode and Tevaluate_Qin_DRX_CatM1 in DRX mode as defined in clause 7.19.2 of TS 36.133.

Proposal 1.  For RSRP1, define N = 0.5* Tevaluate_Qin_CatM1 in non-DRX mode and N = 0.5* Tevaluate_Qin_DRX_CatM1 in DRX mode.

Observation 2. For second RSRP measurement, UE is most likely in RRC_IDLE state and must meet the transmit timing accuracy requirements. How old RSRP2 can be with respect to an upcoming PUR occasion depends on the UE XO drift rate in idle mode among other factors.

Proposal 2. For RSRP2, define M = 0.064s for CE mode A and M = 0.128s for CE mode B.

# References

[1] R4-1910107

[2] R4-1910559