3GPP TSG-RAN Working Group 4 (Radio) meeting #92bis	R4-1912382

# Chongqing, China, 14th– 18th Oct, 2019

Source:	LG Electronics

Title:	On channel raster, SSB raster and wide-band operation for NR-U

Agenda item:	8.1.1.2	Wideband operations (UE and BS)

Document for:	Approval and Discussion

# Introduction

RAN4 has been discussing on the channel raster, SSB raster, spectrum emissions and guard-bands needed for wide-band operation. This contribution addresses all those aspects for 5GHz NR-U band as these topics are related to one another.

# Discussion

## Channel raster

In order to support the use of single FFT for wide-band operation with all SCS the channel center frequencies need to be defined in a way that gap between the adjacent channels is integer multiple of 60kHz and channels needs to be defined with ~10MHZ raster.

Complete proposal is shown in table 1:

Table 1. Down sampled channel raster

| CHANNEL CENTER FERQUENCIES | NREF | Fc (MHz) | Delta to center | Delta N*60k | Delta NREF |
| --- | --- | --- | --- | --- | --- |
| 5160 | 744000 | 5160,000 | 0,0000 |  |  |
| 5170 | 744668 | 5170,020 | 0,0200 | 167,000 | 668 |
| 5180 | 745332 | 5179,980 | -0,0200 | 166,000 | 664 |
| 5190 | 746000 | 5190,000 | 0,0000 | 333,000 | 668 |
| 5200 | 746668 | 5200,020 | 0,0200 | 334,000 | 668 |
| 5210 | 747332 | 5209,980 | -0,0200 | 333,000 | 664 |
| 5220 | 748000 | 5220,000 | 0,0000 | 333,000 | 668 |
| 5230 | 748668 | 5230,020 | 0,0200 | 334,000 | 668 |
| 5240 | 749332 | 5239,980 | -0,0200 | 333,000 | 664 |
| 5250 | 750000 | 5250,000 | 0,0000 | 333,000 | 668 |
| 5260 | 750668 | 5260,020 | 0,0200 | 334,000 | 668 |
| 5270 | 751332 | 5269,980 | -0,0200 | 333,000 | 664 |
| 5280 | 752000 | 5280,000 | 0,0000 | 333,000 | 668 |
| 5290 | 752668 | 5290,020 | 0,0200 | 334,000 | 668 |
| 5300 | 753332 | 5299,980 | -0,0200 | 333,000 | 664 |
| 5310 | 754000 | 5310,000 | 0,0000 | 333,000 | 668 |
| 5320 | 754668 | 5320,020 | 0,0200 | 334,000 | 668 |
| 5330 | 755332 | 5329,980 | -0,0200 | 333,000 | 664 |
| 5340 | 756000 | 5340,000 | 0,0000 | 333,000 | 668 |
| 5480 | 765332 | 5479,980 | -0,0200 | 2333,000 | 9332 |
| 5490 | 766000 | 5490,000 | 0,0000 | 167,000 | 668 |
| 5500 | 766668 | 5500,020 | 0,0200 | 167,000 | 668 |
| 5510 | 767332 | 5509,980 | -0,0200 | 166,000 | 664 |
| 5520 | 768000 | 5520,000 | 0,0000 | 167,000 | 668 |
| 5530 | 768668 | 5530,020 | 0,0200 | 167,000 | 668 |
| 5540 | 769332 | 5539,980 | -0,0200 | 166,000 | 664 |
| 5550 | 770000 | 5550,000 | 0,0000 | 167,000 | 668 |
| 5560 | 770668 | 5560,020 | 0,0200 | 167,000 | 668 |
| 5570 | 771332 | 5569,980 | -0,0200 | 166,000 | 664 |
| 5580 | 772000 | 5580,000 | 0,0000 | 167,000 | 668 |
| 5590 | 772668 | 5590,020 | 0,0200 | 167,000 | 668 |
| 5600 | 773332 | 5599,980 | -0,0200 | 166,000 | 664 |
| 5610 | 774000 | 5610,000 | 0,0000 | 167,000 | 668 |
| 5620 | 774668 | 5620,020 | 0,0200 | 167,000 | 668 |
| 5630 | 775332 | 5629,980 | -0,0200 | 166,000 | 664 |
| 5640 | 776000 | 5640,000 | 0,0000 | 167,000 | 668 |
| 5650 | 776668 | 5650,020 | 0,0200 | 167,000 | 668 |
| 5660 | 777332 | 5659,980 | -0,0200 | 166,000 | 664 |
| 5670 | 778000 | 5670,000 | 0,0000 | 167,000 | 668 |
| 5680 | 778668 | 5680,020 | 0,0200 | 167,000 | 668 |
| 5690 | 779332 | 5689,980 | -0,0200 | 166,000 | 664 |
| 5700 | 780000 | 5700,000 | 0,0000 | 167,000 | 668 |
| 5710 | 780668 | 5710,020 | 0,0200 | 167,000 | 668 |
| 5720 | 781332 | 5719,980 | -0,0200 | 166,000 | 664 |
| 5745 | 783000 | 5745,000 | 0,0000 | 417,000 | 1668 |
| 5755 | 783668 | 5755,020 | 0,0200 | 167,000 | 668 |
| 5765 | 784332 | 5764,980 | -0,0200 | 166,000 | 664 |
| 5775 | 785000 | 5775,000 | 0,0000 | 167,000 | 668 |
| 5785 | 785668 | 5785,020 | 0,0200 | 167,000 | 668 |
| 5795 | 786332 | 5794,980 | -0,0200 | 166,000 | 664 |
| 5805 | 787000 | 5805,000 | 0,0000 | 167,000 | 668 |
| 5815 | 787668 | 5815,020 | 0,0200 | 167,000 | 668 |
| 5825 | 788332 | 5824,980 | -0,0200 | 166,000 | 664 |
| 5835 | 789000 | 5835,000 | 0,0000 | 167,000 | 668 |
| 5845 | 789668 | 5845,020 | 0,0200 | 167,000 | 668 |
| 5855 | 790332 | 5854,980 | -0,0200 | 166,000 | 664 |
| 5865 | 791000 | 5865,000 | 0,0000 | 167,000 | 668 |
| 5875 | 791668 | 5875,020 | 0,0200 | 167,000 | 668 |
| 5885 | 792332 | 5884,980 | -0,0200 | 166,000 | 664 |
| 5895 | 793000 | 5895,000 | 0,0000 | 167,000 | 668 |
| 5905 | 793668 | 5905,020 | 0,0200 | 167,000 | 668 |
| 5915 | 794332 | 5914,980 | -0,0200 | 166,000 | 664 |
|  |  | Max offset | 0,02 |  |  |
|  |  | Min offset | -0,02 |  |  |

## SSB raster

For each 20MHz sub-band defined above a location for SSB is selected in a way that there is always in minimum 0.5x 60kHz RB remaining on left side of the SSB so that one RB may be removed from full allocation if wider guard-band is needed to meet the protection requirements for adjacent systems. Note that in this analysis NRB,max of 25 was used for 60kHz SCS to align with ongoing discussion SU. For 30 and 15kHz SCS the offset in number of RBs is larger.

15kHz SCS: Min 5.33, Max 12.67 RBs

30kHz SCS: Min 1.67, Max 5.33 RBs

60kHz SCS: Min 0.58, Max 2.42 RBs

Complete proposal is shown in table below:

Table 2. Down sampled SSB raster

| CHANNEL CENTER FERQUENCIES | GSCN | Fc (MHz) | SSB LEFT EDGE | Delta to channel CF | Delta GSCN |
| --- | --- | --- | --- | --- | --- |
| 5160 | 8996 | 5155,68 | 5152,080 | -4,320 |  |
| 5180 | 9010 | 5175,84 | 5172,240 | -4,140 | 14 |
| 5200 | 9024 | 5196 | 5192,400 | -4,020 | 14 |
| 5220 | 9038 | 5216,16 | 5212,560 | -3,840 | 14 |
| 5240 | 9052 | 5236,32 | 5232,720 | -3,660 | 14 |
| 5260 | 9065 | 5255,04 | 5251,440 | -4,980 | 13 |
| 5280 | 9079 | 5275,2 | 5271,600 | -4,800 | 14 |
| 5300 | 9093 | 5295,36 | 5291,760 | -4,620 | 14 |
| 5320 | 9107 | 5315,52 | 5311,920 | -4,500 | 14 |
| 5340 | 9121 | 5335,68 | 5332,080 | -4,320 | 14 |
| 5480 | 9218 | 5475,36 | 5471,760 | -4,620 | 97 |
| 5500 | 9232 | 5495,52 | 5491,920 | -4,500 | 14 |
| 5520 | 9246 | 5515,68 | 5512,080 | -4,320 | 14 |
| 5540 | 9260 | 5535,84 | 5532,240 | -4,140 | 14 |
| 5560 | 9274 | 5556 | 5552,400 | -4,020 | 14 |
| 5580 | 9288 | 5576,16 | 5572,560 | -3,840 | 14 |
| 5600 | 9302 | 5596,32 | 5592,720 | -3,660 | 14 |
| 5620 | 9315 | 5615,04 | 5611,440 | -4,980 | 13 |
| 5640 | 9329 | 5635,2 | 5631,600 | -4,800 | 14 |
| 5660 | 9343 | 5655,36 | 5651,760 | -4,620 | 14 |
| 5680 | 9357 | 5675,52 | 5671,920 | -4,500 | 14 |
| 5700 | 9371 | 5695,68 | 5692,080 | -4,320 | 14 |
| 5720 | 9385 | 5715,84 | 5712,240 | -4,140 | 14 |
| 5745 | 9402 | 5740,32 | 5736,720 | -4,680 | 17 |
| 5765 | 9416 | 5760,48 | 5756,880 | -4,500 | 14 |
| 5785 | 9430 | 5780,64 | 5777,040 | -4,380 | 14 |
| 5805 | 9444 | 5800,8 | 5797,200 | -4,200 | 14 |
| 5825 | 9458 | 5820,96 | 5817,360 | -4,020 | 14 |
| 5845 | 9472 | 5841,12 | 5837,520 | -3,900 | 14 |
| 5865 | 9486 | 5861,28 | 5857,680 | -3,720 | 14 |
| 5885 | 9499 | 5880 | 5876,400 | -4,980 | 13 |
| 5905 | 9513 | 5900,16 | 5896,560 | -4,860 | 14 |
| 5915 | 9520 | 5910,24 | 5906,640 | -4,740 | 7 |

## Wide-band operation

In the following sections a way to operate wide-band carriers is presented following the principles listed below:

- In-carrier guard-bands are integer multiple of RBs.

- Cerrier center frequency offset from the mid-point of 20MHz sub-band is always as small as possible. 200kHz offset allowed in ETSI regulation [3].

- The width of Carrier-guard-bands and In-carrier-guard-bands is maximized and delta to minimum guard band specified 38.101 is calculated.

  - As it is not possible to locate the carriers into the mid-point of the 20MHz sub-band while following the principle 1. the guard-bands will become in some cases wider and in some cases narrower than the currently specified minimum guard band for NR FR1 [2].

For 15kHz SCS 106 RBs is allocated into each sub-band.

For 30kHz SCS 51 RBs is allocated into each sub-band.

For 60kHz SCS 25 RBs is allocated into each sub-band. 25 is one higher than the existing maximum RBs defined in 38.101 to reflect the ongoing discussion on SU for NRU.

Following observations can be made:

Observation #1: Maximum delta between the mid-point of 20MHz sub-band and center of the carrier is different for each SCS, as the delta is impacted by the width of the RB. Maximum delta for each SCS and bandwidth is shown in Table 3. The maximum delta also equals to violation of currently specified minimum guard bands for NR FR1.

Table 3. Maximum delta between the mid-point of 20MHz sub-band and center of the carrier

|  | 20MHz | 40MHz | 60MHz | 80MHz | 100MHz | Min GB |
| --- | --- | --- | --- | --- | --- | --- |
| 15k SCS | 20kHz | 30kHz |  |  |  | 452.5kHz |
| 30k SCS | 20kHz | 120kHz | 180kHz | 140kHz | 180kHz | 805kHz |
| 60k SCS | 20kHz | 100kHz | 220kHz | 260kHz | 340kHz | 970kHz |

Observation #2: Maximum guard band violation is always less than half of the RB width and therefore the minimum guard band width can be achieved by operating the sub-band carriers with N-1 RBs if operation with narrower guard bands is not feasible.

Observation #3: Maximum delta between the mid-point of 20MHz sub-band and center of the carrier is more than 200kHz ETSI requirement with 60kHz SCS and 60, 80 and 100MHz wide-band-carriers. However, this is not considered to be critical as currently the NR-U will not be optimized for 60 kHz SCS. Implications of this may be further studied in the future.

When it comes to operating the wide-band carriers as required by the regulation [3] one needs to keep in mind that PSD is limited to +10dBm/1MHz and that occupied bandwidth shall be between 80% and 100% of the Nominal Channel Bandwidth. Operation with less than 80%, but with more than 2MHz wide carrier is only temporarily allowed. With these limitations the worst case PSD is significantly lower than in NR FR1 and therefore narrower guard bands may be feasible.

It should also be noted that in certain cases the total number of used RBs is +1 larger than currently define NRB,max in 38.101. This additional RB is used to improve the carrier center frequency offset as in principle 2.

ACLR and SEM performance for different wide-band-operation use cases is studied in 2.4 using the simulator model for the UE transmitter.

### 20MHz wide carrier

The channel raster proposal in 2.1 introduces -20, 0 or +20kHz offset between the mid-point of the 20MHz sub-band and channel center frequency. The channels that are impacted by the non-zero offset will also experience 20kHz narrower guard bands than the currently specified minimum guard band for NR FR1.

### 40MHz wide-band carrier

Proposal for 40MHz wide-band operation with 15, 30 and 60kHz SCS is shown below. Sum of in-carrier-guard-bands is integer multiple of RBs. Maximum guard band violation for 15, 30 and 60kHz SCS are 30, 120 and 100kHz respectively.

15kHz SCS: NRB,max= 216 (+1 vs. existing 38.101)

|  | Guard | Carrier | Guard | Guard | Carrier | Guard | TOTAL RB | TOTAL MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RBs | 2,61 | 106,00 | 2,50 | 2,50 | 106,00 | 2,61 | 222,22 | 40,0 |
| F-center -20k offset |  | -10 |  |  | -30 |  |  |  |
| F-center 0 offset |  | 10 |  |  | -10 |  |  |  |
| F-center +20k offset |  | 30 |  |  | 10 |  |  |  |
| Limit | 200 |  |  |  |  |  |  |  |
| Guards -20k offset | 442,5 |  | 462,5 | 422,5 |  | 482,5 |  |  |
| Guards 0 offset | 462,5 |  | 442,5 | 442,5 |  | 462,5 |  |  |
| Guards +20k offset | 482,5 |  | 422,5 | 462,5 |  | 442,5 |  |  |
| Limit | 452,5 |  |  |  |  |  |  |  |
| Max GB violation | 30 |  |  |  |  |  |  |  |

30kHz SCS: NRB,max= 106

|  | Guard | Carrier | Guard | Guard | Carrier | Guard | TOTAL RB | TOTAL MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RBs | 2,56 | 51,00 | 2,00 | 2,00 | 51,00 | 2,56 | 111,11 | 40,0 |
| F-center -20k offset |  | 80 |  |  | -120 |  |  |  |
| F-center 0 offset |  | 100 |  |  | -100 |  |  |  |
| F-center +20k offset |  | 120 |  |  | -80 |  |  |  |
| Limit | 200 |  |  |  |  |  |  |  |
| Guards -20k offset | 885 |  | 725 | 685 |  | 925 |  |  |
| Guards 0 offset | 905 |  | 705 | 705 |  | 905 |  |  |
| Guards +20k offset | 925 |  | 685 | 725 |  | 885 |  |  |
| Limit | 805 |  |  |  |  |  |  |  |
| Max GB violation | 120 |  |  |  |  |  |  |  |

60kHz SCS: NRB,max= 53 (+2 vs. existing 38.101)

|  | Guard | Carrier | Guard | Guard | Carrier | Guard | TOTAL RB | TOTAL MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RBs | 1,28 | 25,00 | 1,50 | 1,50 | 25,00 | 1,28 | 55,56 | 40,0 |
| F-center -20k offset |  | -100 |  |  | 60 |  |  |  |
| F-center 0 offset |  | -80 |  |  | 80 |  |  |  |
| F-center +20k offset |  | -60 |  |  | 100 |  |  |  |
| Limit | 200 |  |  |  |  |  |  |  |
| Guards -20k offset | 870 |  | 1070 | 1030 |  | 910 |  |  |
| Guards 0 offset | 890 |  | 1050 | 1050 |  | 890 |  |  |
| Guards +20k offset | 910 |  | 1030 | 1070 |  | 870 |  |  |
| Limit | 970 |  |  |  |  |  |  |  |
| Max GB violation | 100 |  |  |  |  |  |  |  |

### 60MHz wide-band carrier

Proposal for 60MHz wide-band operation with 30 and 60kHz SCS is shown below. Sum of in-carrier-guard-bands is integer multiple of RBs. Maximum guard band violation for 15, 30 and 60kHz SCS are 40, 180 and 220kHz respectively.

30kHz SCS: NRB,max= 163 (+1 vs. existing 38.101)

|  | Guard | Carrier | Guard | Guard | Carrier | Guard | Guard | Carrier | Guard | TOTAL RB | TOTAL MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RBs | 1,83 | 51,00 | 2,72 | 2,28 | 51,00 | 2,28 | 2,72 | 51,00 | 1,83 | 166,67 | 60,0 |
| F-center -20k offset |  | -180 |  |  | -20 |  |  | 140 |  |  |  |
| F-center 0 offset |  | -160 |  |  | 0 |  |  | 160 |  |  |  |
| F-center +20k offset |  | -140 |  |  | 20 |  |  | 180 |  |  |  |
| Limit | 200 |  |  |  |  |  |  |  |  |  |  |
| Guards -20k offset | 625 |  | 985 | 785 |  | 825 | 945 |  | 665 |  |  |
| Guards 0 offset | 645 |  | 965 | 805 |  | 805 | 965 |  | 645 |  |  |
| Guards +20k offset | 665 |  | 945 | 825 |  | 785 | 985 |  | 625 |  |  |
| Limit | 805 |  |  |  |  |  |  |  |  |  |  |
| Max GB violation | 180 |  |  |  |  |  |  |  |  |  |  |

60kHz SCS: NRB,max= 81 (+2 vs. existing 38.101)

|  | Guard | Carrier | Guard | Guard | Carrier | Guard | Guard | Carrier | Guard | TOTAL RB | TOTAL MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RBs | 1,17 | 25,00 | 1,11 | 1,89 | 25,00 | 1,89 | 1,11 | 25,00 | 1,17 | 83,33 | 60,0 |
| F-center -20k offset |  | -180 |  |  | -20 |  |  | 140 |  |  |  |
| F-center 0 offset |  | -160 |  |  | 0 |  |  | 160 |  |  |  |
| F-center +20k offset |  | -140 |  |  | 20 |  |  | 180 |  |  |  |
| Limit | 200 |  |  |  |  |  |  |  |  |  |  |
| Guards -20k offset | 790 |  | 790 | 1310 |  | 1350 | 750 |  | 830 |  |  |
| Guards 0 offset | 810 |  | 770 | 1330 |  | 1330 | 770 |  | 810 |  |  |
| Guards +20k offset | 830 |  | 750 | 1350 |  | 1310 | 790 |  | 790 |  |  |
| Limit | 970 |  |  |  |  |  |  |  |  |  |  |
| Max GB violation | 220 |  |  |  |  |  |  |  |  |  |  |

### 80MHz wide-band carrier

Proposal for 80MHz wide-band operation with 30 and 60kHz SCS is shown below. Sum of in-carrier-guard-bands is integer multiple of RBs. Maximum guard band violation for 15, 30 and 60kHz SCS are 50, 140 and 260kHz respectively.

30kHz SCS: NRB,max= 217

|  | Guard | Carrier | Guard | Guard | Carrier | Guard | Guard | Carrier | Guard | Guard | Carrier | Guard | TOTAL RB | TOTAL MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RBs | 2,61 | 51,00 | 1,94 | 2,06 | 51,00 | 2,50 | 2,50 | 51,00 | 2,06 | 1,94 | 51,00 | 2,61 | 222,22 | 80,0 |
| F-center -20k offset |  | 100 |  |  | -100 |  |  | 60 |  |  | -140 |  |  |  |
| F-center 0 offset |  | 120 |  |  | -80 |  |  | 80 |  |  | -120 |  |  |  |
| F-center +20k offset |  | 140 |  |  | -60 |  |  | 100 |  |  | -100 |  |  |  |
| Limit | 200 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Guards -20k offset | 905 |  | 705 | 705 |  | 905 | 865 |  | 760 | 680 |  | 945 |  |  |
| Guards 0 offset | 925 |  | 685 | 725 |  | 885 | 885 |  | 740 | 700 |  | 925 |  |  |
| Guards +20k offset | 945 |  | 665 | 745 |  | 865 | 905 |  | 720 | 720 |  | 905 |  |  |
| Limit | 805 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Max violation | 140 |  |  |  |  |  |  |  |  |  |  |  |  |  |

60kHz SCS: NRB,max= 109 (+2 vs. existing 38.101)

|  | Guard | Carrier | Guard | Guard | Carrier | Guard | Guard | Carrier | Guard | Guard | Carrier | Guard | TOTAL RB | TOTAL MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RBs | 1,06 | 25,00 | 1,72 | 1,28 | 25,00 | 1,50 | 1,50 | 25,00 | 1,28 | 1,72 | 25,00 | 1,06 | 111,11 | 80,0 |
| F-center -20k offset |  | -260 |  |  | -100 |  |  | 60 |  |  | 220 |  |  |  |
| F-center 0 offset |  | -240 |  |  | -80 |  |  | 80 |  |  | 240 |  |  |  |
| F-center +20k offset |  | -220 |  |  | -60 |  |  | 100 |  |  | 260 |  |  |  |
| Limit | 200 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Guards -20k offset | 710 |  | 1230 | 870 |  | 1070 | 1030 |  | 940 | 1220 |  | 750 |  |  |
| Guards 0 offset | 730 |  | 1210 | 890 |  | 1050 | 1050 |  | 920 | 1240 |  | 730 |  |  |
| Guards +20k offset | 750 |  | 1190 | 910 |  | 1030 | 1070 |  | 900 | 1260 |  | 710 |  |  |
| Limit | 970 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Max violation | 260 |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 100MHz wide-band carrier

Proposal for 100MHz wide-band operation with 30 and 60kHz SCS is shown below. Sum of in-carrier-guard-bands is integer multiple of RBs. Maximum guard band violation for 15, 30 and 60kHz SCS are 60, 180 and 340kHz respectively.

30kHz SCS: NRB, max= 273

|  | Guard | Carrier | Guard | Guard | Carrier | Guard | Guard | Carrier | Guard | Guard | Carrier | Guard | Guard | Carrier | Guard |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RBs | 2,39 | 51,00 | 2,17 | 1,83 | 51,00 | 2,72 | 2,28 | 51,00 | 2,28 | 2,72 | 51,00 | 1,83 | 2,17 | 51,00 | 2,39 |
| F-center -20k offset |  | 20 |  |  | -180 |  |  | -20 |  |  | 140 |  |  | -60 |  |
| F-center 0 offset |  | 40 |  |  | -160 |  |  | 0 |  |  | 160 |  |  | -40 |  |
| F-center +20k offset |  | 60 |  |  | -140 |  |  | 20 |  |  | 180 |  |  | -20 |  |
| Limit | 200 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Guards -20k offset | 825 |  | 785 | 625 |  | 985 | 785 |  | 840 | 960 |  | 680 | 760 |  | 865 |
| Guards 0 offset | 845 |  | 765 | 645 |  | 965 | 805 |  | 820 | 980 |  | 660 | 780 |  | 845 |
| Guards +20k offset | 865 |  | 745 | 665 |  | 945 | 825 |  | 800 | 1000 |  | 640 | 800 |  | 825 |
| Limit | 805 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Max violation | 180 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

60kHz SCS: NRB,max= 137 (+2 vs. existing 38.101)

|  | Guard | Carrier | Guard | Guard | Carrier | Guard | Guard | Carrier | Guard | Guard | Carrier | Guard | Guard | Carrier | Guard |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| RBs | 0,94 | 25,00 | 1,83 | 1,17 | 25,00 | 1,89 | 1,11 | 25,00 | 1,11 | 1,89 | 25,00 | 1,17 | 1,83 | 25,00 | 0,94 |
| F-center -20k offset |  | -340 |  |  | -180 |  |  | -20 |  |  | 140 |  |  | 300 |  |
| F-center 0 offset |  | -320 |  |  | -160 |  |  | 0 |  |  | 160 |  |  | 320 |  |
| F-center +20k offset |  | -300 |  |  | -140 |  |  | 20 |  |  | 180 |  |  | 340 |  |
| Limit | 200 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Guards -20k offset | 630 |  | 1310 | 790 |  | 1350 | 750 |  | 820 | 1340 |  | 860 | 1300 |  | 670 |
| Guards 0 offset | 650 |  | 1290 | 810 |  | 1330 | 770 |  | 800 | 1360 |  | 840 | 1320 |  | 650 |
| Guards +20k offset | 670 |  | 1270 | 830 |  | 1310 | 790 |  | 780 | 1380 |  | 820 | 1340 |  | 630 |
| Limit | 970 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Max violation | 340 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

## SEM and ACLR analysis with proposed alternative for wide-band operation

The SEM and ACLR performance of the NR-U signals is analysed using simulator model for UE transmitter that includes full TX chain and implements about 30dBc performance for both LO and IMAGE. The operating point of the transmitter is adjusted in a way that 15kHz SCS full band operation at +20dBm output power meets the Wifi mask and under these conditions ACLR is 27dB and EVM is 8%.

Figure 1. Transmitter performance with 20MHz QPSK CP-OFDMA signal

The results for studies done for 40, 60, 80 and 100MHz wide-band operation are shown in sections 2.1.1, 2.1.2, 2.1.3 and 2.1.4. The SEM mask shown is aligned with Alt. 8 from [4] and is so called modified ETSI/ IEEE 802.11ac/ax mask with -25 dB floor across failed LBT sub-bands.

Based on the simulated results following observation can be made.

Observation #4: The spectrum of the fully allocated 40, 60, 80 and 100MHz wide-band carriers follows the scaled emission mask with similar small margin as in 20MHz case and ACLR remains at 27dB. In-band emission mask aligned with Alt. 8 can be fulfilled with contiguous UL allocations except for the LO that needs an exception but power back-off is needed with non-contiguous allocations to meet SEM and ACLR..

### 40MHz wide-band operation

The SEM and ACLR performance for 40MHz wide-band-operation with 15kHz SCS is studied.

### 60MHz wide-band operation

The SEM and ACLR performance for 60MHz wide-band-operation with 30kHz SCS is studied.

Non-contiguous allocations are more problematic than contiguous ones as IM3 products of the transmitter fall outside of the own band and violate the SEM. Power back-off is needed to meet the ACLR and SEM in these cases.

### 80MHz wide-band operation

The SEM and ACLR performance for 80MHz wide-band-operation with 30kHz SCS is studied.

Results for non-contiguous allocations are not shown to limit the number of results shown, but they violate the SEM and ACLR the same way as seen with 60MHz operation. Power back-off is needed to meet the ACLR and SEM in these cases.

### 100MHz wide-band operation

The SEM and ACLR performance for 100MHz wide-band-operation with 30kHz SCS is studied.

# Conclusion

This contribution proposes

- Channel raster and SSB raster for NR-U at 5GHz band

- a way to operate wide-band carrier in a way that regulatory requirements are met and in-carrier guards are always an integer multiple of RBs.

and analyses the SEM and ACLR performance with simulator model aligned with the above mentioned proposals 1 and 2.

In addition, following observations area made:

Observation #1: Maximum delta between the mid-point of 20MHz sub-band and center of the carrier is different for each SCS, as the delta is impacted by the width of the RB. Maximum delta for each SCS and bandwidth is shown in Table 3. The maximum delta also equals to violation of currently specified minimum guard bands for NR FR1.

Observation #2: Maximum guard band violation is always less than half of the RB width and therefore the minimum guard band width can be achieved by operating the sub-band carriers with N-1 RBs if operation with narrower guard bands is not feasible.

Observation #3: Maximum delta between the mid-point of 20MHz sub-band and center of the carrier is more than 200kHz ETSI requirement with 60kHz SCS and 60, 80 and 100MHz wide-band-carriers. However, this is not considered to be critical as currently the NR-U will not be optimized for 60 kHz SCS. Implications of this may be further studied in the future.

Observation #4: The simulated spectrum of the fully allocated 40, 60, 80 and 100MHz wide-band carriers follows the scaled emission mask with similar small margin as in 20MHz case and ACLR remains at 27dB. In-band emission mask aligned with Alt. 8 can be fulfilled with contiguous UL allocations except for the LO that needs an exception but power back-off is needed with non-contiguous allocations to meet SEM and ACLR.

# References

[1]	R1-1909905 “LS on simultaneous transmission of PSFCH”, RAN WG1, LG Electronics, RAN1 #98, August 2019

[2]	3GPP TS 38.101, “User Equipment (UE) radio transmission and reception”

[3]	ETSI EN 301 893, “5 GHz RLAN; Harmonised Standard covering the essential requirements of article 3.2 of Directive 2014/53/EU”, version 2.1.1 (2017-05)

[4]	R4-1910387 “WF on Emission Requirements for NR-U”, Nokia, RAN4 #92 August 2019