3GPP TSG-RAN WG4 Meeting#92bis	R4-1912423

Chongqing, China, 14th Oct – 18th Oct, 2019

Source: 	Qualcomm Incorporated

Title: 	Views on Tests for High Speed Train Scenarios

Agenda item:	8.17.2.2

Document for:	Discussion

# Introduction

In WF [1], some of the test parameters were discussed and options for different parameters were provided. In this paper, we discuss our views on those parameters for high speed train scenarios.

# Maximum Doppler for HST-SFN

In the last meeting, WF [1] listed a lot of options for maximum Doppler spread values. We provide some more analysis on this below.

- For TDD 30kHz

  - For HST-SFN, further study the maximum Doppler frequency:

    - Target speed of 500km/h

      - Option 1: 1667Hz

      - Option 2: 1500Hz

    - Target speed of 350km/h

      - 1167Hz

- For FDD 15kHz

  - HST-SFN

    - Target speed of 500km/h

      - Option 1: 1667Hz

      - Option 2: 1250Hz

      - Option 3: 875Hz

      - Option 4: 972Hz

      - Option 5: 780Hz

    - Target speed of 350km/h

      - Option 1: 875Hz

      - Option 2: 780Hz

Maximum Doppler spread between two taps

As only 2 out of 4 taps would be more prominent, Doppler spread when only considering only two taps can be given as below at a distance x from one RRH:

Doppler Spread between two taps = fD,maxxx2+Dmin2+Ds-xDs-x2+Dmin2 ,				(1)

where fD,max = CF*v/c, with CF being carrier frequency, v as speed and c as speed of light. If we consider 4 taps, Doppler spread can only get larger than the above. The maximum value of above Doppler spread will be at x = Ds/2 and it is given as below:

Maximum Doppler Spread between two taps = fD,maxDsDs24+Dmin2 	(2)

If we assume Ds = 1000m and Dmin = 50m, then maximum Doppler spread between two taps is 1.99*fD,max.

FTL Limitations:

If FTL loop follows the baseline assumption of only using TRS for estimating frequency error, it’s pull-in range will be 1750*2μ since there is 4-symbol spacing between TRS symbols. Here, μ denotes the SCS index (0 for 15kHz, 1 for 30kHz SCS).

In LTE, we assumed that FTL can only correct frequency error up to the accuracy of 0.1ppm of carrier frequency [2, 3]. So, we can make the same assumption for NR. On top of it, we need to leave some margin for RF impairments.

Evaluation of Maximum Doppler Options in WF [1] for 15kHz SCS:

For 15kHz SCS, 500km/h speed, maximum Doppler spread between two taps for Option 1, 2, and 4 are 3317, 2487.5, 1934 Hz, while FTL pull-in range is only 1750Hz. Therefore, options 1, 2, and 4 are not feasible to achieve.

For 500km/h Option 3, maximum Doppler spread between two taps is 1741Hz, and carrier frequency to achieve 875Hz of Doppler at 500km/h speed is 1890 MHz. So, FTL can only correct up to the accuracy of 189Hz frequency error, so FTL pull-in range for this carrier frequency will be reduced to 1561Hz, which is smaller than 1741 Hz. So, Option 3 is also not feasible.

For 500km/h Option 5, maximum Doppler spread between two taps is 1560Hz, and carrier frequency to achieve 780Hz of Doppler at 500km/h speed is 1685 MHz. So, FTL can only correct up to the accuracy of 168.5Hz frequency error, so FTL pull-in range for this carrier frequency will be reduced to 1581.5Hz, which is very close to 1560Hz and doesn’t leave much margin for RF impairments. So, Option 5 is also not feasible.

Similar analysis can be done for 350km/h options to show that both the options are not feasible.

Observation 1: None of the options for maximum Doppler in WF [1] are feasible for FDD 15kHz SCS.

Therefore, based on above constraints, we propose the following.

Proposal 1: Define FDD 15kHz SCS test cases for high speed train with maximum Doppler shift of 712Hz for both 350km/h and 500km/h.

Evaluation of Maximum Doppler Options in WF [1] for 30kHz SCS:

For 30kHz SCS, FTL pull-in range is 1750*2 = 3500Hz. We can do the similar analysis as for 15kHz SCS for all the options.

Option 1 has maximum Doppler spread between two taps as 1667*1.99 = 3317Hz, and carrier frequency to achieve 1667Hz of Doppler at 500km/h speed is 3600 MHz. So, FTL can only correct up to the accuracy of 360Hz frequency error, so FTL pull-in range for this carrier frequency will be reduced to 3140Hz, which is smaller than 3317Hz. So, Option 1 is not feasible.

For Option 2, maximum Doppler spread between two taps is 2985Hz, and carrier frequency to achieve 1500Hz of Doppler at 500km/h speed is 3240 MHz. So, FTL can only correct up to the accuracy of 324Hz frequency error, so FTL pull-in range for this carrier frequency will be reduced to 3176Hz, which provides enough margin for RF impairments. So, Option 2 is feasible.

Proposal 2: Define TDD 30kHz SCS test cases for high speed train with maximum Doppler shift of 1500Hz for 500km/h.

# HST-SFN Model

In WF [1], following options were agreed for HST-SFN model parameters.

- For HST-SFN, reuse LTE HST-SFN bi-directional channel model, and number of taps can be further discussed.

  - Option 1: Ds=1000m, Dmin=50m

  - Option 2: Ds=700m, Dmin=150m

  - Option 3: Ds= 300m, Dmin=2m

Based on analysis in section 2, maximum Doppler spread between two taps for Ds = 1000m and Dmin = 50m is 1.99*fD,max. If we assume Ds = 700m and Dmin = 150m, maximum Doppler spread between two taps reduces to 1.84*fD,max. Therefore, Option 2 for HST-SFN in WF [1] can sustain same speed at higher carrier frequencies compared to Option 1, if everything else is the same. Option 3 has very low spacing between two RRH, which does not seem very practical. Therefore, we propose the following.

Proposal 3: Use Ds = 700m, Dmin = 150m for HST-SFN channel model.

# Number of DMRS symbols

In WF [1], there were two options for number of DMRS symbols: 1+2 and 1+3. As 1+3 DMRS symbols is an optional UE feature, we propose the following.

Proposal 4: Use 1+2 DMRS symbols for defining tests for high speed train scenarios.

# Conclusions

This paper proposes parameters related to test cases for high speed train scenarios. Following has been observed and proposed.

Observation 1: None of the options for maximum Doppler in WF [1] are feasible for FDD 15kHz SCS.

Proposal 1: Define FDD 15kHz SCS test cases for high speed train with maximum Doppler shift of 712Hz for both 350km/h and 500km/h.

Proposal 2: Define TDD 30kHz SCS test cases for high speed train with maximum Doppler shift of 1500Hz for 500km/h.

Proposal 3: Use Ds = 700m, Dmin = 150m for HST-SFN channel model.

Proposal 4: Use 1+2 DMRS symbols for defining tests for high speed train scenarios.

# References

[1] R4-1910050, “WF on UE demodulation for NR HST”, 3GPP TSG RAN WG4 Meeting #92, Ljubljana, Slovenia, Aug 26-30, 2019.

[2] R4-1906951, “On HST UE Demod Test”, 3GPP TSG RAN WG4 Meeting #91, Reno, USA, May 13-17, 2019.

[3] R4-1907745, “Way forward on UE demodulation enhancement for HST”, 3GPP TSG RAN WG4 Meeting #91, Reno, USA, May 13-17, 2019.