3GPP TSG-RAN WG4 Meeting #92bis	R4-1912447

Chongqing, China, 14th– 18th October 2019

Agenda item:	7.13.4.1	PUSCH demodulation requirements

[LTE_high_speed_enh2-Perf]

Source: 		NTT DOCOMO, INC.

Title: 	Views on PUSCH and PRACH for LTE HST enhancement

Document for:	Approval

1.	Introduction

In RAN4 #92, WF on BS demodulation for high speed in Rel.16 was approved [1]. In this contribution, we discuss on open issues for PUSCH performance requirements under enhanced HST conditions and provide preferences.

2.	Discussion

2.1	PUSCH

2.1.1	HST scenario

In RAN4 #92, it was agreed to introduce both open space and tunnel scenarios as enhanced HST scenarios, and the following table was implemented in TS.36.104.  The parameters of Ds, Dmin and v have been already agreed, but fd has not yet been agreed.

Table B.3-2: Parameters for high speed train conditions for UE velocity up to 500km/h

| Parameter | Value |  |
| --- | --- | --- |
|  | Scenario 1-LTE500 | Scenario 3-LTE500 |
|  | 1000 m | 300 m |
|  | 50 m | 2 m |
|  | 500 km/h | 500 km/h |
|  | TBD | TBD |

According to WF [1], candidate values for the maximum Doppler shift are as follows:

1944 Hz, 1894 Hz, 1844 Hz, 1794 Hz, 1744 Hz, 1694 Hz.

Table 1 shows UE velocity based on Band 1 (2.1GHz) and calculated from candidate maximum Doppler shift assumptions. To support band 1 which is same assumption as Rel.8 HST, the 1944 Hz Doppler shift shall be used.

Table 1: UE velocity corresponding to maximum Doppler shift at Band 1

| Maximum Doppler Shift | UE velocity |
| --- | --- |
|  | @Band 1 (2.1GHz) |
| 1944Hz | 500km/h |
| 1894Hz | 486km/h |
| 1844Hz | 473km/h |
| 1794Hz | 460km/h |
| 1774Hz | 455km/h |
| 1694Hz | 435km/h |

According simulation results provided by different companies in the last meeting [2-4], it is majority view that the 1944Hz maximum Doppler shift is feasible in both open space and tunnel scenarios. Therefore, we can conclude to adopt 1944Hz as the maximum Doppler shift for enhanced HST BS performance requirement in Rel.16.

Proposal 1: Agree to adopt the maximum Doppler shift 1944Hz for both BS enhanced HST open space and tunnel scenarios.

2.1.2	Test coverage

In TS 36.104, the test coverage of PUSCH requirements for HST is summarized as the follows.

Table 2: Summary of the test coverage on existing PUSCH requirements for HST

| Parameters | Value |  |
| --- | --- | --- |
|  | Scenario 1 (Open space) | Scenario 3 (Tunnel) |
| CBW | 1.4/3/5/10/15/20 MHz | 1.4/3/5/10/15/20 MHz |
| CP | Normal | Normal |
| FRC | A.3-2 to A.3-7 (QPSK) | A.3-2 to A.3-7 (QPSK) |
| Antenna configuration | 1Tx2Rx | 1Tx1Rx |
| Test metric | 70%ile of the max. throughput 30%ile of the max. throughput | 70%ile of the max. throughput 30%ile of the max. throughput |

As described in previous section, the same Ds and Dmin as existing HST were agreed in the last meeting. Basically, deployment scenarios for enhanced HST are quite similar to the existing HST except for the UE velocity and maximum Doppler shift assumptions, and the actual deployment is expected to deploy the same number of antennas and other conditions. Thus, the same test coverage can be considered and should be introduced.

Proposal 2: Define enhanced HST test cases with the same test coverage as Rel.8 LTE HST requirements.

Table 8.2.3.1-1 Minimum requirements of PUSCH for high speed train

| Channel Bandwidth [MHz] | Cyclic prefix | FRC (Annex A) | Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex B) | Fraction of maximum throughput | SNR [dB] |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1.4 | Normal | A3-2 | 1 | 1 | HST Scenario 3 | 30% | -1.5 |
|  |  |  |  |  |  | 70% | 1.9 |
|  |  |  |  |  | HST Scenario 3-LTE500 | 30% | TBD |
|  |  |  |  |  |  | 70% | TBD |
|  |  |  |  | 2 | HST Scenario 1 Low | 30% | -3.9 |
|  |  |  |  |  |  | 70% | -0.6 |
|  |  |  |  |  | HST Scenario 1-LTE500 Low | 30% | TBD |
|  |  |  |  |  |  | 70% | TBD |
| 3 | Normal | A3-3 |  | 1 | HST Scenario 3 | 30% | -2.1 |
|  |  |  |  |  |  | 70% | 1.6 |
|  |  |  |  |  | HST Scenario 3-LTE500 | 30% | TBD |
|  |  |  |  |  |  | 70% | TBD |
|  |  |  |  | 2 | HST Scenario 1 Low | 30% | -4.5 |
|  |  |  |  |  |  | 70% | -1.0 |
|  |  |  |  |  | HST Scenario 1-LTE500 Low | 30% | TBD |
|  |  |  |  |  |  | 70% | TBD |
| 5 | Normal | A3-4 |  | 1 | HST Scenario 3 | 30% | -2.6 |
|  |  |  |  |  |  | 70% | 1.3 |
|  |  |  |  |  | HST Scenario 3-LTE500 | 30% | TBD |
|  |  |  |  |  |  | 70% | TBD |
|  |  |  |  | 2 | HST Scenario 1 Low | 30% | -5.1 |
|  |  |  |  |  |  | 70% | -1.4 |
|  |  |  |  |  | HST Scenario 1-LTE500 Low | 30% | TBD |
|  |  |  |  |  |  | 70% | TBD |
| 10 | Normal | A3-5 |  | 1 | HST Scenario 3 | 30% | -2.7 |
|  |  |  |  |  |  | 70% | 1.2 |
|  |  |  |  |  | HST Scenario 3-LTE500 | 30% | TBD |
|  |  |  |  |  |  | 70% | TBD |
|  |  |  |  | 2 | HST Scenario 1 Low | 30% | -5.4 |
|  |  |  |  |  |  | 70% | -1.5 |
|  |  |  |  |  | HST Scenario 1-LTE500 Low | 30% | TBD |
|  |  |  |  |  |  | 70% | TBD |
| 15 | Normal | A3-6 |  | 1 | HST Scenario 3 | 30% | -2.7 |
|  |  |  |  |  |  | 70% | 1.2 |
|  |  |  |  |  | HST Scenario 3-LTE500 | 30% | TBD |
|  |  |  |  |  |  | 70% | TBD |
|  |  |  |  | 2 | HST Scenario 1 Low | 30% | -5.2 |
|  |  |  |  |  |  | 70% | -1.4 |
|  |  |  |  |  | HST Scenario 1-LTE500 Low | 30% | TBD |
|  |  |  |  |  |  | 70% | TBD |
| 20 | Normal | A3-7 |  | 1 | HST Scenario 3 | 30% | -2.7 |
|  |  |  |  |  |  | 70% | 1.2 |
|  |  |  |  |  | HST Scenario 3-LTE500 | 30% | TBD |
|  |  |  |  |  |  | 70% | TBD |
|  |  |  |  | 2 | HST Scenario 1 Low | 30% | -5.3 |
|  |  |  |  |  |  | 70% | -1.4 |
|  |  |  |  |  | HST Scenario 1-LTE500 Low | 30% | TBD |
|  |  |  |  |  |  | 70% | TBD |

The simulation assumptions for PUSCH HST requirements are summarized in table 3.

Table 3: Simulation assumptions for PUSCH HST requirements

| Parameters | Value |  |
| --- | --- | --- |
|  | Scenario 1 (Open space) | Scenario 3 (Tunnel) |
| CBW | 1.4/3/5/10/15/20 MHz | 1.4/3/5/10/15/20 MHz |
| CP | Normal | Normal |
| FRC | A.3-2 to A.3-7 (QPSK) | A.3-2 to A.3-7 (QPSK) |
| Antenna configuration | 1Tx2Rx | 1Tx1Rx |
| Reference receiver | MRC | MRC |
| Noise estimation | Practical | Practical |
| Time and frequency track | Practical | Practical |
| Test metric | 70%ile of the max. throughput 30%ile of the max. throughput | 70%ile of the max. throughput 30%ile of the max. throughput |

2.1.3	Applicability

In Rel.8 HST, the requirement has been defined as optional and do not apply to Local area BS and Home BS.

Sec. 8.2.3, TS 36.104

The performance requirement of PUSCH for high speed train is determined by a minimum required throughput for a given SNR. The required throughput is expressed as a fraction of maximum throughput for the FRCs listed in Annex A. The performance requirements assume HARQ retransmissions. The performance requirements for high speed train are optional.

This requirement shall not be applied to Local Area BS and Home BS.

The same approach as the existing HST can be considered, so the PUSCH requirement for enhanced HST is also optional and shall not be applied to Local Area BS and Home BS.

Proposal 3: The performance requirements for Rel.16 enhanced HST is optional and do not apply to Local Area BS and Home BS.

2.2	PRACH

2.2.1	Restricted set type B

In WF [1], it was agreed to specify new test case if larger maximum Doppler shift than 1875Hz is adopted for HST.

WF on PRACH restricted set type B

- Agreement:

  - If we adopt maximum Doppler shift more than 1875Hz for PUSCH HST, the new test case with frequency offset equivalent to or more the maximum Doppler shift value to be adopted will be introduced.

- Simulation assumption for alignment

  - Channel model: AWGN

  - Antenna configuration: 1x2

  - Frequency offset:

    - 1944 Hz

    - 1894 Hz

  - Other assumptions are same as existing PRACH restricted set type B requirement

In current specification, there are PRACH test cases with frequency offset up to 1875 Hz for restricted set type B. If higher maximum Doppler shift value than 1875 Hz for HST scenarios, the existing test cases are not enough to ensure the performance under enhanced HST condition. Therefore, the new test cases shall be added to PRACH requirements for high-speed mode restricted set type B.

Proposal 4: Introduce new test cases with frequency offset = 1944Hz for PRACH restricted set type B.

Table 8.4.2.1-5 PRACH missed detection requirements for High speed Mode restricted set type B

| Number of TX antennas | Number of RX antennas | Propagation conditions and correlation matrix (Annex B) | Frequency offset | SNR [dB] |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Burst format 0 | Burst format 1 | Burst format 2 | Burst format 3 |
| 1 | 2 | AWGN | 0 | -14.5 | -14.1 | -16.7 | -16.8 |
|  |  | AWGN | 625 | -12.0 | -11.7 | -13.9 | -13.9 |
|  |  | ETU 70 Low | 270 Hz | -7.3 | -6.9 | -9.1 | -9.2 |
|  |  | AWGN | 1875 Hz | -11.8 | -11.4 | -13.8 | -14.0 |
|  |  | AWGN | 1944 Hz | TBD | TBD | TBD | TBD |
|  | 4 | AWGN | 0 | -17.1 | -16.6 | -19.1 | -19.1 |
|  |  | AWGN | 625 | -14.4 | -14.1 | -16.1 | -16.2 |
|  |  | ETU 70 Low | 270 Hz | -11.8 | -11.3 | -13.5 | -13.4 |
|  |  | AWGN | 1875 Hz | -14.2 | -13.8 | -15.9 | -16.3 |
|  |  | AWGN | 1944 Hz | TBD | TBD | TBD | TBD |
|  | 8 | AWGN | 0 | -19.6 | -19.1 | -21.2 | -21.2 |
|  |  | AWGN | 625 | -16.4 | -16.3 | -18.1 | -18.2 |
|  |  | ETU 70 Low | 270 Hz | -15.3 | -15.1 | -17.1 | -17.5 |
|  |  | AWGN | 1875 Hz | -16.3 | -16.0 | -18.0 | -18.4 |
|  |  | AWGN | 1944 Hz | TBD | TBD | TBD | TBD |

3.	Conclusion

In this contribution, we provide our views on BS demodulation requirements for LTE high speed in Rel.16. The following proposals are obtained.

For PUSCH:

Proposal 1: Agree to adopt the maximum Doppler shift 1944Hz for both BS enhanced HST open space and tunnel scenarios.

Proposal 2: Define enhanced HST test cases with the same test coverage as Rel.8 LTE HST requirements.

Proposal 3: The performance requirements for Rel.16 enhanced HST is optional and do not apply to Local Area BS and Home BS.

For PRACH:

Proposal 4: Introduce new test cases with frequency offset = 1944Hz for PRACH restricted set type B.

References

- R4-1907747, “Way forward on LTE BS demodulation performance requirements”, NTT DOCOMO, INC., RAN4 #92, August 2019

- R4-1908851, “Doppler shift evaluation for LTE HST Bidirectional Scenario”, Ericsson, RAN4 #92, August 2019

- R4-1909998,	“Discussion and simulation results for BS performance with HST in LTE Rel-16 Samsung”, RAN4 #92, August 2019

- R4-1910013,	“Discuss and simulation results for BS demodulation requirements for LTE HST enhancement”, RAN4 #92, August 2019

- R4-1909887,	“BS demodulation simulation results of PUSCH for Rel-16 LTE HST”, Nokia, Nokia Shanghai Bell, RAN4 #92, August 2019