3GPP TSG-RAN4 Meeting #92-bis	R4-1912467

Chongqing , CN

Source: 	Qualcomm Incorporated

Title: 	UE capability of requiring no guard period for SRS with frequency hopping and antenna switching

Agenda item:	7.15.1

Document for:	Approval

# Introduction

In [1], RAN1 asks RAN4 if a guard period is required for frequency hopping and for antenna switching. The following is the agreement in RAN1:

Agreement

Guard period for frequency hopping and/or antenna switching can be configured regardless of intra-subframe repetition configuration.

- It is up to RAN4 to introduce a UE capability for those UEs which do not require guard period.

  - Including whether to have separate UE capabilities for frequency hopping and antenna switching if such UE capability is introduced

RAN1 asks for two sets of capabilities: one for frequency hopping and the other for Antenna switching.

# Discussion

RAN1 is discussing different patterns for SRS transmissions on consecutive symbols and would like to know if for a given UE operating in a band if a guard period is necessary when frequency hopping or Antenna switching is scheduled in consecutive SRS symbols.

Based on this information eNB could schedule a guard period or a repetition of the previous SRS in that symbol.

Proposal 1: To introduce two capabilities: one for frequency hopping and another for antenna switching. The capability will indicate whether a guard period is not required for frequency hopping and for antenna switching.

Proposal 2: This capability will be band dependent.

Proposal 3: Send an LS to RAN 1 indicating the same.

# Conclusion

RAN1 has sent an LS to RAN4 to consider defining signalling a capability if a UE doesn’t require a guard period when frequency hopping or Antenna switching has been scheduled between two consecutive SRS transmissions. In this paper, we propose to introduce the requested capabilities as stated in Proposals 1-3 above.

# References

[1] R1-1909908, LS on UE capability of guard period for SRS with frequency hopping and antenna switching, 3GPP TSG-RAN WG1 Meeting #98, Prague, Czech Republic, August 26 – 30, 2019