3GPP TSG-RAN WG4 Meeting #92bis	R4-1912476

Chongqing, China, 14 – 18 October 2019

Source:	Nokia, Nokia Shanghai Bell

Title:	Simulation results of PRACH preamble restricted set type B for Rel-16 LTE HST

Agenda item:	7.13.4.2

Document for:	Approval

# 1	Introduction

During the RAN4#92 meeting, the way forward on LTE BS demodulation performance requirements [1] was approved, which captured agreements for evaluation of PUSCH and PRACH performance with different parameters values Doppler shift for vehicular speed at 500 km/h.

This document presents simulation results for PRACH preamble restricted set type B. In conclusion, we present our views based on the simulation results.

# 2	Discussion

From [1], the agreed simulation assumptions for high speed mode PRACH restricted set type B are listed in Table.

Table 1: Simulation assumptions [1]

| Parameter | Value |
| --- | --- |
| Channel model | AWGN |
| Antenna configuration | 1x2 |
| Frequency offset | 1944 Hz, 1894 Hz |
| Other assumptions are the same as existing PRACH restricted set type B requirements |  |

From TS 36.104 in A.6, the test preambles for high speed mode restricted set type B are listed in Table 2.

Table 2: Test preambles for high speed mode restricted set type B

| Burst format | Ncs | Logical sequence index | v |
| --- | --- | --- | --- |
| 0 | 15 | 30 | 30 |
| 1 | 100 | 168 | 20 |
| 2 | 118 | 204 | 10 |
| 3 | 137 | 264 | 0 |

Table 3: Minimum SNR for achieving the detection probability = 99%

| Frequency offset | SNR |  |  |  |
| --- | --- | --- | --- | --- |
|  | Burst Format 0 | Burst Format 1 | Burst Format 2 | Burst Format 3 |
| 1894 Hz | -11.8 dB | -11.4 dB | -13.8 dB | -14.0 dB |
| 1944 Hz | -11.8 dB | -11.4 dB | -13.8 dB | -14.0 dB |

Figure 1: Simulation results for PRACH Formats 0 and 1

Figure 2: Simulation results for PRACH Formats 2 and 3

Figures 1 and 2 show the PRACH detection simulation results for preamble Formats 0 and 1, and Formats 2 and 3, respectively. According to TS 36.104, the target PRACH detection probability is 99%.

As can be observed from the figures, the target detection probability is obtained at the same SNR for both Doppler shifts fd = 1894 Hz and 1944 Hz for each of the different PRACH preamble formats.

Observation 1: The minimum SNR needed to achieve the target PRACH detection probability of 99% is the same for both Doppler shifts fd = 1894 Hz and 1944 Hz for each of the different PRACH preamble formats.

Based on the simulation results, the minimum SNR for achieving the target PRACH detection probability of 99% is shown in Table 3.

# 3	Conclusions

This document provides simulation results for PRACH preamble restricted set type B. As a result, the following observations and proposal can be made:

Observation 1: The minimum SNR needed to achieve the target PRACH detection probability of 99% is the same for both Doppler shifts fd = 1894 Hz and 1944 Hz for each of the different PRACH preamble formats.

The minimum SNR for achieving the target PRACH detection probability of 99% for each of the preamble formats is as follows:

| Frequency offset | SNR |  |  |  |
| --- | --- | --- | --- | --- |
|  | Burst Format 0 | Burst Format 1 | Burst Format 2 | Burst Format 3 |
| 1894 Hz | -11.8 dB | -11.4 dB | -13.8 dB | -14.0 dB |
| 1944 Hz | -11.8 dB | -11.4 dB | -13.8 dB | -14.0 dB |

# References

- R4-1910102, Way forward on LTE BS demodulation performance requirements, NTT DoCoMo