3GPP TSG-RAN WG4 Meeting #92bis	R4-1912503

# Chongqing, China, 14- 18 October, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | --- | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on CSI reporting requirements for EN-DC including FR1 and FR2 CCs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-10-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There is no CSI reporting requirements specified for EN-DC including both FR1 and FR2 NR CCs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify FR1-FR2 EN-DC CSI reporting requirements following the existing principles to define EN-DC (only verifying the NR CC performance) and FR1-FR2 band combination (only verifying FR2 NR CC performance) performance requirements. In that way the UE supporting EN-DC including FR1 NR and FR2 NR CCs shall be verified under the EN-DC fallback mode including FR1 NR CC only or FR2 NR CC only against the corresponding EN-DC performance requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The CSI reporting requirements for EN-DC including both FR1 and FR2 CCs are missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes>>

10.2	Reporting of Channel Quality Indicator (CQI)

10.2A	Reporting of Channel Quality Indicator (CQI) for CA

(Void)

10.2B	Reporting of Channel Quality Indicator (CQI) for DC

10.2B.1	EN-DC

10.2B.1.1	EN-DC within FR1

The test setup for E-UTRA PCell is specified in Clause 9.1.2. The NR CQI reporting requirements are specified in Clause 6.2. During the test, only the performance based on NR requirements on the NR cell(s) shall be verified.

10.2B.1.2	EN-DC including FR2 NR carrier

The test setup for E-UTRA PCell is specified in Clause 9.1.2. The NR CQI reporting requirements are specified in Clause 8.2. During the test, only the performance based on NR requirements on the NR cell(s) on FR2 carriers shall be verified.

10.2B.1.3	EN-DC including FR1 and FR2 NR carriers

The test setup for E-UTRA PCell is specified in Section 9.1.2. During the test, only the CQI performance on the NR cell(s) shall be verified. The performance on FR1 carriers and FR2 carriers shall be verified separately. The test coverage is considered fulfilled, if the CQI reporting requirements specified in Section 10.2B.1.1 for EN-DC within FR1 and the CQI reporting requirements specified in Section 10.2B.1.2 for EN-DC including FR2 NR carrier only are executed separately for UE under test in the fallback mode of supported EN-DC configurations including FR1 and FR2 NR carriers.

10.2B.2	NR DC between FR1 and FR2

10.3	Reporting of Precoding Matrix Indicator (PMI)

10.3A	Reporting of Precoding Matrix Indicator (PMI) for CA

(Void)

10.3B	Reporting of Precoding Matrix Indicator (PMI) for DC

10.3B.1	EN-DC

10.3B.1.1	EN-DC within FR1

The test setup for E-UTRA PCell is specified in Clause 9.1.2. The NR PMI reporting requirements are specified in Clause 6.3. During the test, only the performance based on NR requirements on the NR cell(s) shall be verified.

10.3B.1.2	EN-DC including NR FR2 carrier

The test setup for E-UTRA PCell is specified in Clause 9.1.2. The NR PMI reporting requirements are specified in Clause 8.3. During the test, only the performance based on NR requirements on the NR cell(s) on FR2 carriers shall be verified.

10.3B.1.3	EN-DC including FR1 and FR2 NR carriers

The test setup for E-UTRA PCell is specified in Section 9.1.2. During the test, only the PMI performance on the NR cell(s) shall be verified. The performance on FR1 carriers and FR2 carriers shall be verified separately. The test coverage is considered fulfilled, if the PMI reporting requirements specified in Section 10.3B.1.1 for EN-DC within FR1 and the PMI reporting requirements specified in Section 10.3B.1.2 for EN-DC including FR2 NR carrier only are executed separately for UE under test in the fallback mode of supported EN-DC configurations including FR1 and FR2 NR carriers.

10.3B.2	NR DC between FR1 and FR2

10.4	Reporting of Rank Indicator (RI)

10.4A	Reporting of Rank Indicator (RI) for CA

10.4B	Reporting of Rank Indicator (RI) for DC

10.4B.1	EN-DC

10.4B.1.1	EN-DC within FR1

The test setup for E-UTRA PCell is specified in Clause 9.1.2. The NR RI reporting requirements are specified in Clause 6.4. During the test, only the performance based on NR requirements on the NR cell(s) shall be verified.

10.4B.1.2	EN-DC including NR FR2 carrier

The test setup for E-UTRA PCell is specified in Clause 9.1.2. The NR RI reporting requirements are specified in Clause 8.4. During the test, only the performance based on NR requirements on the NR cell(s) on FR2 carriers shall be verified.

10.4B.1.3	EN-DC including FR1 and FR2 NR carriers

The test setup for E-UTRA PCell is specified in Section 9.1.2. During the test, only the RI performance on the NR cell(s) shall be verified. The performance on FR1 carriers and FR2 carriers shall be verified separately. The test coverage is considered fulfilled, if the RI reporting requirements specified in Section 10.4B.1.1 for EN-DC within FR1 and the RI reporting requirements specified in Section 10.4B.1.2 for EN-DC including FR2 NR carrier only are executed separately for UE under test in the fallback mode of supported EN-DC configurations including FR1 and FR2 NR carriers.

10.4B.2	NR DC between FR1 and FR2

<< End of changes >>