3GPP TSG-RAN WG4 Meeting #92bis                                                                 R4-1912518

Chongqing, China, 14th – 18th October, 2019

Title:	Draft LS reply to RAN1 on NR V2X S-SSB design

Response to:	R1-1909874

Release:	Release 16

Work Item:	5G_V2X_NRSL-Core

Source:	TSG RAN WG4

To:	TSG RAN WG1

Cc:

Contact Person:

Name:	Liu Ye

E-mail Address:	leo.liuye@huawei.com

Attachments:	None

1. Overall Description:

RAN4 has discussed the transient period issue in RAN1 LS on NR S-SSB structure. Transient period in current RAN4 specification is allowed between continuous ON-power transmissions for cases with power change or RB hopping. For the NR S-SSB, RAN4 noticed that the PAPR of PSBCH, S-PSS and S-SSS symbols are different. For the NR S-SSB structure agreed by RAN1, the transmission power between the first PSBCH and first S-PSS symbols or the S-PSS and S-SSS symbols could be slightly different. Adding PSBCH symbols between S-PSS and S-SSS symbols can’t solve this issue at all.

From RAN4 prospectively, for the NR S-SSB structure agreed by RAN1, power change could be avoided according to proper UE implementation, e.g. the transmission power of two S-PSS symbols can be reduced such that there are no transmission power differences within the S-SSB.

Therefore, the transient period for this specific S-SSB design is not necessary from RAN4 perspective.

2. Actions:

To RAN1

ACTION: 	RAN4 kindly asks RAN1 to take the above information into account in the future work.

3. Date of Next TSG-RAN WG2 Meetings:

3GPP RAN4 #93		November 18 – 22, 2019    		Reno, USA

3GPP RAN4 #94		February 24 – 28, 2020		Athens, Greece