3GPP TSG-RAN WG4 RAN4#92bis	R4-1912553

Chongqing, China, Oct 14th – Oct 18th, 2019

Agenda item:	8.7.3

Source: 	Qualcomm Incorporated

Title: 	Updating RRM requirements due to power saving work item

Document for:	Discussion

# Introduction

3GPP has designed a study and work item on ‘power saving’ topic. RAN1 and RAN2 have already completed several meetings to discuss this topic. In this contribution, we discuss how this work topic impacts existing RRM requirements.

# WI Background

[1] contain the most recent work item document (WID) corresponding to the topic ‘UE power saving in NR’. We show RAN4 related objectives of this WID below.

- Specify power saving techniques with PDCCH-based power saving signal/channel triggering UE adaptation in RRC_CONNECTED mode [RAN1, RAN2, RAN4] ……….. - Specify the procedure of cross-slot scheduling power saving techniques [RAN1, RAN4] NOTE: The procedure is in addition to Rel-15 cross-slot scheduling procedure - Specify the power saving techniques of UE adaptation to the maximum number of MIMO layers [RAN1, RAN2, RAN4] - Specify configuration of a different MIMO layer configuration of the initial/default BWP compared with other BWPs of a Serving Cell. [RAN2, RAN4] - …. - Evaluate if switching and interruption times for UE dynamic adaptation to the maximum number of MIMO layers are needed and which case assuming a relationship between the number of RF ports and the MIMO layer configuration [RAN4] NOTE: Switching on/off the RF is part of the evaluation …………… - Specify network-configured mechanism to relax intra and inter-frequency RRM measurement for neighbour cells for RRC_IDLE/INACTIVE with minimal mobility performance impacts [RAN2, RAN4] - Specify RRM measurement relaxation by allowing measurements with longer intervals, and/or by reducing the number of cells/carriers to be measured [RAN4, RAN2] - Define triggering criteria for the UE to move between relaxed and normal RRM measurements, that considers at least if UE is not at cell edge, or if UE is stationary or with low mobility [RAN2, RAN4]. - Evaluate and specify requirements (if needed) depending on RAN2 criteria [ RAN4] NOTE: No new measurement quantities should be introduced

Above description shows that 3GPP has agreed to investigate following three RAN4 related topics in ‘power saving’ work item:

            - Relaxation of intra and inter-frequency RRM measurement for neighbor cells for RRC_IDLE/INACTIVE

            - Cross-slot scheduling power saving technique

            - Power saving techniques of UE adaptation to the maximum number of MIMO layers

RAN4 RF session is already discussing the third topic. RAN1 is still discussing the second topic. Hence, we will just focus on the first topic in this contribution. The second and third topics can be discussed after RAN4 RF and RAN1 finalizes their discussions.

Besides, we also discuss if wake up signal impacts RLM/BFD/CBD evaluation.

# Relaxation of Measurement of Neighbour Cells

The WID description shows that RRM measurement for serving cells will not be relaxed in Rel-16 power saving work item. Measurement relaxation is only applicable for neighbour cell for RRC_IDLE/INACTIVE mode.

Observation 1: According to WID, RRM measurement relaxation is only applicable for neighbour cell for RRC_IDLE/INACTIVE mode.

RAN2 made the following agreement in the most recent meeting [2].

Agreements - Measurement relaxation criteria can consider both low mobility and UE location in the cell (e.g. whether the UE is in cell-edge). - UE may activate relaxed measurement criteria if at least any of the following conditions are met: a) Serving Cell measurement does not change more than a relative threshold during a time period - LTE relaxed monitoring criteria in 36.304 is considered as a baseline. Additional enhancements to address aspects that are specific to NR can be considered. b) UE is not a cell edge, meaning that serving cell/beam RSRP/RSRQ/SINR is above a threshold FFS: Whether neighbour cell RSRP should also be considered.

Observation 2: RAN2 agreed that UE can activate relaxed measurement criteria is some conditions are met. The relaxed monitoring criteria in 36.304 is considered as a baseline.

The LTE relaxed monitoring criteria of 36.304 is shown below.

- When the UE is required to perform intra-frequency or inter-frequency measurement according to the measurement rules in sub-clause 5.2.4.2 or 5.2.4.2a, the UE may choose not to perform intra-frequency or inter-frequency measurements when: - The relaxed monitoring criterion in sub-clause 5.2.4.12.1 is fulfilled for a period of TSearchDeltaP, and - Less than 24 hours have passed since measurements for cell reselection were last performed, and - The UE has performed intra-frequency or inter-frequency measurements for at least TSearchDeltaP after selecting or reselecting a new cell.

For narrowband-IOT, LTE captured UE’s behaviour regarding measurement relaxation in the following way:

- If all the relaxed monitoring criteria defined in clause 5.2.4.12 [1] are fulfilled, the UE’s intra-frequency measurement is not required to meet Tdetect,NB_Intra_NC, Tmeasure,NB_Intra_NC and Tevaluate,NB_intra_NC as defined in Table 4.6.2.2-1 and Table 4.6.2.2-2.

Observation 3: In LTE NB-IoT, if relaxation conditions defined by RAN2 was met, UE was not required to meet intra-frequency and inter-frequency measurement periods.

We can apply the same logic to NR power saving work item, as well. With power saving feature, if the UE will not be required to meet neighbour cell intra-frequency and inter-frequency measurement periods during RRC idle/inactive mode.

Proposal 1: With power saving feature, if the relaxed monitoring criteria defined by RAN2 is satisfied, UE will not be required to meet neighbour cell intra-frequency and inter-frequency measurement periods during RRC idle/inactive mode.

# Impact of WUS on RLM/BFD/CBD Evaluation Periods

RAN2 has already agreed that the design of WUS don’t impact RLM and RMM measurements [5]

RAN2 Agreements:

………….

- RLM and RRM measurements are not impacted by WUS design (i.e. the UE continues to measure the required reference signals as per RRM requirements)

However, RAN1 or RAN2 have not yet decided if WUS design would impact BFD or CBD evaluation periods. RAN4 should discuss if BFD and CBD evaluation periods get impacted by WUS design.

Observation 4: RAN4 should discuss if BFD and CBD evaluation periods get impacted by WUS design.

# Conclusion

Observation 1: According to WID, RRM measurement relaxation is only applicable for neighbour cell for RRC_IDLE/INACTIVE mode.

Observation 2: RAN2 agreed that UE can activate relaxed measurement criteria if some conditions are met. The relaxed monitoring criteria in 36.304 is considered as a baseline.

Observation 3: In LTE NB-IoT, if relaxation conditions defined by RAN2 was met, UE was not required to meet intra-frequency and inter-frequency measurement periods.

Observation 4: RAN4 should discuss if BFD and CBD evaluation periods get impacted by WUS design.

Proposal 1: With power saving feature, if the relaxed monitoring criteria defined by RAN2 is satisfied, UE will not be required to meet neighbour cell intra-frequency and inter-frequency measurement periods during RRC idle/inactive mode.

# References

1.  R4-191607, “New WID: UE Power Saving in NR”, CATT, CAICT.

2. Chairman’s notes, RAN WG2 Meeting #106bis.

3. Chairman’s notes, RAN WG1 Meeting #97.

4. 38.214, NR Physical layer procedures for data.

5. Chairman’s notes, RAN WG2 Meeting #107