3GPP TSG-RAN WG4 Meeting #92	R4-1912586

# Chongqing, China, 14th– 18th October, 2019

Source:	Qualcomm

Title:	TP for TR 37.716-33: DC_66_n5-n260 and DC_66_n5-n261

Agenda item:	9.13.2

Document for:	Approval

# Introduction

In RAN#85 meeting, the revised WID on EN-DC for 3 bands DL with 3 bands UL (1CC LTE band +  1CC NR FR1 band +  NR FR2 band, or 2 LTE different band UL and 1 NR FR2 band UL) was approved [1]. This contribution is a text proposal for TR 37.716-33 to include DC_66_n5-n260 and DC_66_n5-n261 combos.

# Reference

[1] 	RP-191882, revised WID on EN-DC for 3 bands DL with 3 bands UL (1CC LTE band +  1CC NR FR1 band +  NR FR2 band, or 2 LTE different band UL and 1 NR FR2 band UL), ZTE

# Text Proposal

<Start of Text Proposal>

## 6.1.1	DC_66_n5-n260

### 6.1.1.1	Operating bands for DC_66_n5-n260

Table 6.1.1.1-1: Band combinations for DC_66_n5-n260

| EN_DC Band | E-UTRA Band | NR Band |
| --- | --- | --- |
| DC_66_n5-n260 | 66 | n5, n260 |

### 6.1.1.2	Configurations for DC_66_n5-n260

Table 6.1.1.2-1: : Inter-band EN-DC configurations for DC_66_n5-n260

| EN_DC Configuration | Uplink EN_DC Configuration |
| --- | --- |
| DC_66A_n5A-n260G DC_66A_n5A-n260H DC_66A_n5A-n260I DC_66A_n5A-n260J DC_66A_n5A-n260K DC_66A_n5A-n260L DC_66A_n5A-n260M | DC_66A-n5A-n260A |
| DC_66A_n5A-n260(2A) DC_66A_n5A-n260(3A) DC_66A_n5A-n260(4A) DC_66A_n5A-n260(5A) DC_66A_n5A-n260(6A) DC_66A_n5A-n260(2H) DC_66A_n5A-n260(2G) DC_66A_n5A-n260(A-2G) DC_66A_n5A-n260(A-H) DC_66A_n5A-n260(A-G) DC_66A_n5A-n260(G-H) DC_66A_n5A-n260(2A-G) DC_66A_n5A-n260(2A-2G) DC_66A_n5A-n260(3A-G) | DC_66A-n5A-n260A |

### 6.1.1.3	∆TIB and ∆RIB values

For DC_66A_n5A-n260A, the TIB,c and RIB,c values are given in the tables below.

Table 6.1.1.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66_n5-n260 | 66 | 0.3 |
|  | n5 | 0.3 |
|  | n260 | 0 |

Table 6.1.1.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_66_n5-n260 | 66 | 0 |
|  | n5 | 0 |
|  | n260 | 0 |

### 6.1.1.4	REFSENS requirements

No additional MSD is required for DC_66_n5-n260

## 6.1.2	DC_66_n5-n261

### 6.1.2.1	Operating bands for DC_66_n5-n261

Table 6.1.2.1-1: Band combinations for DC_66_n5-n261

| EN_DC Band | E-UTRA Band | NR Band |
| --- | --- | --- |
| DC_66_n5-n261 | 66 | n5, n261 |

### 6.1.2.2	Configuration for DC_66_n5-n261

Table 6.1.2.2-1: Inter-band DC_66_n5-n261 configurations

| EN_DC Configuration | Uplink EN_DC Configuration |
| --- | --- |
| DC_66A_n5A-n261A DC_66A_n5A-n261G DC_66A_n5A-n261H DC_66A_n5A-n261I DC_66A_n5A-n261J DC_66A_n5A-n261K DC_66A_n5A-n261L DC_66A_n5A-n261M | DC_66A_n5A-n261A |
| DC_66A_n5A-n261(2A) DC_66A_n5A-n261(3A) DC_66A_n5A-n261(2G) DC_66A_n5A-n261(2H) DC_66A_n5A-n261(A-G) DC_66A_n5A-n261(A-H) DC_66A_n5A-n261(A-I) DC_66A_n5A-n261(A-J) DC_66A_n5A-n261(A-K) DC_66A_n5A-n261(G-H) DC_66A_n5A-n261(G-I) DC_66A_n5A-n261(G-J) DC_66A_n5A-n261(H-I) DC_66A_n5A-n261(A-2G) DC_66A_n5A-n261(A-G-H) DC_66A_n5A-n261(A-G-I) DC_66A_n5A-n261(2A-G) DC_66A_n5A-n261(2A-H) DC_66A_n5A-n261(2A-I) DC_66A_n5A-n261(3A-G) | DC_66A_n5A-n261A |

### 6.1.2.3	∆TIB and ∆RIB values

For DC_66A_n5A-n261A, the TIB,c and RIB,c values are given in the tables below.

Table 6.1.2.3-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_66_n5-n261 | 66 | 0.3 |
|  | n5 | 0.3 |
|  | n261 | 0 |

Table 6.1.2.3-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_66_n5-n261 | 66 | 0 |
|  | n5 | 0 |
|  | n261 | 0 |

### 6.1.2.4	REFSENS requirements

No additional MSD is required for DC_66_n5-n261