3GPP TSG-RAN WG4 Meeting #92bis                                                                     R4-1912597

Chongqing, China, 14th-18th Oct, 2019

Source:	Samsung, KT

Title:	TP for TR 37.716-21-21: DC_1A-8A_n78A-n257A

Agenda item:			9.8.3

Document for:	Approval

# 1. Introduction

In RAN#85 meeting, the revised WID on EN-DC for LTE (xDL/UL x=1, 2, 3, 4) with NR 2 bands (2DL/1UL) was approved [1], in which the latest EN-DC for LTE (xDL/UL x=1, 2, 3, 4) with NR 2 bands (2DL/1UL) was attached. This contribution is a text proposal for TR 37.716-21-21 to include related band combinations for DC_1A-8A_n78A-n257.

# 2. Reference

- RP-191744, Revised WID on LTE (xDL/UL x=1, 2, 3, 4) with NR 2 bands (2DL/1UL) EN DC in Rel-16, LG Electronics France.

# 3. Text Proposal

<Start of Text Proposal>

## 7.x	DC_1-8_n78-n257

### 7.x.1	Operating bands for DC

Table 7.x.1-1: DC band combination of LTE 2DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_1-8_n78-n257 | 1 | 1920 MHz | – | 1980 MHz | 2110 MHz | – | 2170 MHz | FDD |
|  | 8 | 880MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | n78 | 3300 MHz | – | 3800 MHz | 3300 MHz | – | 3800 MHz | TDD |
|  | n257 | 26500 MHz | – | 29500 MHz | 26500 MHz | – | 29500 MHz |  |

### 7.x.2	Channel bandwidths per operating band for DC

Table 7.x.2-1: Supported bandwidths per DC LTE 2DL/1UL + inter-band NR 2DL/1UL

|  |  | CA configuration / channel bandwidth [MHz] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration |  | UL Configuration | NR Band | SCS [kHz] | 5 |  | 10 | 15 | 20 | 40 | 50 | 60 | 80 | 90 | 100 | 200 | 400 | Maximum aggregated bandwidth [MHz] |  |
| DC_1A-8A_n78A-n257A |  | DC_1A_n78A DC_8A_n78A DC_1A_n257A DC_8A_n257A | 1 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 530 |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | n257 | 60 |  |  |  |  |  |  | Yes |  |  |  | Yes | Yes |  |  |  |
|  |  |  |  | 120 |  |  |  |  |  |  | Yes |  |  |  | Yes | Yes | Yes |  |  |
| DC_1A-8A_n78A-n257D |  | DC_1A_n78A DC_8A_n78A DC_1A_n257A DC_8A_n257A | 1 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 530 |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257D in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-8A_n78A-n257E |  | DC_1A_n78A DC_8A_n78A DC_1A_n257A DC_8A_n257A | 1 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 730 |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257E in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-8A_n78A-n257F |  | DC_1A_n78A DC_8A_n78A DC_1A_n257A DC_8A_n257A | 1 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 930 |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257F in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-8A_n78A-n257G |  | DC_1A_n78A DC_8A_n78A DC_1A_n257A DC_8A_n257A | 1 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 330 |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257G in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-8A_n78A-n257H |  | DC_1A_n78A DC_8A_n78A DC_1A_n257A DC_8A_n257A | 1 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 430 |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257H in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-8A_n78A-n257I |  | DC_1A_n78A DC_8A_n78A DC_1A_n257A DC_8A_n257A | 1 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 530 |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257I in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-8A_n78A-n257J |  | DC_1A_n78A DC_8A_n78A DC_1A_n257A DC_8A_n257A | 1 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 630 |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257J in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-8A_n78A-n257K |  | DC_1A_n78A DC_8A_n78A DC_1A_n257A DC_8A_n257A | 1 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 730 |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257K in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-8A_n78A-n257L |  | DC_1A_n78A DC_8A_n78A DC_1A_n257A DC_8A_n257A | 1 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 830 |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257L in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |
| DC_1A-8A_n78A-n257M |  | DC_1A_n78A DC_8A_n78A DC_1A_n257A DC_8A_n257A | 1 | 15 | Yes |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  | 930 |  |
|  |  |  | 8 | 15 | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | n78 | 15 |  |  | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  |  | 30 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  |  | 60 |  |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |
|  |  |  | n257 |  |  | See CA_n257M in Table 5.5A1-2 in TS 38.101-2 |  |  |  |  |  |  |  |  |  |  |  |  |  |

### 7.x.3	Co-existence studies

Co-existence studies of this 4DL/2UL DC configuration are already covered by those of the 2DL/2UL and 3DL/2UL captured in TR 37.863-01-01, TR 37.863-02-01 and TR 37.716-21-21 respectively. Therefore, additional analysis is not needed.

### 7.x.4	∆TIB and ∆RIB values

For DC_1-8_n78-n257, the TIB,c and RIB,c values are given in the tables below.

Table 7.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-8_n78-n257 | 1 | 0.3 |
|  | 8 | 0.6 |
|  | n78 | 0.8 |
|  | n257 | 0 |

Table 7.x.4-2: ΔRIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB,c [dB] |
| --- | --- | --- |
| DC_1-8_n78-n257 | 1 | 0 |
|  | 8 | 0.2 |
|  | n78 | 0.5 |
|  | n257 | 0 |

### 7.x.5	MSD

There is no need to specify additional MSD requirement for this UL DC configuration.

<End of Text Proposal>