3GPP TSG-RAN WG4 Meeting #92bis	R4-1912754

# Chongqing, China, 14- 18 October, 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-4 | CR | --- | rev | - | Current version: | 15.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on CSI reporting requirements for EN-DC including FR1 and FR2 CCs |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-10-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | There is no CSI reporting requirements specified for EN-DC including both FR1 and FR2 NR CCs. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify FR1-FR2 EN-DC CSI reporting requirements following the existing principles to define EN-DC (only verifying the NR CC performance) and FR1-FR2 band combination (only verifying FR2 NR CC performance) performance requirements. In that way the UE supporting EN-DC including FR1 NR and FR2 NR CCs shall be verified under the EN-DC fallback mode including FR1 NR CC only or FR2 NR CC only against the corresponding EN-DC performance requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The CSI reporting requirements for EN-DC including both FR1 and FR2 CCs are missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.1.1,10.2B.1.3,10.3B.1.3,10.4B.1.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Start of changes>>

10	CSI reporting requirements for interworking

10.1	General

This clause specifies CSI performance requirements for EN-DC, NE-DC, inter-band NR-DC between FR1 and FR2, and inter-band NR CA between FR1 and FR2.

The definition of frequency ranges (FR1 and FR2) are specified in table 5.1-1 of TS 38.101-3 [8].

10.1.1	Applicability of requirements

The following applicability rules are specified for demodulation performance requirements for interworking:

-	For UEs supporting both SA and NSA,

-	The performance requirements specified in Clause 6 will be verified only for SA.

-	The performance requirements specified in Clause 8 will be verified only for SA.

-	The FR1 EN-DC test cases with the NR TDD DL-UL configurations which are not aligned with LTE's can be tested on the corresponding EN-DC band combinations where UE supports simultaneous transmission and reception.

-	For UEs supporting EN-DC including FR2 and/or EN-DC including FR1 and FR2, the requirements applicability is specified in Table 10.1.1-1.

Table 10.1.1-1: Requirements applicability for UEs supporting EN-DC including FR2 and/or EN-DC including FR1 and FR2

| Supported scenarios | CQI requirements | PMI requirements | RI requirements |
| --- | --- | --- | --- |
| EN-DC including FR2 | Section 10.2B.1.2 | Section 10.3B.1.2 | Section 10.4B.1.2 |
| EN-DC including FR1 and FR2 | Section 10.2B.1.3 | Section 10.3B.1.3 | Section 10.4B.1.3 |
| Both EN-DC including FR2 and EN-DC including FR1 and FR2 | Section 10.2B.1.2 | Section 10.3B.1.2 | Section 10.4B.1.2 |

10.1.1.1	Applicability of requirements for optional UE features

For UE which supports optional UE features the additional performance requirements from Table 10.1.1.1-1 should be applied.

<< Next of changes>>

10.2	Reporting of Channel Quality Indicator (CQI)

10.2A	Reporting of Channel Quality Indicator (CQI) for CA

(Void)

10.2B	Reporting of Channel Quality Indicator (CQI) for DC

10.2B.1	EN-DC

10.2B.1.1	EN-DC within FR1

The test setup for E-UTRA PCell is specified in Clause 9.1.2. The NR CQI reporting requirements are specified in Clause 6.2. During the test, only the performance based on NR requirements on the NR cell(s) shall be verified.

10.2B.1.2	EN-DC including FR2 NR carrier

The test setup for E-UTRA PCell is specified in Clause 9.1.2. The NR CQI reporting requirements are specified in Clause 8.2. During the test, only the performance based on NR requirements on the NR cell(s) on FR2 carriers shall be verified.

10.2B.1.3	EN-DC including FR1 and FR2 NR carriers

The test setup for E-UTRA PCell is specified in Clause 9.1.2. The NR CQI reporting requirements are specified in Clause 10.2B.1.1 and Section 10.2B.1.2. During the test, only the performance based on NR requirements on the NR cell(s) on FR2 carriers shall be verified.

10.2B.2	NR DC between FR1 and FR2

10.3	Reporting of Precoding Matrix Indicator (PMI)

10.3A	Reporting of Precoding Matrix Indicator (PMI) for CA

(Void)

10.3B	Reporting of Precoding Matrix Indicator (PMI) for DC

10.3B.1	EN-DC

10.3B.1.1	EN-DC within FR1

The test setup for E-UTRA PCell is specified in Clause 9.1.2. The NR PMI reporting requirements are specified in Clause 6.3. During the test, only the performance based on NR requirements on the NR cell(s) shall be verified.

10.3B.1.2	EN-DC including NR FR2 carrier

The test setup for E-UTRA PCell is specified in Clause 9.1.2. The NR PMI reporting requirements are specified in Clause 8.3. During the test, only the performance based on NR requirements on the NR cell(s) on FR2 carriers shall be verified.

10.3B.1.3	EN-DC including FR1 and FR2 NR carriers

The test setup for E-UTRA PCell is specified in Section 9.1.2. The NR PMI reporting requirements are specified in Section 10.3B.1.1 and Section 10.3B.1.2. During the test, only the performance based on NR requirements on the NR cell(s) on FR2 carriers shall be verified.

10.3B.2	NR DC between FR1 and FR2

10.4	Reporting of Rank Indicator (RI)

10.4A	Reporting of Rank Indicator (RI) for CA

10.4B	Reporting of Rank Indicator (RI) for DC

10.4B.1	EN-DC

10.4B.1.1	EN-DC within FR1

The test setup for E-UTRA PCell is specified in Clause 9.1.2. The NR RI reporting requirements are specified in Clause 6.4. During the test, only the performance based on NR requirements on the NR cell(s) shall be verified.

10.4B.1.2	EN-DC including NR FR2 carrier

The test setup for E-UTRA PCell is specified in Clause 9.1.2. The NR RI reporting requirements are specified in Clause 8.4. During the test, only the performance based on NR requirements on the NR cell(s) on FR2 carriers shall be verified.

10.4B.1.3	EN-DC including FR1 and FR2 NR carriers

The test setup for E-UTRA PCell is specified in Section 9.1.2. The NR PMI reporting requirements are specified in Clause 10.4B.1.1 and Section 10.4B.1.2. During the test, only the performance based on the NR requirements on the NR cell(s) on FR2 carriers shall be verified.

10.4B.2	NR DC between FR1 and FR2

<< End of changes >>