3GPP TSG-RAN4 Meeting #92bis	R4-1912893

# Chongqing, China, October 14th -18th, 2019

Source:	Ericsson

Title:	TP to TR 38.883: Section 5.2.1 Link level simulation

Agenda item:	8.12.2

Document for:	Approval

# 1.	Introduction

Revision of R4-1911801 to remove conclusion section based on online discussion to separate conclusion section in separate TP.

# 2.	References

[1]	R4-1910480, "TP for TR 38.883: Link level simulation results for the feasibility study of FR2 DL 256QAM" (China Telecom).

[2]	R4-1910478, “TR skeleton for TR 38.883: Study on support of NR downlink 256 Quadrature Amplitude Modulation (QAM) for frequency range 2 (FR2)” (China Telecom).

[Start of Text Proposal]

### 5.2.1	Link level simulation

Link level simulation is targeted as mainstream way to evaluate if FR2 256QAM can achieve benefit by comparing to 64QAM. The simulation results from companies are listed as below.

#### 5.2.1.1	Simulation assumptions

[Unchanged text]

Table 5.2.1.1-2 link level simulation assumptions down-selected by companies

| Parameter |  |  |  | CTC[5] | Nokia[6] | Docomo[7] | Huawei[8] | Ericsson[9] | CATT[10] | Intel[11] | Qualcomm[12] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Carrier frequency |  |  |  | 29 GHz | 29 GHz | 29 GHz | 29 GHz |  | 29 GHz |  |  |
| CBW |  |  |  | 50MHz | 100MHz | 100MHz | 50MHz | 50 MHz | 50MHz | 50MHz | 100MHz |
| SCS |  |  |  | 120kHz | 60kHz | 120kHz | 120kHz | 60kHz | 60kHz | 60kHz | 120kHz |
| Allocated RBs |  |  |  |  |  |  |  |  |  |  |  |
| Propagation |  |  | TDL-A |  |  |  |  |  |  |  |  |
|  |  |  | TDL-D |  |  |  |  |  |  |  |  |
|  |  |  | Static |  |  |  |  |  |  |  |  |
| MCS |  |  | 64QAM | 28 | 26,28 | 24,25,26,28 | 23,24,26,28 | 23,24,26,28 | 23 | 23,24,26,28 | 26,27,28 |
|  |  |  | 256QAM | 27 | 21,23 | 21,23,25,27 | 21,23,25,27 | 21,23,25,27 | 21 | 21,23,25,27 | 20,21,22 |
| Precoding |  |  |  |  |  |  |  |  |  |  |  |
| Symbol type |  |  |  |  |  |  |  |  |  |  |  |
| HARQ |  |  |  |  | None |  | None |  | 8 |  |  |  |  |  | 8 |  | 8 |  | 8 |  |
| Antenna configuration |  | Fading |  | 2x2 for Rank1 | 2x2 for Rank1 |  |  | 1x2, 2x2 for Rank1 | 2x2 for Rank1 |  | 2x2 for Rank2 |
|  |  | Static |  |  |  | 2x2 for Rank2 |  |  | 1x2 for Rank1 |  | 2x2 for Rank2 |
| Channel estimation |  |  |  |  |  |  |  |  |  |  |  |
| Receiver type |  |  |  |  |  |  |  |  |  |  |  |
| PDSCH configuration |  |  |  |  |  |  |  |  |  |  |  |
| DMRS configuration |  |  |  |  |  |  |  | No additional |  |  |  |
| PTRS configuration |  |  |  | None |  |  |  |  |  |  |  |
| Phase noise compensation |  |  |  | None |  | Ideal |  |  |  |  |  |
| Phase noise model | Option a) |  |  |  |  |  |  |  |  |  |  |
|  | Option b) |  |  |  |  |  |  |  |  |  |  |
|  | Option c) |  |  |  |  |  |  |  |  |  |  |
|  | Option d) |  |  |  |  |  |  |  |  |  |  |
|  | Option e) |  |  |  |  |  |  |  |  |  | example1BS+example2UE internal PN model |
| txEVM + rxEVM excluding phase noise for 256QAM |  |  |  | Tx+Rx: 3%, 4% | txEVM: 3%, rxEVM: 3% | txEVM: 0%, 3%, rxEVM: 0%, 3% | txEVM: 1%-3%, rxEVM: 1%-3% |  |  | txEVM: 1%-3%, rxEVM: 1%-3% | Tx:3% Rx:internal |
| Other parameters |  |  |  |  |  |  |  |  |  |  |  |
| Note: The symbol of means selecting the parameters corresponding to table 5.2.1.1-1. |  |  |  |  |  |  |  |  |  |  |  |

[Unchanged text]

### 5.2.2	Simulation results

#### 5.2.1.6	Results from company E [9] (E///)

As part of the study a companion paper [2] discusses further the complications on testability for this feature when it comes to receiver demodulation and the required SINR.  Looking at the parameters, the higher order MCS (256 QAM MCS 25 and 27) are not presented here as initial results yielded little to no throughput.  Although, to keep ease in simulation time, HARQ was not applied and it may be possible to see more tangible throughput numbers.

The following results show the throughput performance at 256 QAM in a TDL-A channel

Figure 5.2.1.6-1: SCS 60 kHz, MCS 23

Comparably, when looking at 64 QAM throughput performance is better than 256 QAM when the expected EVM at transmitter and receiver is 5%.  The overall performance of 256 QAM at low SNR is rather sensitive to any added receiver and/or transmitter noise.  The fading channel conditions also provide some aspects to the results below.  Further simulations using HARQ could help this aspect; no link adaptation was simulated for this scenario.

Figure 5.2.1.6-2: 256 QAM and 64 QAM throughput performance comparison

The following are results simulated at 29 GHz with TDL-D channel model with 256 QAM link adaptation with CPE compensation.  The link adaptation adjusted different MCS specified in the simulation assumptions and HARQ of 8.  On the left is 64 QAM with 8% while figure on the right hand side is 256 QAM with 3% EVM.  To observe a gain in throughput towards a single user, a SNR of >22 dB is needed for the case of studying TLD-D channel conditions; a slightly lower expected SNR needed compared to that of TDL-A as shown in previous 2 figures.

Figure 5.2.1.6-3: 64 QAM and 256 QAM throughput performance comparison in LOS scenario (TDL-D)

[End of Text Proposal]