TSG-RAN Working Group 4 (Radio) meeting #92-bis	R4-1913008

Chongqing, China, 14th – 18th October 2019

Source:	Ericsson

Title:	TP to TR 38.820: PA indicative ACLR values in subclause 5.5.2

Agenda item:	10.4.6

Document for:	Approval

# Introduction

At last meeting in Ljubljana (RAN4#92), technical background information about essential PA characteristics was include in TR 38.820.

In this contribution we propose some conclusions based on measurement results and PA trends for indicative values for BS ACLR. At the end of this contribution a text proposal is attached for approval.

# Discussion

In [1], the complex dependencies between PA output power, ACLR and PAE was discussed in detail which is already captured in 7-24 GHz frequency range technical report. Some indicative and feasible values were also presented in [1]. Measurement results for three different commercially available power amplifiers around 10 GHz both with and without linearization for bandwidths of 200 MHz and 400 MHz was presented in [2] which confirmed the analysis and indicative values in [1].

The intention with this paper is to propose text to capture the indicative and feasible BS ACLR values for example frequencies for 7-24 GHz range.

# Conclusion

It is proposed that the attached text proposal is included in TR 38.820 [3].

# References

[1]	R4-1906180, “Further elaboration on 7-24 GHz PA dependencies for different example frequencies”, Ericsson

[2] 	R4-1908761, “Further elaboration on PA aspects for 10 GHz example frequency”, Ericsson

[3]	R4-1909280, “Draft TR 38.820”, Huawei

TEXT PROPOSAL:

### 5.5.2	Power amplifiers

In subclauses 5.4.1.1, 5.4.1.2, 5.4.2 and 5.4.3, the PA trends and saturated output power, PAE trends, AAS power scaling principles and PA dependencies respectively are described. The analysis resulted in example ACLR values without linearization of ~33 dB, ~31 dB and ~30 dB for example frequencies of 10, 15 and 20 GHz respectively. The example ACLR values for does not fully cater for the larger bandwidth likely for spectrum within 7-24 GHz frequency range and do not include the effects of any possible linearization applied to the transmitter chain which would have the effect of reducing ACLR

For 10 GHz example frequency, where an AAS BS may possibly re-use the existing deployment grid due to propagation condition it would benefit from higher PA power and high PAE. To investigate the potential improvements achievable from linearization three different PA’s based on GaN technology from 3 different vendors (vendor A, vendor B and vendor C) were investigated without and with linearization. The measurements were performed at ~8 GHz for practical reasons indicating that the measured results will be slightly worse for ~10 GHz.

Due to possible size of spectrum at higher frequencies, the bandwidth of 200 MHz and 400 MHz corresponding to 2 or 4 carriers, each 100 MHz was assumed.

The measurement results of the important characteristics are summarized in table 5.5.2-1.

Table 5.5.2-1: Measurement results

|  | Power (dBm) 200MHz/400MHz | PAE (%) 200MHz/400MHz | ACLR (dB) Without linearization 200MHz/400MHz | ACLR (dB) With linearization 200MHz/400MHz |
| --- | --- | --- | --- | --- |
| Vendor A | 35 | 11.7 / 11.5 | 36.2 / 35.5 | 42.1 / 39.8 |
| Vendor B | 37 / 36 (1 dB back off to reach EVM) | 12.7 / 12.3 | 26.4/ 21.6 | 38.9 / 29.8 |
| Vendor C | 33 | 16.8 / 17 | 32.7 / 23.2 | 42.8 / 24.3 |

The measured PAE for different PA’s are quite aligned with the PAE analysis in sub-clause 5.4.1.2. Note that the PAE needs to be further improved in the AAS BS context given the need for fairly high power at these frequencies.

The result also shows the impact of bandwidth where the achievable ACLR is lower for 400 MHz bandwidth (which is quite a reasonable bandwidth at these frequencies), it is not clear if this is a limitation of the linearizer the Tx chain or the PA.

Considering the need for a fairly high power and efficiency for these frequency ranges and the empirical analysis and measurement results in this paper the achievable ACLR value for 10 GHz example frequency considering a reasonable linearization is around 33 to 35 dB.