3GPP TSG-RAN WG4 Meeting #92Bis	R4-1910914

# Chongqing, China, 14th – 18th Oct., 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | - | rev | - | Current version: | 15.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on TS38.133 E-UTRAN – NR inter-RAT measurements test case with FR1 target cell and FR2 target cell (Section A.8.5.2) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Mediatek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Perf |  |  |  |  |  | Date: |  |  | 2019-10-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Modify the test cases to reduce the Io value (below -50dBm) |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Modify the Noc and corresponding value to reduce the Io to below -50dBm. - Delete the square bracket. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Io value exceeds the upper limit that the core reuqirement can cover. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.8.5.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

------------------------------------------------------- Beginning of Change ------------------------------------------------------------

### A.8.5.2	E-UTRA – NR Inter-RAT Measurement Performance requirements

#### A.8.5.2.1	SS-RSRP

##### A.8.5.2.1.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell

###### A.8.5.2.1.1.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in clause 9.11.1 in TS 36.133 [15] for inter-RAT FR1 SS-RSRP measurements.

###### A.8.5.2.1.1.2	Test Parameters

Supported test configurations are shown in Table A.8.5.2.1.1.2-1. In this test case there are two cells on different carriers. Cell 1 is the E-UTRA cell which specific test parameters for this test case are specified in Table A.3.7.2.1-1. Cell 2 is the inter-RAT NR FR1 target cell. The absolute accuracy requirements of SS-RSRP inter-RAT measurement is tested by using test parameters in Table A.8.5.2.1.1.2-2.

Table A.8.5.2.1.1.2-1: SS-RSRP Inter-RAT SS-RSRP supported test configurations

| Config | Description |
| --- | --- |
| 1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.8.5.2.1.1.2-2: SS-RSRP inter-RAT test parameters

| Parameter |  |  |  | Unit | Test 1 |  |  |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Cell 2 |  |  |  | Cell 2 |  |
| SSB ARFCN |  |  |  |  | freq1 |  |  |  | freq1 |  |
| Duplex mode |  |  | Config 1,4 |  | FDD |  |  |  |  |  |
|  |  |  | Config 2,3,5,6 |  | TDD |  |  |  |  |  |
| TDD configuration |  |  | Config 1,4 |  | Not Applicable |  |  |  |  |  |
|  |  |  | Config 2,5 |  | TDDConf.1.1 |  |  |  |  |  |
|  |  |  | Config 3,6 |  | TDDConf.2.1 |  |  |  |  |  |
| Downlink initial BWP configuration |  |  |  |  | DLBWP.0.1 |  |  |  |  |  |
| Downlink dedicated BWP configuration |  |  |  |  | DLBWP.1.1 |  |  |  |  |  |
| Uplink initial BWP configuration |  |  |  |  | ULBWP.0.1 |  |  |  |  |  |
| Uplink dedicated BWP configuration |  |  |  |  | ULBWP.1.1 |  |  |  |  |  |
| DRX Cycle configuration |  |  |  | ms | Not Applicable |  |  |  |  |  |
| TRS configuration |  | Config 1,4 |  |  | TRS.1.1 FDD |  |  |  |  |  |
|  |  | Config 2,5 |  |  | TRS.1.1 TDD |  |  |  |  |  |
|  |  | Config 3,6 |  |  | TRS.1.2 TDD |  |  |  |  |  |
| PDSCH Reference measurement channel |  |  | Config 1,4 |  | - |  |  |  | - |  |
|  |  |  | Config 2,5 |  |  |  |  |  |  |  |
|  |  |  | Config 3,6 |  |  |  |  |  |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1,4 |  | - |  |  |  | - |  |
|  |  |  | Config 2,5 |  |  |  |  |  |  |  |
|  |  |  | Config 3,6 |  |  |  |  |  |  |  |
| Dedicated CORESET Reference Channel |  |  | Config 1,4 |  | - |  |  |  | - |  |
|  |  |  | Config 2,5 |  |  |  |  |  |  |  |
|  |  |  | Config 3,6 |  |  |  |  |  |  |  |
| OCNG Patterns |  |  |  |  | OP.1 |  |  |  |  |  |
| SS-RSSI-Measurement |  |  |  |  | Not Applicable |  |  |  |  |  |
| SMTC configruation |  |  |  |  | SMTC.1 |  |  |  |  |  |
| SSB configuration |  |  | Config 1,2,4,5 |  | SSB.1 FR1 |  |  |  |  |  |
|  |  |  | Config 3,6 |  | SSB.2 FR1 |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  | Config 1,2,4,5 | kHz | 15 |  |  |  |  |  |
|  |  |  | Config 3,6 |  | 30 |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  | dB | 0 | 0 |  |  | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2,4,5 |  | NR_FDD_FR1_A NR_TDD_FR1_A | dBm/15kHz | -94.65 |  |  |  | -117 |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  | -116.5 |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  | -116 |  |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  | -115.5 |  |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  | -115 |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  | -114 |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  | -113.5 |  |
| Note2 | Config 1,2,4,5 |  |  | dBm/SCS | -94.65 |  |  |  | Same as Noc for 15kHz |  |
|  | Config 3,6 |  | NR_FDD_FR1_A NR_TDD_FR1_A |  | -91.65 |  |  |  | -114 |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  | -113.5 |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  | -113 |  |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  | -112.5 |  |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  | -112 |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  | -111 |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  | -110.5 |  |
|  |  |  |  | dB | 10 |  |  |  | -4 |  |
|  |  |  |  | dB | 10 |  |  |  | -4 |  |
| SS-RSRPNote3 | Config 1,2,4,5 |  | NR_FDD_FR1_A NR_TDD_FR1_A | dBm/SCS | -84.65 |  |  |  | -121 |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  | -120.5 |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  | -120 |  |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  | -119.5 |  |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  | -119 |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  | -118 |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  | -117.5 |  |
|  | Config 3,6 |  | NR_FDD_FR1_A NR_TDD_FR1_A |  | -81.65 |  |  |  | -124 |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  | -123.5 |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  | -123 |  |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  | -122.5 |  |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  | -122 |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  | -121 |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  | -120.5 |  |
| IoNote3 | Config 1,2,4,5 |  | NR_FDD_FR1_A NR_TDD_FR1_A | dBm/ 9.36MHz | -56.28 |  |  |  | -87.76 |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  | -87.26 |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  | -86.76 |  |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  | -86.26 |  |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  | -85.76 |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  | -84.76 |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  | -84.26 |  |
|  | Config 3,6 |  | NR_FDD_FR1_A NR_TDD_FR1_A | dBm/ 38.16MHz | -50.19 |  |  |  | -84.76 |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  | -84.26 |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  | -83.76 |  |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  | -83.26 |  |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  | -82.76 |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  | -81.76 |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  | -81.26 |  |
| Propagation condition |  |  |  | - | AWGN |  |  |  |  |  |
| Antenna configuration |  |  |  | - | 1x2 |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in clause 3.5.2. |  |  |  |  |  |  |  |  |  |  |

###### A.8.5.2.1.1.3	Test Requirements

The SS-RSRP measurement accuracy for Cell 2 shall fulfil the requirement in clause 9.11.1 in TS 36.133 [15].

##### A.8.5.2.1.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell

###### A.8.5.2.1.2.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRP measurement accuracy is within the specified limits. This test will verify the requirements in clause 9.11.1 in TS 36.133 [15] for inter-RAT FR2 SS-RSRP measurements.

###### A.8.5.2.1.2.2	Test Parameters

Supported test configurations are shown in Table A.8.5.2.1.2.2-1. In this test case there are two cells on different carriers. Both absolute accuracy and relative accuracy requirements of SS-RSRP inter-RAT measurement are tested by using test setup in Table A.8.5.2.1.2.2-2 and Table A.8.5.2.1.2.2-3. In all test cases, Cell 2 is target cell. Cell 1 is the E-UTRA cell which specific test parameters for this test case are specified in Table A.3.7.2.1-1.

Table A.8.5.2.1.2.2-1: SS-RSRP Inter-RAT SS-RSRP supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.8.5.2.1.2.2-2: SS-RSRP Inter-RAT general test parameters

| Parameter | Unit | Test 1 |  | Test 2 |
| --- | --- | --- | --- | --- |
|  |  | Cell 2 |  | Cell 2 |
| SSB ARFCN |  | Freq1 |  | freq1 |
| Duplex mode |  | TDD |  | TDD |
| TDD configuration |  | TDDConf.3.1 |  | TDDConf.3.1 |
| BWchannel | MHz | 100: NRB,c = 66 |  | 100: NRB,c = 66 |
| Downlink initial BWP configuration |  | DLBWP.0.1 |  |  |
| Downlink dedicated BWP configuration |  | DLBWP.1.1 |  |  |
| Uplink initial BWP configuration |  | ULBWP.0.1 |  |  |
| Uplink dedicated BWP configuration |  | ULBWP.1.1 |  |  |
| DRX cycle configuration | ms | Not applicable |  |  |
| TRS configuration |  | TRS.2.1 TDD |  |  |
| TCI state |  | TCI.State.0 |  |  |
| AoA setup |  | Setup 3 defined in A.3.15 |  |  |
| PDSCH Reference measurement channel |  | - |  | - |
| RMSI CORESET Reference Channel |  | - |  | - |
| OCNG Patterns |  | OP.1 |  | OP.1 |
| SMTC configuration |  | SMTC.1 FR2 |  | SMTC.1 FR2 |
| PDSCH/PDCCH subcarrier spacing | kHz | 120 |  | 120 |
| EPRE ratio of PSS to SSS | dB | 0 |  | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |
|  | dB | 10 |  | N/A |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |

Table A.8.5.2.1.2.2-3: SS-RSRP Inter-RAT OTA related test parameters

| Parameter |  | Unit | Test 1 |  | Test 2 |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 2 |  | Cell 2 |
| Angle of arrival configuration |  | degrees | Setup 1 according to A.3.15.1 |  | Setup 1 according to A.3.15.1 |
| Note1 | NR_TDD_FR2_A | dBm/15kHz Note4 | -100 |  | N/A |
|  | NR_TDD_FR2_B |  |  |  | N/A |
|  | NR_TDD_FR2_F |  |  |  | N/A |
|  | NR_TDD_FR2_G |  |  |  | N/A |
|  | NR_TDD_FR2_T |  |  |  | N/A |
|  | NR_TDD_FR2_Y |  |  |  | N/A |
| Note1 | NR_TDD_FR2_A | dBm/SCS Note3 | -96 |  | N/A |
|  | NR_TDD_FR2_B |  |  |  | N/A |
|  | NR_TDD_FR2_F |  |  |  | N/A |
|  | NR_TDD_FR2_G |  |  |  | N/A |
|  | NR_TDD_FR2_T |  |  |  | N/A |
|  | NR_TDD_FR2_Y |  |  |  | N/A |
| SS-RSRPNote2 | NR_TDD_FR2_A | dBm/SCS Note4 | -85 |  | Note7 |
|  | NR_TDD_FR2_B |  |  |  | Note7 |
|  | NR_TDD_FR2_F |  |  |  | Note7 |
|  | NR_TDD_FR2_G |  |  |  | Note7 |
|  | NR_TDD_FR2_T |  |  |  | Note7 |
|  | NR_TDD_FR2_Y |  |  |  | Note7 |
|  |  | dB | 11 |  | N/A |
| IoNote2 | NR_TDD_FR2_A | dBm/95.04 MHz Note4 | -55.4 |  | Note8 |
|  | NR_TDD_FR2_B |  |  |  | Note8 |
|  | NR_TDD_FR2_F |  |  |  | Note8 |
|  | NR_TDD_FR2_G |  |  |  | Note8 |
|  | NR_TDD_FR2_T |  |  |  | Note8 |
|  | NR_TDD_FR2_Y |  |  |  | Note8 |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 6: NR operating band groups are as defined in clause 3.5.2. Note 7: SS_RSRP is applied at level the same as the minimum level specified in Table B.2.3-2 for sphereical coverage. Note 8: Io is applied at level 10log10(792) dB above the minimum level specified in Table B.2.3-2 for sphereical coverage. |  |  |  |  |  |

###### A.8.5.2.1.2.3	Test Requirements

The SS-RSRP measurement accuracy for Cell 2 shall fulfil the requirement in clause 9.11.1 in TS 36.133 [15].

#### A.8.5.2.2	SS-RSRQ

##### A.8.5.2.2.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell

###### A.8.5.2.2.1.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in clause 9.11.2 in TS 36.133 [15] for inter-RAT FR1 SS-RSRQ measurements.

###### A.8.5.2.2.1.2	Test Parameters

Supported test configurations are shown in Table A.8.5.2.2.1.2-1. In this test case there are two cells on different carriers. Cell 1 is the E-UTRA cell which specific test parameters for this test case are specified in Table A.3.7.2.1-1. Cell 2 is the inter-RAT NR FR1 target cell. The absolute accuracy requirements of SS-RSRP inter-RAT measurement is tested by using test parameters in Table A.8.5.2.2.1.2-2.

Table A.8.5.2.2.1.2-1: SS-RSRQ Inter-RAT SS-RSRQ supported test configurations

| Config | Description |
| --- | --- |
| 1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | LTE FDD, NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6 | LTE TDD, NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.8.5.2.2.1.2-2: SS-RSRQ inter-RAT test parameters

| Parameter |  |  |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Cell 2 |  | Cell 2 |  | Cell 2 |  |
| SSB ARFCN |  |  |  |  | freq1 |  | freq1 |  | freq1 |  |
| Duplex mode |  |  | Config 1,4 |  | FDD |  |  |  |  |  |
|  |  |  | Config 2,3,5,6 |  | TDD |  |  |  |  |  |
| TDD configuration |  |  | Config 1,4 |  | Not Applicable |  |  |  |  |  |
|  |  |  | Config 2,5 |  | TDDConf.1.1 |  |  |  |  |  |
|  |  |  | Config 3,6 |  | TDDConf.2.1 |  |  |  |  |  |
| Downlink initial BWP configuration |  |  |  |  | DLBWP.0.1 |  |  |  |  |  |
| Downlink dedicated BWP configuration |  |  |  |  | DLBWP.1.1 |  |  |  |  |  |
| Uplink initial BWP configuration |  |  |  |  | ULBWP.0.1 |  |  |  |  |  |
| Uplink dedicated BWP configuration |  |  |  |  | ULBWP.1.1 |  |  |  |  |  |
| DRX Cycle configuration |  |  |  | ms | Not Applicable |  |  |  |  |  |
| TRS configuration |  | Config 1,4 |  |  | TRS.1.1 FDD |  |  |  |  |  |
|  |  | Config 2,5 |  |  | TRS.1.1 TDD |  |  |  |  |  |
|  |  | Config 3,6 |  |  | TRS.1.2 TDD |  |  |  |  |  |
| PDSCH Reference measurement channel |  |  | Config 1,4 |  | - |  | - |  | - |  |
|  |  |  | Config 2,5 |  |  |  |  |  |  |  |
|  |  |  | Config 3,6 |  |  |  |  |  |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1,4 |  | - |  | - |  | - |  |
|  |  |  | Config 2,5 |  |  |  |  |  |  |  |
|  |  |  | Config 3,6 |  |  |  |  |  |  |  |
| Dedicated CORESET Reference Channel |  |  | Config 1,4 |  | - |  | - |  | - |  |
|  |  |  | Config 2,5 |  |  |  |  |  |  |  |
|  |  |  | Config 3,6 |  |  |  |  |  |  |  |
| OCNG Patterns |  |  |  |  | OP.1 |  |  |  |  |  |
| SS-RSSI-Measurement |  |  |  |  | Not Applicable |  |  |  |  |  |
| SMTC configruation |  |  |  |  | SMTC.1 |  |  |  |  |  |
| SSB configuration |  |  | Config 1,2,4,5 |  | SSB.1 FR1 |  |  |  |  |  |
|  |  |  | Config 3,6 |  | SSB.2 FR1 |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  | Config 1,2,4,5 | kHz | 15 |  |  |  |  |  |
|  |  |  | Config 3,6 |  | 30 |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2,4,5 |  | NR_FDD_FR1_A NR_TDD_FR1_A | dBm/15kHz | -80.18 |  | -106 |  | -116 |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  | -115.5 |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  | -115 |  |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  | -114.5 |  |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  | -114 |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  | -113 |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  | -112.5 |  |
| Note2 | Config 1,2,4,5 |  |  | dBm/SCS | -80.18 |  | -106 |  | Same as Noc for 15kHz |  |
|  | Config 3,6 |  | NR_FDD_FR1_A NR_TDD_FR1_A |  | -83.27 |  | -110 |  | -113 |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  | -112.5 |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  | -112 |  |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  | -111.5 |  |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  | -111 |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  | -110 |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  | -109.5 |  |
|  |  |  |  | dB | -1.75 |  | -1.75 |  | -1.75 |  |
|  |  |  |  | dB | -1.75 |  | -1.75 |  | -1.75 |  |
| SS-RSRPNote3 | Config 1,2,4,5 |  | NR_FDD_FR1_A NR_TDD_FR1_A | dBm/SCS | -81.93 |  | -107.75 |  | -117.75 |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  | -117.25 |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  | -116.75 |  |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  | -116.25 |  |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  | -115.75 |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  | -114.75 |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  | -114.25 |  |
|  | Config 3,6 |  | NR_FDD_FR1_A NR_TDD_FR1_A |  | -85.02 |  | -111.75 |  | -114.75 |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  | -114.25 |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  | -113.75 |  |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  | -113.25 |  |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  | -112.75 |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  | -111.75 |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  | -111.25 |  |
| SS-RSRQ Note3 |  |  | NR_FDD_FR1_A NR_TDD_FR1_A | dB | -14.77 |  | -40.59 |  | -14.76 |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |
| IoNote3 | Config 1,2,4,5 |  | NR_FDD_FR1_A NR_TDD_FR1_A | dBm/ 9.36MHz | -50 |  | -75.83 |  | -85.83 |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  | -85.33 |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  | -84.83 |  |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  | -84.33 |  |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  | -83.83 |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  | -82.83 |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  | -82.33 |  |
|  | Config 3,6 |  | NR_FDD_FR1_A NR_TDD_FR1_A | dBm/ 38.16MHz | -50 |  | -76.73 |  | -79.73 |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  | -79.23 |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  | -78.73 |  |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  | -78.23 |  |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  | -77.73 |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  | -76.73 |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  | -76.53 |  |
| Propagation condition |  |  |  | - | AWGN |  |  |  |  |  |
| Antenna configuration |  |  |  | - | 1x2 |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRQ, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRQ minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in clause 3.5.2. |  |  |  |  |  |  |  |  |  |  |

###### A.8.5.2.2.1.3	Test Requirements

The SS-RSRQ measurement accuracy for Cell 2 shall fulfil the requirement in clause 9.11.2 in TS 36.133 [15].

##### A.8.5.2.2.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell

###### A.8.5.2.2.2.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in clause 9.11.2 in TS 36.133 [15] for inter-RAT FR2 SS-RSRQ measurements.

###### A.8.5.2.2.2.2	Test Parameters

Supported test configurations are shown in Table A.8.5.2.2.2.2-1. In this test case there are two cells on different carriers. Both absolute accuracy and relative accuracy requirements of SS-RSRQ inter-RAT measurement are tested by using test setup in Table A.8.5.2.2.2.2-2 and Table A.8.5.2.2.2.2-3. In all test cases, Cell 2 is target cell. Cell 1 is the E-UTRA cell which specific test parameters for this test case are specified in Table A.3.7.2.1-1.

Table A.8.5.2.2.2.2-1: SS-RSRQ Inter-RAT SS-RSRQ supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.8.5.2.2.2.2-2: SS-RSRQ Inter-RAT general test parameters

| Parameter | Unit | Test 1 |  | Test 2 |
| --- | --- | --- | --- | --- |
|  |  | Cell 2 |  | Cell 2 |
| SSB ARFCN |  | Freq1 |  | freq1 |
| Duplex mode |  | TDD |  | TDD |
| TDD configuration |  | TDDConf.3.1 |  | TDDConf.3.1 |
| BWchannel | MHz | 100: NRB,c = 66 |  | 100: NRB,c = 66 |
| Downlink initial BWP configuration |  | DLBWP.0.1 |  |  |
| Downlink dedicated BWP configuration |  | DLBWP.1.1 |  |  |
| Uplink initial BWP configuration |  | ULBWP.0.1 |  |  |
| Uplink dedicated BWP configuration |  | ULBWP.1.1 |  |  |
| DRX cycle configuration | ms | Not applicable |  |  |
| TRS configuration |  | TRS.2.1 TDD |  |  |
| TCI state |  | TCI.State.0 |  |  |
| AoA setup |  | Setup 3 defined in A.3.15 |  |  |
| PDSCH Reference measurement channel |  | - |  | - |
| RMSI CORESET Reference Channel |  | - |  | - |
| OCNG Patterns |  | OP.1 |  | OP.1 |
| SMTC configuration |  | SMTC.1 FR2 |  | SMTC.1 FR2 |
| PDSCH/PDCCH subcarrier spacing | kHz | 120 |  | 120 |
| EPRE ratio of PSS to SSS | dB | 0 |  | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |
|  | dB | -0.5 |  | -1.75 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-SINR, SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-SINR and SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |

Table A.8.5.2.2.2.2-3: SS-RSRQ Inter-RAT OTA related test parameters

| Parameter |  | Unit | Test 1 |  | Test 2 |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 2 |  | Cell 2 |
| Angle of arrival configuration |  | degrees | Setup 1 according to A.3.15.1 |  | Setup 1 according to A.3.15.1 |
| Note1 | NR_TDD_FR2_A | dBm/15kHzNote4 | -105 |  | Note7 |
|  | NR_TDD_FR2_B |  |  |  | Note7 |
|  | NR_TDD_FR2_F |  |  |  | Note7 |
|  | NR_TDD_FR2_G |  |  |  | Note7 |
|  | NR_TDD_FR2_T |  |  |  | Note7 |
|  | NR_TDD_FR2_Y |  |  |  | Note7 |
| Note1 | NR_TDD_FR2_A | dBm/SCSNote3 | -96 |  | Note7 |
|  | NR_TDD_FR2_B |  |  |  | Note7 |
|  | NR_TDD_FR2_F |  |  |  | Note7 |
|  | NR_TDD_FR2_G |  |  |  | Note7 |
|  | NR_TDD_FR2_T |  |  |  | Note7 |
|  | NR_TDD_FR2_Y |  |  |  | Note7 |
| SS-RSRPNote2 | NR_TDD_FR2_A | dBm/SCS Note4 | -96.5 |  | Note8 |
|  | NR_TDD_FR2_B |  |  |  | Note8 |
|  | NR_TDD_FR2_F |  |  |  | Note8 |
|  | NR_TDD_FR2_G |  |  |  | Note8 |
|  | NR_TDD_FR2_T |  |  |  | Note8 |
|  | NR_TDD_FR2_Y |  |  |  | Note8 |
| SS-RSRQNote2 | NR_TDD_FR2_A | dB | -14.4 |  | -14.82 |
|  | NR_TDD_FR2_B |  |  |  | -14.82 |
|  | NR_TDD_FR2_F |  |  |  | -14.82 |
|  | NR_TDD_FR2_G |  |  |  | -14.82 |
|  | NR_TDD_FR2_T |  |  |  | -14.82 |
|  | NR_TDD_FR2_Y |  |  |  | -14.82 |
|  |  | dB | -0.5 |  | -1.75 |
| IoNote2 | NR_TDD_FR2_A | dBm/95.04 MHz Note4 | -63.9 |  | Note 9 |
|  | NR_TDD_FR2_B |  |  |  | Note 9 |
|  | NR_TDD_FR2_F |  |  |  | Note 9 |
|  | NR_TDD_FR2_G |  |  |  | Note 9 |
|  | NR_TDD_FR2_T |  |  |  | Note 9 |
|  | NR_TDD_FR2_Y |  |  |  | Note 9 |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SS-RSRQ, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: SS-RSRQ and SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 6: NR operating band groups are as defined in clause 3.5.2. Note 7: Noc for SCS 15kHz is applied at level -10log10(8)+4dB above the minimum level specified in Table B.2.3-2 for sphereical coverage. Noc for SCS 120kHz is applied at 4 dB above the minimum level specified in Table B.2.3-2 for sphereical coverage. Note 8: SS_RSRP is applied at level 2.25dB above the minimum level specified in Table B.2.3-2 for sphereical coverage. Note 9: Io is applied at level 10log10(792)+6.22dB above the minimum level specified in Table B.2.3-2 for sphereical coverage. |  |  |  |  |  |

###### A.8.5.2.2.2.3	Test Requirements

The SS-RSRQ measurement accuracy for Cell 2 shall fulfil the requirement in clause 9.11.2 in TS 36.133 [15].

In this test case there are two cells on different carriers and measurement gaps are provided

#### A.8.5.2.3	SS-SINR

##### A.8.5.2.3.1	E-UTRAN – NR inter-RAT measurements with FR1 target cell

###### A.8.5.2.3.1.1	Test Purpose and Environment

The purpose of this test is to verify that the SS- SINR measurement accuracy is within the specified limits. This test will verify the requirements in clause 9.11.3 in TS 36.133 [15] for inter-RAT FR1 SS-SINR measurements.

###### A.8.5.2.3.1.2	Test Parameters

Supported test configurations are shown in Table A.8.5.2.3.1.2-1. In this test case there are two cells on different carriers. Cell 1 is the E-UTRA cell which specific test parameters for this test case are specified in Table A.3.7.2.1-1. Cell 2 is the inter-RAT NR FR1 target cell. The absolute accuracy requirements of SS-RSRP inter-RAT measurement is tested by using test parameters in Table A.8.5.2.3.1.2-2.

Table A.8.5.2.3.1.2-1: SS- SINR Inter-RAT SS- SINR supported test configurations

| Config | Description |
| --- | --- |
| 1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | LTE FDD, NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 6 | LTE TDD, NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.8.5.2.3.1.2-2: SS-SINR inter-RAT test parameters

| Parameter |  |  |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | Cell 2 |  | Cell 2 |  | Cell 2 |  |
| SSB ARFCN |  |  |  |  | freq1 |  | freq1 |  | freq1 |  |
| Duplex mode |  |  | Config 1,4 |  | FDD |  |  |  |  |  |
|  |  |  | Config 2,3,5,6 |  | TDD |  |  |  |  |  |
| TDD configuration |  |  | Config 1,4 |  | Not Applicable |  |  |  |  |  |
|  |  |  | Config 2,5 |  | TDDConf.1.1 |  |  |  |  |  |
|  |  |  | Config 3,6 |  | TDDConf.2.1 |  |  |  |  |  |
| Downlink initial BWP configuration |  |  |  |  | DLBWP.0.1 |  |  |  |  |  |
| Downlink dedicated BWP configuration |  |  |  |  | DLBWP.1.1 |  |  |  |  |  |
| Uplink initial BWP configuration |  |  |  |  | ULBWP.0.1 |  |  |  |  |  |
| Uplink dedicated BWP configuration |  |  |  |  | ULBWP.1.1 |  |  |  |  |  |
| DRX Cycle configuration |  |  |  | ms | Not Applicable |  |  |  |  |  |
| TRS configuration |  | Config 1,4 |  |  | TRS.1.1 FDD |  |  |  |  |  |
|  |  | Config 2,5 |  |  | TRS.1.1 TDD |  |  |  |  |  |
|  |  | Config 3,6 |  |  | TRS.1.2 TDD |  |  |  |  |  |
| PDSCH Reference measurement channel |  |  | Config 1,4 |  | - |  | - |  | - |  |
|  |  |  | Config 2,5 |  |  |  |  |  |  |  |
|  |  |  | Config 3,6 |  |  |  |  |  |  |  |
| RMSI CORESET Reference Channel |  |  | Config 1,4 |  | - |  | - |  | - |  |
|  |  |  | Config 2,5 |  |  |  |  |  |  |  |
|  |  |  | Config 3,6 |  |  |  |  |  |  |  |
| Dedicated CORESET Reference Channel |  |  | Config 1,4 |  | - |  | - |  | - |  |
|  |  |  | Config 2,5 |  |  |  |  |  |  |  |
|  |  |  | Config 3,6 |  |  |  |  |  |  |  |
| OCNG Patterns |  |  |  |  | OP.1 |  |  |  |  |  |
| SS-RSSI-Measurement |  |  |  |  | Not Applicable |  |  |  |  |  |
| SMTC configruation |  |  |  |  | SMTC.1 |  |  |  |  |  |
| SSB configuration |  |  | Config 1,2,4,5 |  | SSB.1 FR1 |  |  |  |  |  |
|  |  |  | Config 3,6 |  | SSB.2 FR1 |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  | Config 1,2,4,5 | kHz | 15 |  |  |  |  |  |
|  |  |  | Config 3,6 |  | 30 |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2,4,5 |  | NR_FDD_FR1_A NR_TDD_FR1_A | dBm/15kHz | -88 |  | -108.5 |  | -119.5 |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  | -119 |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  | -118.5 |  |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  | -118 |  |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  | -117.5 |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  | -116.5 |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  | -116 |  |
| Note2 | Config 1,2,4,5 |  |  | dBm/SCS | -88 |  | -108.5 |  | Same as Noc for 15kHz |  |
|  | Config 3,6 |  | NR_FDD_FR1_A NR_TDD_FR1_A |  | -85 |  | -105.5 |  | -116.5 |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  | -116 |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  | -115.5 |  |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  | -115 |  |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  | -114.5 |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  | -114.5 |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  | -113 |  |
|  |  |  |  | dB | -1.75 |  | 20 |  | -4.0 |  |
|  |  |  |  | dB | -1.75 |  | 20 |  | -4.0 |  |
| SS-RSRPNote3 | Config 1,2,4,5 |  | NR_FDD_FR1_A NR_TDD_FR1_A | dBm/SCS | -89.75 |  | -88.5 |  | -123.5 |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  | -123 |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  | -122.5 |  |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  | -122 |  |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  | -121.5 |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  | -120.5 |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  | -120 |  |
|  | Config 3,6 |  | NR_FDD_FR1_A NR_TDD_FR1_A |  | -86.75 |  | -85.5 |  | -120.5 |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  | -120 |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  | -119.5 |  |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  | -119 |  |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  | -118.5 |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  | -117.5 |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  | -117 |  |
| SS-SINR Note3 |  |  | NR_FDD_FR1_A NR_TDD_FR1_A | dB | -1.75 |  | 20 |  | -4.0 |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  |  |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  |  |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  |  |  |
| IoNote3 | Config 1,2,4,5 |  | NR_FDD_FR1_A NR_TDD_FR1_A | dBm/ 9.36MHz | -57.83 |  | -60.5 |  | -90.09 |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  | -89.59 |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  | -89.09 |  |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  | -88.59 |  |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  | -88.09 |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  | -87.09 |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  | -86.59 |  |
|  | Config 3,6 |  | NR_FDD_FR1_A NR_TDD_FR1_A | dBm/ 38.16MHz | -51.73 |  | -54.41 |  | -84 |  |
|  |  |  | NR_FDD_FR1_B |  |  |  |  |  | -83.5 |  |
|  |  |  | NR_TDD_FR1_C |  |  |  |  |  | -83 |  |
|  |  |  | NR_FDD_FR1_D NR_TDD_FR1_D |  |  |  |  |  | -82.5 |  |
|  |  |  | NR_FDD_FR1_E NR_TDD_FR1_E |  |  |  |  |  | -82 |  |
|  |  |  | NR_FDD_FR1_G |  |  |  |  |  | -81 |  |
|  |  |  | NR_FDD_FR1_H |  |  |  |  |  | -80.5 |  |
| Propagation condition |  |  |  | - | AWGN |  |  |  |  |  |
| Antenna configuration |  |  |  | - | 1x2 |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-SINR, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-SINR, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in clause 3.5.2. |  |  |  |  |  |  |  |  |  |  |

###### A.8.5.2.3.1.3	Test Requirements

The SS-SINR measurement accuracy for Cell 2 shall fulfil the requirement in clause 9.11.3 in TS 36.133 [15].

##### A.8.5.2.3.2	E-UTRAN – NR inter-RAT measurements with FR2 target cell

###### A.8.5.2.3.2.1	Test Purpose and Environment

The purpose of this test is to verify that the SS- SINR measurement accuracy is within the specified limits. This test will verify the requirements in clause 9.11.3 in TS 36.133 [15] for inter-RAT FR2 SS-SINR measurements.

###### A.8.5.2.3.2.2	Test Parameters

Supported test configurations are shown in Table A.8.5.2.3.2.2-1. In this test case there are two cells on different carriers. Both absolute accuracy and relative accuracy requirements of SS-SINR inter-RAT measurement are tested by using test setup in Table A.8.5.2.3.2.2-2 and A.8.5.2.3.2.2-3. In all test cases, Cell 2 is target cell. Cell 1 is the E-UTRA cell which specific test parameters for this test case are specified in Table A.3.7.2.1-1.

Table A.8.5.2.3.2.2-1: SS-SINR Inter-RAT SS-SINR supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.8.5.2.3.2.2-2: SS-SINR Inter-RAT general test parameters

| Parameter | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- |
|  |  | Cell 2 | Cell 2 | Cell 2 |
| SSB ARFCN |  | Freq1 | freq1 | freq1 |
| Duplex mode |  | TDD | TDD | TDD |
| TDD configuration |  | TDDConf.3.1 | TDDConf.3.1 | TDDConf.3.1 |
| BWchannel | MHz | 100: NRB,c = 66 | 100: NRB,c = 66 | 100: NRB,c = 66 |
| Downlink initial BWP configuration |  | DLBWP.0.1 |  |  |
| Downlink dedicated BWP configuration |  | DLBWP.1.1 |  |  |
| Uplink initial BWP configuration |  | ULBWP.0.1 |  |  |
| Uplink dedicated BWP configuration |  | ULBWP.1.1 |  |  |
| DRX cycle configuration | ms | Not applicable |  |  |
| TRS configuration |  | TRS.2.1 TDD |  |  |
| TCI state |  | TCI.State.0 |  |  |
| AoA setup |  | Setup 3 defined in A.3.15 |  |  |
| PDSCH Reference measurement channel |  | - | - | - |
| RMSI CORESET Reference Channel |  | - | - | - |
| OCNG Patterns |  | OP.1 | OP.1 | OP.1 |
| SMTC configuration |  | SMTC.1 FR2 | SMTC.1 FR2 | SMTC.1 FR2 |
| PDSCH/PDCCH subcarrier spacing | kHz | 120 | 120 | 120 |
| EPRE ratio of PSS to SSS | dB | 0 | 0 | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |
|  | dB | -0.5 | 11.0 | -3.0 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-SINR, SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-SINR and SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |

Table A.8.5.2.3.2.2-3: SS-SINR Inter-RAT OTA related test parameters

| Parameter |  | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 2 | Cell 2 | Cell 2 |
| Angle of arrival configuration |  | degrees | Setup 1 according to A.3.15.1 | Setup 1 according to A.3.15.1 | Setup 1 according to A.3.15.1 |
| Note1 | NR_TDD_FR2_A | dBm/15kHz Note4 | -105 | -105 | Note7 |
|  | NR_TDD_FR2_B |  |  |  | Note7 |
|  | NR_TDD_FR2_F |  |  |  | Note7 |
|  | NR_TDD_FR2_G |  |  |  | Note7 |
|  | NR_TDD_FR2_T |  |  |  | Note7 |
|  | NR_TDD_FR2_Y |  |  |  | Note7 |
| Note1 | NR_TDD_FR2_A | dBm/SCS Note3 | -96 | -96 | Note7 |
|  | NR_TDD_FR2_B |  |  |  | Note7 |
|  | NR_TDD_FR2_F |  |  |  | Note7 |
|  | NR_TDD_FR2_G |  |  |  | Note7 |
|  | NR_TDD_FR2_T |  |  |  | Note7 |
|  | NR_TDD_FR2_Y |  |  |  | Note7 |
| SS-RSRPNote2 | NR_TDD_FR2_A | dBm/SCS Note4 | -96.5 | -85 | Note8 |
|  | NR_TDD_FR2_B |  |  |  | Note8 |
|  | NR_TDD_FR2_F |  |  |  | Note8 |
|  | NR_TDD_FR2_G |  |  |  | Note8 |
|  | NR_TDD_FR2_T |  |  |  | Note8 |
|  | NR_TDD_FR2_Y |  |  |  | Note8 |
| SS-SINRNote2 | NR_TDD_FR2_A | dB | -0.5 | 11 | -3.0 |
|  | NR_TDD_FR2_B |  |  |  | -3.0 |
|  | NR_TDD_FR2_F |  |  |  | -3.0 |
|  | NR_TDD_FR2_G |  |  |  | -3.0 |
|  | NR_TDD_FR2_T |  |  |  | -3.0 |
|  | NR_TDD_FR2_Y |  |  |  | -3.0 |
|  |  | dB | -0.5 | 11 | -3.0 |
| IoNote2 | NR_TDD_FR2_A | dBm/95.04 MHz Note4 | -69.3 | -55.4 | Note9 |
|  | NR_TDD_FR2_B |  |  |  | Note9 |
|  | NR_TDD_FR2_F |  |  |  | Note9 |
|  | NR_TDD_FR2_G |  |  |  | Note9 |
|  | NR_TDD_FR2_T |  |  |  | Note9 |
|  | NR_TDD_FR2_Y |  |  |  | Note9 |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SS-SINR, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: SS-SINR and SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: As observed with 0 dBi gain antenna at the centre of the quiet zone Note 6: NR operating band groups are as defined in clause 3.5.2. Note 7: Noc for SCS 15kHz is applied at level -10log10(8)+4dB above the minimum level specified in Table B.2.3-2 for sphereical coverage. Noc for SCS 120kHz is applied at 4 dB above the minimum level specified in Table B.2.3-2 for sphereical coverage. Note 8: SS_RSRP is applied at level 3dB above the minimum level specified in Table B.2.3-2 for sphereical coverage. Note 9: Io is applied at level 10log10(792)+6.54dB above the minimum level specified in Table B.2.3-2 for sphereical coverage. |  |  |  |  |  |

###### A.8.5.2.3.2.3	Test Requirements

The SS-SINR measurement accuracy for Cell 2 shall fulfil the requirement in clause 9.11.3 in TS 36.133 [15].

----------------------------------------------------------- End of Change ----------------------------------------------------------------