3GPP TSG-RAN WG4 Meeting #92bis	R4-1911069

# Chongqing, China, October 14th– 18th 2019

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 6641 | rev | - | Current version: | 14.13.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to misused variables in intra-frequency cell identification requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI13 |  |  |  |  |  | Date: |  |  | 2019-09-25 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | Some variables in Table 8.11.2.1.1.2-1 are wrong. M is used to count unavailable occasions during Tmeasure_intra_FS3_CRS_DRX, not Tidentify_intra_FS3_DRX. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Correct the variables in Table 8.11.2.1.1.2-1 from M to L. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification remains wrong. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.11.2.1.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 36.521-1 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

| This CR's revision history: |  |
| --- | --- |

<start of the change>

8.11.2.1.1.2	Requirements when DRX is used

When DRX is in use the UE shall be able to identify a new detectable FS3 intra-frequency cell within the cell identification time Tidentify_intra_FS3_DRX, where the cell identification time of a cell shall include detection of the cell and additionally performing a single measurement with measurement period of Tmeasure_intra_FS3_CRS_DRX, where:.

Tidentify_intra_FS3_DRX is the intra-frequency period for cell identification as shown in Table 8.11.2.1.1.2-1,

Tmeasure_intra_FS3_CRS_DRX is the intra-frequency period for measurements as shown in Table 8.11.2.1.1.2-2,

TDMTC_periodicity is the discovery signal measurement timing configuration periodicity of higher layer,

L is the number of configured discovery signal occasions during ON DURATION and which are not available during Tidentify_intra_FS3_DRX for cell identification at the UE due to the absence of the necessary radio signals from the cell or due to the corresponding downlink subframe being configured as an uplink subframe,

M is the number of configured discovery signal occasions during ON DURATION and which are not available during Tmeasure_intra_FS3_CRS_DRX for the measurements at the UE due to the absence of the necessary radio signals from the cell or due to the corresponding downlink subframe being configured as an uplink subframe.

Table 8.11.2.1.1.2-1: Intra-frequency cell identification requirements under operation with frame structure 3

| SCH Ês/Iot | CRS measurement bandwidth [RB] Note2 | CRS Ês/Iot | Tidentify_intra_FS3_DRX [ms] |
| --- | --- | --- | --- |
| 0 ≤ SCH Ês/Iot | <25 | -6 ≤ CRS Ês/Iot | (6+L)* k1*k2*Max{TDMTC_periodicity, DRX cycle length} |
| -6 ≤ SCH Ês/Iot < 0 | <25 |  | (24+L)* k1*k2*Max{TDMTC_periodicity, DRX cycle length} |
| 0 ≤ SCH Ês/Iot | 25 | 0 ≤ CRS Ês/Iot | (2+L)* k1*k2*Max{TDMTC_periodicity, DRX cycle length} |
| -6 ≤ SCH Ês/Iot < 0 | 25 |  | (8+L)* k1*k2*Max{TDMTC_periodicity, DRX cycle length} |
| NOTE 1: Discovery signal occasion duration (ds-OccasionDuration) is 1 ms. NOTE 2: The requirements for measurement bandwidth ≥25 RB are optional. NOTE 3: k1=2 when the measurement gaps configured for inter-frequency measurements in DMTC occasions or for inter-frequency RSSI measurements in RMTC occasions on a carrier with frame structure 3 overlap with some but not all DMTC occasions of the measured cell during ON DURATION; otherwise, k1=1, e.g., when measurement gaps configured for inter-frequency measurements on a carrier with frame structure 3 do not overlap with DMTC occasions of the measured cell during ON DURATION or when the UE does not require the measurement gaps for the inter-frequency measurements. The requirements apply, provided that the inter-frequency measurement gap pattern does not overlap with all DMTC occasions of the measured cell. k2= when DMTC occasions in the measured cell not overlapping with the inter-frequency measurement gaps measurement gaps overlap with DMTC occasions of NFS3_SCC (NFS3_SCC>0) SCells during ON DURATION on other FS3 carriers; otherwise, k2=1, e.g., when NFS3_SCC is 0. NOTE 4: The requirements apply, provided that L is such that the intra-frequency cell identification period Tidentify_intra_FS3_DRX does not exceed 72*k1*k2* Max{TDMTC_periodicity, DRX cycle length}. |  |  |  |

A cell shall be considered detectable when the following conditions are met during the discovery signal occasions which are available during Tidentify_intra_FS3_DRX:

-	RSRP related side conditions given in Section 9.1.18.2 are fulfilled for a corresponding Band,

-	RSRQ related side conditions given in Section 9.1.18.3 are fulfilled for a corresponding Band,

-	SCH_RP is according to Annex B.2.12 for a corresponding Band and SCH Ês/Iot is according to Table 8.11.2.1.1.2-1.

If higher layer filtering is used, an additional cell identification delay can be expected.

In the RRC_CONNECTED state the measurement period for intra-frequency measurements is Tmeasure_ intra_FS3_CRS_DRX as shown in Table 8.11.2.1.1.2-2, when DRX is in use. The UE shall be capable of p performing RSRP and RSRQ measurements for 3 identified intra-frequency cells, and the UE physical layer shall be capable of reporting measurements to higher layers within the measurement period of Tmeasure_intra_FS3_CRS_DRX.

<end of the change>