3GPP TSG-RAN WG4 Meeting #92-Bis                 	R4-1911192

Chongqing, China, 14th – 18th October 2019

Source:	Nokia, Nokia Shanghai Bell

Title:	NR Rel-16 HST BS demodulation PUSCH simulation results

Agenda item:	8.17.2.3

Document for:	Discussion

Introduction

In the RAN4#92 meeting, a plethora of possible configurations and environments for NR_HST BS demodulation PUSCH performance requirements were proposed [1].

Main clusters of parameterization options are found around, maximum doppler shift, channel modelling, waveform (including CBW, SCS, MCS, etc), as well as reference signal configurations, TDRA, and test metrics.

In this contribution we provide the results of our simulation campaign on possible performance requirement parametrization in PUSCH.
These results will be discussed in our companion discussion paper on remaining NR_HST Rel-16 BS demodulation PUSCH performance requirements.

PUSCH Simulation results

Unless otherwise stated in the following tables, the simulation setup follows the standard non-HST PUSCH setup, as captured in [2] with the modifications required by the RAN4#92 HST way forward [1], in particular for the PUSCH symbol allocation, type A with S=0 and L=14, was used.

2.1GHz/350kph, CP-OFDM, 1T2R, Type A TDRA

Table 1: PUSCH, 2.1GHz/350kph, CP-OFDM, 1T2R, Type A TDRA: simulation setup and results summary

| ID | Propagation conditions | BW/SCS | DM-RS | PT-RS | MCS | SNR@30% TPUT | SNR@70% TPUT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2001 | Single Tap-Scen3-1340Hz | 10MHz/15kHz | 1+1+1 l0=2 | OFF | 2 | -9,52 | -5,93 |
| 2002 | Single Tap-Scen3-1340Hz | 10MHz/15kHz | 1+1+1 l0=2 | OFF | 16 | -0,72 | 5,75 |
| 2003 | Single Tap-Scen3-1340Hz | 10MHz/15kHz | 1+1, l0=2 | K=2 L=1 | 2 | -8,68 | -5,45 |
| 2004 | Single Tap-Scen3-1340Hz | 10MHz/15kHz | 1+1, l0=2 | K=2 L=1 | 16 | -0,37 | 6,68 |
| 2005 | Single Tap-Scen1-1340Hz-option1 | 10MHz/15kHz | 1+1+1, l0=2 | OFF | 2 | -9,58 | -5,97 |
| 2006 | Single Tap-Scen1-1340Hz-option1 | 10MHz/15kHz | 1+1+1, l0=2 | OFF | 16 | -0,76 | 5,74 |
| 2007 | Single Tap-Scen1-1340Hz-option1 | 10MHz/15kHz | 1+1, l0=2 | K=2 L=1 | 2 | -8,68 | -5,47 |
| 2008 | Single Tap-Scen1-1340Hz-option1 | 10MHz/15kHz | 1+1, l0=2 | K=2 L=1 | 16 | -0,43 | 6,62 |
| 2009 | Single Tap-Scen3-2334Hz | 10MHz/30kHz | 1+1+1, l0=2 | OFF | 2 | -9,40 | -5,91 |
| 2010 | Single Tap-Scen3-2334Hz | 10MHz/30kHz | 1+1+1, l0=2 | OFF | 16 | -0,79 | 5,72 |
| 2011 | Single Tap-Scen3-2334Hz | 10MHz/30kHz | 1+1, l0=2 | K=2 L=1 | 2 | -7,85 | -5,01 |
| 2012 | Single Tap-Scen3-2334Hz | 10MHz/30kHz | 1+1, l0=2 | K=2 L=1 | 16 | -0,19 | 6,71 |
| 2013 | Single Tap-Scen1-2334Hz-option1 | 10MHz/30kHz | 1+1+1, l0=2 | OFF | 2 | -9,48 | -5,94 |
| 2014 | Single Tap-Scen1-2334Hz-option1 | 10MHz/30kHz | 1+1+1, l0=2 | OFF | 16 | -0,87 | 5,71 |
| 2015 | Single Tap-Scen1-2334Hz-option1 | 10MHz/30kHz | 1+1, l0=2 | K=2 L=1 | 2 | -7,86 | -5,02 |
| 2016 | Single Tap-Scen1-2334Hz-option1 | 10MHz/30kHz | 1+1, l0=2 | K=2 L=1 | 16 | -0,20 | 6,70 |
| 2019 | Single Tap-Scen3-2334Hz | 40MHz/30kHz | 1+1+1, l0=2 | OFF | 2 | -9,57 | -5,85 |
| 2020 | Single Tap-Scen3-2334Hz | 40MHz/30kHz | 1+1+1, l0=2 | OFF | 16 | -0,67 | 5,75 |
| 2021 | Single Tap-Scen3-1340Hz | 10MHz/15kHz | 1+1+1 l0=3 | OFF | 2 | -9,55 | -5,94 |
| 2022 | Single Tap-Scen3-1340Hz | 10MHz/15kHz | 1+1+1 l0=3 | OFF | 16 | -0,72 | 5,75 |
| 2031 | TDLC-<DelSp>300-<DopSp>100 <FO>2334Hz | 10MHz/30kHz | 1+1+1 l0=2 | OFF | 2 | -8,87 | -5,02 |
| 2032 | TDLC-<DelSp>300-<DopSp>100 <FO>2334Hz | 10MHz/30kHz | 1+1+1, l0=2 | OFF | 16 | 0,42 | 7,48 |
| 2035 | AWGN-FO=1340Hz | 10MHz/15kHz | 1+1+1 l0=2 | OFF | 2 | -9,55 | -5,95 |
| 2036 | AWGN-FO=1340Hz | 10MHz/15kHz | 1+1+1, l0=2 | OFF | 16 | -0,75 | 5,75 |
| 2037 | AWGN-FO=1340Hz | 10MHz/15kHz | 1+1, l0=2 | K=2 L=1 | 2 | -8,68 | -5,46 |
| 2038 | AWGN-FO=1340Hz | 10MHz/15kHz | 1+1, l0=2 | K=2 L=1 | 16 | -0,39 | 6,68 |

Figure 1: Details [2001, 2002; 2003, 2004].

Figure 2: Details [2005, 2006; 2007, 2008].

Figure 3: Details [2009, 2010; 2011, 2012].

Figure 4: Details [2013, 20014; 20015, 20016].

Figure 5: Details [2019, 2020].

Figure 6: Details [2021, 2022].

Figure 7: Details [2031, 2032].

Figure 8: Details [2035, 2036; 2037, 2038].

2.1GHz/350kph, DFT-s-OFDM, 1T2R, Type A TDRA

Table 2: PUSCH, 2.1GHz/350kph, DFT-s-OFDM, 1T2R, Type A TDRA: simulation setup and results summary

| ID | Propagation conditions | BW/SCS | DM-RS | PT-RS | MCS | SNR@30% TPUT | SNR@70% TPUT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2017 | Single Tap-Scen3-1340Hz | 10MHz/15kHz | 1+1+1 l0=2 | OFF | 2 | -9,90 | -6,33 |
| 2018 | Single Tap-Scen3-1340Hz | 10MHz/15kHz | 1+1+1 l0=2 | OFF | 16 | -0,88 | 5,79 |

Figure 5: Details [2017, 2018].

3.6GHz/500kph, CP-OFDM, 1T2R, Type A TDRA

Table 3: PUSCH, 3.6GHz/500kph, CP-OFDM, 1T2R, Type A TDRA: simulation setup and results summary

| ID | Propagation condition | BW/SCS | DM-RS | PT-RS | MCS | SNR@30% TPUT | SNR@70% TPUT |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 2023 | Single Tap-Scen3-3334Hz | 10MHz/15kHz | 1+1+1, l0=2 | OFF | 2 | NaN | NaN |
| 2024 | Single Tap-Scen3-3334Hz | 10MHz/15kHz | 1+1+1, l0=2 | OFF | 16 | NaN | NaN |
| 2025 | Single Tap-Scen3-3334Hz | 10MHz/15kHz | 1+1, l0=2 | K=2 L=1 | 2 | -8,57 | -5,28 |
| 2026 | Single Tap-Scen3-3334Hz | 10MHz/15kHz | 1+1, l0=2 | K=2 L=1 | 16 | -0,16 | 7,91 |
| 2027 | Single Tap-Scen3-1944Hz | 10MHz/15kHz | 1+1+1, l0=2 | OFF | 2 | NaN | NaN |
| 2028 | Single Tap-Scen3-1944Hz | 10MHz/15kHz | 1+1+1, l0=2 | OFF | 16 | NaN | NaN |
| 2029 | Single Tap-Scen3-1944Hz | 10MHz/15kHz | 1+1, l0=2 | K=2 L=1 | 2 | -8,65 | -5,39 |
| 2030 | Single Tap-Scen3-1944Hz | 10MHz/15kHz | 1+1, l0=2 | K=2 L=1 | 16 | -0,23 | 6,79 |
| 2033 | Single Tap-Scen3-3334Hz | 10MHz/15kHz | 2+2, l0=2 | OFF | 2 | NaN | NaN |
| 2034 | Single Tap-Scen3-3334Hz | 10MHz/15kHz | 2+2, l0=2 | OFF | 16 | NaN | NaN |

Figure 9: Details [2023, 2024; 2025, 2026].

Figure 10: Details [2027, 2028; 2029, 2030].

Figure 11: Details [2033, 2034].

Conclusion

In this contribution we provided the results of our simulation campaign on possible performance requirement parametrization in PUSCH for NR_HST Rel-16 BS demodulation performance requirements.
These results are discussed in detail in our companion discussion paper.

- References

- R4-1910128, Way forward on NR HST BS demodulation requirements, 3GPP TSG-RAN WG4 Meeting #92, Huawei, HiSilicon.

- R4-1907241, WF on NR PUSCH demodulation requirements, Nokia, Nokia Shanghai Bell, RAN4#91.