3GPP TSG-RAN WG4 Meeting #92-Bis                                       R4-1911319

Chongqing, China, 14th – 18th Oct, 2019

Agenda Item:	6.11.4.4

Source:	MediaTek Inc.

Title:	Discussion on test case for CSI-RS RLM

Document for:		Discussion

- Introduction

Some observations are provided for the current test cases for CSI-RS RLM.

- Discussion

One confliction on power setting is observed. The CSI-RS for RLM is configured as Resource #4 in TRS (as highlighted in Table 2) and its EPRE ratio to SSS is set to -3dB in TRS RMC (as highlighted in Table 1). SNR on other channels and signals (e.g. SSB) is set to 2 dB as shown in Table 3.

However, SNR level of the CSI-RS for RLM will be changed during the test. Therefore, it is not possible to have a fixed EPRE ratio to SSS and a fixed SNR on SSB simultaneously.

Observation 1: It is not possible to have a fixed EPRE ratio to SSS and a fixed SNR on SSB simultaneously.

Proposal 1: Clarification is required for the power setting of CSI-RS for RLM.

Table 1: Table A.3.17.2.1-1: CSI-RS for tracking for SCS=120kHz Set 1

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | TRS.2.1 TDD |
| Bandwidth |  | BW of Active BWPNote |
| SCS | kHz | 120 |
| ….. |  |  |
| EPRE ratio to SSS | dB | -3 |
| TCI state |  | TCI.State.0 |

Table 2: Table A.5.5.1.5.1-2: General test parameters for FR2 PSCell for CSI-RS out-of-sync testing in non-DRX mode

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
|  |  |  | Test 1 |
| …… |  |  |  |
| RMC CORESET Reference Channel | Config 1 |  | CCR.3.1 TDD CCR.3.2 TDD |
|  | Config 2 |  | CCR.3.1 TDD CCR.3.2 TDD |
| …… |  |  |  |
| CSI-RS for RLM | Config 1, 2 |  | Resource #4 in TRS.2.1 TDD Resource #4 in TRS.2.2 TDD |
| TRS configuration |  |  | TRS.2.1 TDD TRS.2.2 TDD |
| TCI configuration for PDCCH#1/PDSCH |  |  | TCI.State.2 |
| TCI configuration for PDCCH#2 |  |  | TCI.State.3 |
| …… |  |  |  |
| CSI-RS for CSI reporting | Config 1 |  | CSI-RS.3.1 TDD |
|  | Config 2 |  | CSI-RS.3.1 TDD |
| …… |  |  |  |

Table 3: Table A.5.5.1.5.1-3: Cell specific test parameters for FR2 for CSI-RS out-of-sync radio link monitoring in non-DRX mode

| Parameter |  | Unit | Test 1 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |
| …… |  |  |  |  |  |
| SNR on other channels and signals | Config 1, 2 | dB | 2 |  |  |
| …… |  |  |  |  |  |

Another observation is about the RMC CORESET configuration in Table 2. It seems redundant to provide 2 values for one CORESET configuration (highlighted in yellow), so the CCR.3.2 should be removed.

Proposal 2: Remove CCR.3.2 in RMC CORESET configuration.

It can also observe that the configuration of TCI state for CORESET is still missing for some FR1 RLM tests (e.g. A.4.5.1.5). In last meeting, it has been agreed to add TCI state for CORESET in FR1 RLM tests [2], since UE is only required to perform CSI-RS RLM when the CORESET is QCLed to the CSI-RS resource.

Observation 2: TCI state for CORESET is missing for some FR1 RLM tests.

Proposal 3: To add TCI state for CORESET for FR1 RLM tests.

- Summary

In this paper, some observations are provided for the current test cases for CSI-RS RLM. We have the following observations and proposals:

Observation 1: It is not possible to have a fixed EPRE ratio to SSS and a fixed SNR on SSB simultaneously.

Proposal 1: Clarification is required for the power setting of CSI-RS for RLM.

Proposal 2: Remove CCR.3.2 in RMC CORESET configuration.

Observation 2: TCI state for CORESET is missing for some FR1 RLM tests.

Proposal 3: To add TCI state for CORESET for FR1 RLM tests.

References

[1] TS38.133, NR; Requirements for support of radio resource management, V15.7.0.

[2] R4-1910104, WF on RLM test setup, Huawei, HiSilicon, MediaTek, Nokia, Nokia Shanghai Bell, Samsung, Intel, Qualcomm, NTT DOCOMO.