3GPP TSG-RAN WG4 Meeting #92-bis                                               R4-1911613

Chongqing, China, 14th to 18th Oct. 2019

Agenda Item:	9.21.1

Source: 	MediaTek Inc.

Title: 	n28 supporting 30MHz

Document for:	Approval

- 1. Introduction

A new WID in [1] has been approved in RAN_P#85. UE is going to support n28 with 30MHz channel bandwidth. The contribution characterize REFSENS and propose on UL configuration with considering general MPR definition.

- 2. Discussion

REFSENS

The REFSENS calculation assumptions are listed in table 2.1-1 and REFSENS analysis is in table

| Parameter | Value | Unit |
| --- | --- | --- |
| Antenna isolation | 10 | dB |
| n28 filter rejection from TX to RX | 55 | dB |
| n28 diversity receiver filter TX rejection | 45 | dB |
| Front-end loss | 4 | dB |
| Thermal noise at n28 RX ANT port | -164 | dBm/Hz |
| Transceiver effective phase noise due to wider BW | -144 | dBc/Hz |
| SNR requirement for QPSK | -1 | dB |

Table 2.1-1 Typical receiver performance parameters for MSD analysis

| Direct Signal Path |  |  |  |
| --- | --- | --- | --- |
| Parameter | Main | Diversity |  |
| n28 TX power at antenna port (Primary path) | 23 | 23 | dBm |
| n28 PA output noise power at n28 RX freq | -125 | -125 | dBm/Hz |
| n28 TX noise power at n28 PRX LNA port | -96.7 | NA | dBm/30MHz |
| n28 TX noise power at n28 DRX LNA port | NA | -96.7 | dBm/30MHz |
| n28 receiver reciprocal mixing noise at LNA port | -97.3 | -97.3 | dBm/30MHz |
| Combined thermal noise at RX ant port | -161 |  | dBm/Hz |
| TX IM2 due to wider CBW in n28 LNA input port | -97 | -97 | dBm |
| Total noise level refer to receiver LNA input port | -88.2 | -88.2 | dBm |
| Combined |  |  |  |
| REFSENS (30MHz BW) | -88.2 |  | dBm |

Table 2.1-2 Link analysis for n28 SCS=15 KHz, 30 MHz REFSENS calculation

Proposal 1: REFSENS for n28, 30MHz is proposed highlighted as below:

Table 7.3.2-1: Two antenna port reference sensitivity QPSK PREFSENS

| Operating band / SCS / Channel bandwidth / Duplex-mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 40 MHz (dBm) | 50 MHz (dBm) | 60 MHz (dBm) | 80 MHz (dBm) | 90 MHz (dBm) | 100 MHz (dBm) | Duplex Mode |
| n28 | 15 | -98.5 | -95.5 | -93.5 | -90.8 |  | -88.2 |  |  |  |  |  |  | FDD |
|  | 30 |  | -95.6 | -93.6 | -91.0 |  | -88.3 |  |  |  |  |  |  |  |

Table 7.3.2-3: Uplink configuration for reference sensitivity

| Operating band / SCS / Channel bandwidth / Duplex mode |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz | Duplex Mode |
| n28 | 15 | 25 | 251 | 251 | 251 |  | 251 |  |  |  |  |  |  | FDD |
|  | 30 |  | 101 | 101 | 101 |  | 101 |  |  |  |  |  |  |  |

Discussion about UL channel bandwidth

In TS38.101-1 sub clause 6.2.2	UE maximum output power reduction, the MPR assumption is copied below:

UE is allowed to reduce the maximum output power due to higher order modulations and transmit bandwidth configurations. For UE power class 2 and 3, the allowed maximum power reduction (MPR) is defined in Table 6.2.2-2 and Table 6.2.2-1, respectively for channel bandwidths that meets both following criteria:

Channel bandwidth ≤ 100 MHz.

Relative channel bandwidth ≤ 4 % for TDD bands and ≤ 3 % for FDD bands

Where relative channel bandwith = 2*BWChannel / (FUL_low + FUL_high)

For n28 with 30MHz uplink CBW, relative channel bandwidth = 2*30/(703+748) = 4.13% which exceeds the “≤ 3 % for FDD bands”. Then existing MPR table may not be applied for this case.

Obervation 1: If UL bandwidth extend to 30MHz, it will exceed existing MPR BW ratio assumption in TS38.101-1

In order to be compatible with existing spec, we would like to propose to keep current n28 UL channel bandwidth without extension to 30MHz.

Proposal 2: To be compatible with existing spec, the maximum uplink channel bandwidth keeps 20MHz.

The operating bands and supported asymmetric channel bandwidth combinations are defined in table 5.3.6-1.

Table 5.3.6-1: FDD asymmetric UL and DL channel bandwidth combinations

| NR Band | Channel bandwidths for UL (MHz) | Channel bandwidths for DL (MHz) |
| --- | --- | --- |
| n28 | 20 | 20, 30 |

Reference:

- RP-192261 New WI proposal: Addition of wider channel bandwidth in NR band n28, TSG-RAN Meeting #85

- TS38.101-1 User Equipment (UE) radio transmission and reception; Part 1: Range 1 Standalone