3GPP TSG-RAN WG4 Meeting #92bis	R4-1912273

Chongqing, China, 14 October – 18 October 2019

Source:	Ericsson, T-Mobile US

Title:	TP to TR 37.716.21-21: Addition of DC configurations for DC_2-66_n41-n71

Agenda item:	9.8.2

Document for:	Approval

# Background

This text proposal for TR 37.716-21-21 v0.7.0 to add DC_2-66_n41-n71 as defined in WID [1].

# Text Proposal

##### ---Start of changes---

## 6.x	DC_2-66_n41-n71

### 6.x.1	Operating bands for DC

Table 6.x.1-1: DC band combination of LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Band combination | E-UTRA and NR DC Band | Uplink (UL) band |  |  | Downlink (DL) band |  |  | Duplex mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| DC_2-66_n41-n71 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 66 | 1710 MHz | – | 1780 MHz | 2110 MHz | – | 2200 MHz | FDD |
|  | n41 | 2496 MHz | – | 2690 MHz | 2496 MHz | – | 2690 MHz | TDD |
|  | n71 | 663 MHz | – | 698 MHz | 617 MHz | – | 652 MHz | FDD |

### 6.x.2	Channel bandwidths per operating band for DC

Table 6.x.2-1: Supported bandwidths per DC LTE 1DL/1UL + inter-band NR 2DL/1UL

| E-UTRA and NR DC Configuration | UL Configuration | NR Band | SCS [kHz] | 5 | 10 | 15 | 20 | 30 | 40 | 50 | 60 | 80 | 90 | 100 | Max BW [MHz] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DC_2A-66A_n41A-n71A | DC_2A_n41A DC_2A_n71A DC_66A_n41A DC_66A_n71A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  | 160 |
|  |  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  | n41 | 15 |  | Yes | Yes | Yes | Yes | Yes | Yes |  |  |  |  |  |
|  |  |  | 30 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  |  | 60 |  | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes | Yes |  |
|  |  | n71 | 30 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |
|  |  |  | 60 |  | Yes | Yes | Yes |  |  |  |  |  |  |  |  |

### 6.x.3	Co-existence studies

Co-existence analysis for DC_2_n41 shows that there are IMD2 and IMD5 impacts from DC_2_n41 UL to Band n71 DL, but no impact on 66 DL

Co-existence analysis for DC_2_n71 shows that there is IMD2 impact from DC_2_n71 UL to Band n41 DL, but no impact on 66 DL

Co-existence analysis for DC_66_n41 shows that there is IMD4 impact from DC_66_n41 UL to Band 2 DL, but no impact on n71 DL

Co-existence analysis for DC_66_n71 shows that there is no impact from DC_66_n71 UL to either Band 2 DL or Band n41 DL

### 6.x.4	∆TIB and ∆RIB values

For DC_2-66_n41-n71 the TIB,c and RIB,c values are derived from LTE combinations CA_2-7-12-66 and are given in the tables below.

Table 6.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-66_n41-n71 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n41 | 0.81 |
|  |  | 1.32 |
|  | n71 | 0.8 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

Table 6.x.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-66_n41-n71 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | n41 | 0.51 |
|  |  | 12 |
|  | n71 | 0.5 |
| NOTE 1: The requirement is applied for UE transmitting on the frequency range of 2545-2690MHz. NOTE 2: The requirement is applied for UE transmitting on the frequency range of 2496-2545MHz. |  |  |

### 6.x.5	MSD

UL 2_n41 into DL n71 create IMD2 and UL 2_n71 into DL n41 create IMD2, and they are both captured in the lower order combination DC_2_n41-n71.

UL 66_n41 into DL n2 create IMD4, and this is captured in the lower order combination DC_2-66_n41.

Based on the co-existence studies there are no further MSD needed to be defined.

##### ---End of changes---

# Reference

[1]	RP-191744, “Revised WID on EN-DC of x bands (x=1,2,3,4) LTE inter-band CA (xDL/1UL) and 2 bands NR inter-band CA (2DL/1UL)”, LG Electronics